THE THIRTY-SEVENTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/37)

(Tokyo, Japan, 29 October-2 November 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Request for RJAA/NRT Narita Airport Arrival Route Review

(Presented by IATA)

SUMMARY

This working paper is an update to the information paper (IP14) which was submitted by IATA during IPACG/36 that identified concerns with the modified Narita’s published arrival routings.  With previous discussions and feedbacks received from JCAB in mind, IATA would like to continue to request a review by the representatives from JCAB on these arrival routes.

- Introduction

  - Effective with the October 20th, 2011 AIRAC cycle update, airline operators were asked by JCAB to change the arrival routes into Narita per AIC 35/11 “Change of Flight Planned Routes.”   The following describes the three significant arrival route changes where United Airlines has noted an operational impact.

- All flights that enter Japanese airspace from Russia via waypoints AKSUN, LUMIN, ANIMO, BISIV, and ODERI) via CHE (Chitose VOR) are no longer allowed to file airway Y10, and instead are required to file airway Y12.  Reference attached (Attachment A)

- Extended/modified routing on airway Y30 SUIGO-DOVER-AKTUS-MELON prior to final approach vectors into Narita.  Reference attached (Attachment B – circled in Red)

- All flights that are arriving on R220 via NANAC, the relocation of waypoints KETAR and MELON.  Reference attached (Attachment D)

- Discussion

  - Reference Introduction 1.1 (A) - At IPACG/36, IATA received a comment by JCAB that the new routing was developed in consideration for operator’s need to avoid the No-Fly Zone triggered by Fukushima’s nuclear disaster.

- IATA is greatly appreciative of JCAB action.  IATA also recognizes and understands the airspace challenges and flow of traffic issues within the Tohoku region incurred by the nuclear accident in Fukushima.

- With the cessation of aeronautical/airspace impact that resulted directly from Fukushima’s No-Fly Zone, IATA is ready to discuss with JCAB if previous NRT arrival routing over V22 and Y30 can be re-instated.  In addition, JCAB to consider IATA proposal via Y302 (Attachment C)

  - Reference Introduction 1.1 (B) - At IPACG/36.  IATA received a comment by JCAB that the extended/modified routing may be more than adequate to provide a safe sequencing and separation of NRT air traffic.

- IATA would like to formally request a feasibility study of KASMI DCT MELON (or airway equivalent) to final approach vectors to NRT.

  - Economic/Emission impact - For instance, the combination of changes A & B has mileage consequence of 84NM as we have to fly near Niigata VOR (GTC) to proceed southbound, and then plan to fly the extended Y30 before given final approach vectors to Narita.  For the KETAR-MELON relocation, (C above), the distance increase is 13NM.

  - With the stated mileage increases above, United Airlines sees an operational impact associated with fuel and CO2 emissions increases as the following – 2 examples of 10 daily flights to NRT.

CHE (Chitose)–RJAA – Mileage Increase  is *84NM*

UAL79 New York-Tokyo/Narita Flight Planned Route & Data

Route: CHE.Y12.HWE.Y122.OBAKO.Y128.KAMOH.Y303.SWAMP.Y30.MELON..NRT

FUEL Increase (Per Flight)                    		+3,100 LBS 		(1,406 KG)

FUEL ANNUAL Increase (Per Flight)		+339,450 LBS		(157,972 KG)

CO2 Emissions Increase (Per Flight)			+9,610 LBS 		(4,359 KG)

CO2 ANNUAL Emissions Increase (Per Flight)	+1,092,295 LBS	(495,457 KG)

Flight Time Increase/Flight				+10 minutes

NANAC–RJAA - Mileage Increase  is *13NM*

UAL7 Houston – Tokyo/Narita Flight Planned Route & Data

Route: NANAC.Y810.KETAR.Y811.MELON..NRT

FUEL Increase (Per Flight)                    		+800 LBS 		(363 KG)

FUEL ANNUAL Increase (Per Flight)		+262,800 LBS		(119,204 KG)

CO2 Emissions Increase (Per Flight)			+2,480 LBS		(1,125 KG)

CO2 ANNUAL Emissions Increase (Per Flight)	+814,680 LBS 		(369,537 KG)

Flight Time Increase/Flight				+2 minutes

  - Dynamic en route winds and Jet Stream position often dictate where North American flights enter Japanese airspace throughout the year.  Airline operators consider Chitose arrival route to Narita an important route option in addition to NOPAC routes, particularly for flights departing from (but not limited to) New York, Washington DC, Chicago, Atlanta, Detroit,  Toronto, Houston and Dallas, showing many operational benefits especially in the winter months.

- Conclusion

3.1	The meeting is invited to note the information provided and JCAB to advise when we will be able to revert to previous efficient arrival routing to NRT.

Attachment A

Pink –New Route 520NM

Blue – Previous Route 436NM

Attachment B

Attachment C

Attachment D