THE THIRTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/38)

(Mountain View, California, USA, 22-26 April 2013)

Agenda Item 6:  CNS Issues

Suggested VHF Region Definition Changes to Improve ACARS Performance

(Presented by Boeing)

SUMMARY

Avionics that are configured to use VHF subnetworks in areas that have only scattered VHF subnetwork coverage may experience poor ACARS performance.  Some avionics, however, offer a means to mitigate this problem.

- Introduction

  - Aircraft operators typically configure their data communications avionics to prefer VHF subnetworks over SATCOM subnetworks.

  - Some avionics also have a geographic region definition capability that permits aircraft operators to configure the avionics to use and prefer specified subnetworks in defined geographic regions.  These regions may be specified for VHF, SATCOM, and/or HFDL subnetwork use as a set of latitude/longitude-defined rectangles.

  - In areas of the world that have only scattered VHF subnetwork coverage, avionics that are configured to use VHF subnetworks in those areas switch from SATCOM to VHF and back to SATCOM when the aircraft passes an isolated VHF ground station.  The time required for this switching adversely affects ACARS performance for FANS.  (Potential mitigations to this issue are also being discussed in the AEEC Data Link Systems Subcommittee.)

  - Additionally, in some cases the aircraft passes through only the outer edge of the service volume of an isolated VHF ground station.  This allows the avionics to establish a link to the VHF subnetwork but the avionics may not be able to successfully send downlinks due to the tenuous and temporary nature of the link.  Unsuccessful downlink attempts also adversely affect ACARS performance for FANS.

  - Examples of areas of the world in which Boeing has seen these performance problems include the North Pacific in the vicinity of the Aleutian Islands and the Kamchatka Peninsula, in the South Pacific in the vicinity of New Caledonia and Vanuatu, and in the North Atlantic in the vicinity of Bermuda and the Azores.

- Discussion

  - Boeing suggests that aircraft operators with capable avionics consider reconfiguring the avionics to exclude areas of the world with only scattered VHF subnetwork coverage from their VHF region definitions.

  - If aircraft operators reconfigure their avionics in this way, then the avionics will admittedly not use VHF in every possible case, but Boeing expects that the changes will have a positive effect because they would improve ACARS performance for both AOC and ATS purposes.

  - Boeing also recognizes that aircraft operators would make any decisions to reconfigure their avionics in this way on an area-by-area basis in the context of their route structure.

- Conclusion

  - The meeting is invited to note the information provided.

  - Aircraft operators are invited to consider reconfiguring their avionics as suggested.