THE THIRTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/38)

(Mountain View, California, USA, 22-26 April 2013)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

VARIATIONS IN AIRSPEED IN CONTROLLED AIRSPACE AND NAVIGATION ERROR REPORTING

(Presented by the Federal Aviation Administration)

SUMMARY

This paper provides an update on implementation of operator notification of deviation from the planned true Mach number specified in Item 15 of the filed flight plan within the Anchorage Oceanic and Oakland Oceanic Flight Information Regions (FIR).

- Introduction

ICAO letter, AN 13/1.8, 13-5/11/07 dated 31 January 2011, requested comments on the proposal for the amendment of Annexes 2 and 11 concerning speed variations and reduced vertical separation minimum (RVSM) monitoring.

3.6.2.2(c) Change in time estimate:

if the time estimate for the next applicable reporting point, flight information region boundary or destination aerodrome, whichever comes first, is found to be in error in excess of 2 minutes from that notified to air traffic services, or such other period of time as is prescribed by the appropriate ATS authority or on the basis of air navigation regional agreements, a revised estimated time shall be notified as soon as possible to the appropriate air traffic services unit.

  - The United States concurred that the amendment of the requirement to forward estimate revisions from, in excess of 3 minutes, to 2 minutes, would provide some increase in the level of safety associated with the reduced longitudinal separation procedures being applied by an air traffic services unit (ATSU). However, in and of itself, this change will do very little to safeguard against the loss of adequate longitudinal spacing between aircraft.

  - ICAO implemented the change to Annex 2 paragraph 3.6.2.2 effective on 15 November 2012.  Aircrews are now required to update air traffic service units any time an ETA changes by more than 2 minutes.

  - International Civil Aviation Organization (ICAO) working groups composed of industry, air traffic control (ATC) and state regulators meet regularly to discuss oceanic errors in detail.  Oceanic airspace is procedural airspace based on strategic clearances, which means controllers issue oceanic clearances and aircrews must follow specific navigation, speed, and altitude procedures.  Because most oceanic errors occur in the non-radar environment, they tend to be more pronounced and of a longer duration than those within radar coverage.  Therefore, whether or not there is a loss of separation, the reporting of oceanic navigation errors is essential for evaluating the overall safety of oceanic airspace. Besides being hazards to flight safety, they create barriers to future reductions in separation.  Timely reporting of oceanic errors allows prompt corrective action.

  - The FAA has issued guidance to FAA Oceanic Facilities in FAA Order 7110.82D, on reporting Oceanic Errors.  This order establishes procedures for reporting Gross Navigation Errors (GNE), height errors, time (longitudinal) errors, and Special Area of Operations (SAO) verification in oceanic airspace. All aircraft involved in any of the above in an FAA oceanic controller’s airspace, whether U.S., military, or foreign, must be reported in accordance with this Order. These reports include:

    - Gross Navigation Errors.  Lateral errors of 25 nautical miles (NM) or more from the aircraft’s cleared route.

    - Intervention.  Occurs when an aircraft reports an incorrect routing and ATC intervenes to correct the error before the aircraft actually executes the incorrect routing.

    - Height Error.  Errors of 300 feet or more from a clearance altitude.

    - Time Error.  Occurs when an aircraft’s reported actual time of arrival (ATA) is more than 3 minutes before or after the estimated time of arrival (ETA).  (For North Atlantic [NAT], the criterion is 3 minutes or more.)

- Discussion

  - There is significant safety risk associated with allowing speed changes without first notifying the ATSU.  Accordingly, the FAA proposed a change to address the negative impact that speed variations can have on longitudinal separation as separation minima shrink, concurrently with increased use of aircraft flight management systems to automatically adjust (reduce) airspeed for more economical operation.  For turbo-jet aircraft intending to operate within the Anchorage Oceanic and Oakland Oceanic FIRs, if for any reason the Mach number/true airspeed at cruising level varies by plus or minus 0.02 Mach/10 knots or more, from the first filed speed entry in flight plan form Item 15, the appropriate ATSU shall be so informed immediately.

  - The Proposal for Amendment of the Regional Supplementary Procedures Pacific Region was forwarded to ICAO Bangkok on 29 July 2011 and is available for review at Attachment A.

  - The reporting of oceanic navigation errors has been in place for many years.  FAA controllers are to report time errors per the requirements of FAAO 7110.82D, but a busy controller may not have time to file a report for a time error when working traffic if no loss of separation occurs.  These reports are important for evaluating the overall safety of oceanic airspace.

  - In order to relieve pressure on busy controllers and allow them to focus on their active separation and safety responsibilities, Oakland Air Route Traffic Control Center (ARTCC) has developed an automated tool which will capture instances where high frequency (HF) aircraft fail to update their ETA at a reporting point and subsequently progress 4 or more minutes different than their ETA for the reporting point..

  - Failure to revise ETAs for a reporting point as required, like un-notified speed changes, poses a significant safety risk.  Aircrews are reminded of their responsibility to notify the ATSU when the ETA for a reporting point changes by more than two minutes as soon as possible.  Failure to notify an FAA ATSU of an updated ETA as defined in the requirements, may lead to a Time Error report being filed for a flight.

  - Upon submission of the Oceanic Navigation Error report (which includes Time Errors), the FAA will take the following actions:

    - The FAA Navigation Specialists will coordinate with the appropriate office to investigate each error, as appropriate.

    - FAA Flight Standards and ICAO scrutiny groups review the errors and use the data to identify trends, training issues, and procedural problems.

    - The FAA Technical Center will use the data for statistical analysis.

- Action by the meeting

  - The meeting is requested to recognize the significant safety risk that un-notified aircraft speed changes and Oceanic Navigation Errors place on the ATC system, and

  - Support changes to ICAO documents which outline speed change notification to ATSUs, and

  - Operators are requested to ensure their pilots are aware of the ICAO requirements to revise ETAs for reporting points and FAA Oceanic Navigation Error reporting procedures.

PROPOSAL FOR AMENDMENT OF THE

REGIONAL SUPPLEMENTARY PROCEDURES

PACIFIC (PAC) REGION (Doc. 7030/5)

(Serial No.: XXX – ASIA/PAC 6-2)

a)	Regional Supplementary Procedures:

ASIA/PAC

b)	Proposed by:

United States

c)	Proposed amendment:

Editorial Note: Amendments are arranged to show deleted text using strikeout (text to be deleted), and added text with grey shading (text to be inserted).

Amend the following in the ASIA/PAC SUPPS, Chapter 6.

Chapter 2.   FLIGHT PLANS

2.1   CONTENT – GENERAL

(A-2, Chapter 3; P-ATM – Chapter 4 and Appendix 2)

2.1.11   Mach Number

2.1.11.1	For turbo-jet aircraft intending to operate within the Anchorage Oceanic and Oakland Oceanic FIRs, the planned true Mach number shall be specified in Item 15 of the flight plan.

2.1.11.2	For turbo-jet aircraft intending to operate within the Anchorage Oceanic and Oakland Oceanic FIRs, if for any reason, the Mach number/true airspeed at cruising level varies by plus or minus 0.02 Mach/10 knots or more from the first filed speed entry in flight plan form Item 15, the appropriate air traffic service unit shall be so informed immediately.

d)	Date when proposal received:

XXX

e)	Proposers reason for amendment:

1)	Modern air traffic control (ATC) automation systems project the future positions of aircraft using expected airspeed.  The resulting ATC decision support functions base future aircraft clearances on these projected positions.  Because of the reliance on the expected airspeed and the recent reductions in longitudinal separations, any variation in airspeed can affect the horizontal separation of aircraft in controlled airspace.  As horizontal separation minima are reduced, the tolerance for error in the execution of the clearance is limited.  Thus, it is important that operators and ATC units understand the effects of such variations and have a mutual understanding of permissible, if any, airspeed variations to ensure the continued safe operation of controlled airspace.

2)	Separation assurance involves the application of separation standards to ensure aircraft remain an appropriate minimum distance or altitude from other known aircraft.  Air Traffic Service Units in a procedural control environment must be aware of the speed an aircraft is flying in order to maintain separation assurance.  Air Traffic Controllers utilize the first filed speed entry in the aircraft flight plan when making control decisions.  Aircraft must fly at the flight planned speed or advise ATC of any deviations from that speed.  This allows controllers to have more assurance in applying longitudinal separation thereby allowing flights to operate more efficiently without compromising safety.

3)	Just as an aircraft makes a request to ATC to change altitude, even though the planned altitude change is within Item 15 route of flight, an aircraft must request a change of speed from ATC also.  ATSUs are implementing ICAO approved reduced separation minima such as ADS 30nm longitudinal separation.  With the implementation of reduced separation minima, known aircraft speed becomes even more critical to ensure there is no loss of planned longitudinal separation.

4)	There is significant safety risk associated with allowing speed changes without first notifying the air traffic service unit.  The following data prepared by the Federal Aviation Administration Air Transportation System Evaluation Group, Separation Standards Analysis Team, is presented as supporting evidence.

The requirements for the application of a 30NM longitudinal separation standard using ADS-C are listed in Section 5.4.2.6.4 of ICAO Document 4444, Air Traffic Management.  Among other items, this Section requires that aircraft be approved to RNP-4, and specifies the need for ADS with a maximum periodic reporting interval of 14 minutes.  Given this periodic reporting interval, 14 minutes is the maximum expected time between consecutive ADS position reports for flights eligible for the 30NM longitudinal separation standard.  This maximum expected time between consecutive position reports occurs when the reporting times of both aircraft are synchronized in time.

The actual position of aircraft between consecutive position reports is unknown to ATC.  Aircraft performance and weather affect the speed of the airplane.  The collision risk model which supported the 30 nm longitudinal separation change assumed aircraft operate at constant speed during the time interval in which risk is estimated.  The collision risk model included along-track and across-track errors to account for the difference between the nominal and actual position of the aircraft.  The along-track and cross-track errors were also assumed to be constant during the time interval in which risk is estimated.  In most cases these are valid assumptions.  However, given the observed use of economic cruise modes and the expected increase in the application of the reduced separation standards in the Pacific, it is important to consider the effect on the probability of an overtake when airspeed change occurs.

The distance-based longitudinal model developed when initially assessing the risk for this procedure provides a relationship for computing the longitudinal distance between a pair of airplanes.  However, this model and the model developed in a later study assume constant airspeed during the interval for which risk is estimated.

Let A1 and A2 be two airplanes that fly along the same route, in the same direction, and at the same flight level.  Let A1 denote the leading airplane, and A2, the trailing airplane.  A1 and A2 are already flying on the same track and flight level.  Let to be the time at the start of the 14 minute reporting interval.

At a time t, t ≥ to, during the 14 minute time interval between consecutive ADS reports, in which A1 and A2 are operating on the same route and flight level, the separation distance between A1 and A2 is denoted as S(t).  The distance of A1 from the position of A2 at to is denoted by D1(t).  Additionally, D2(t) is the distance of A2 from the position of A2 at time to.  At time to, the start of the interval over which risk is estimated, D2(to) is equal to zero, and the separation, S(to),  between A1 and A2 is simply equal to D1(to).  Equation 1 provides a general form for estimating S(t).

S(t) = D1(t) – D2(t)  for t ≥ to						(1)

At some time t, where t > to, a change of speed occurs for one or both airplanes.  It is assumed that this change in speed occurs almost immediately after time to.  Let V1 and V2 denote new speed for A1 and A2, respectively.  The new speed for each airplane is the initial speed plus the change in speed.

Therefore

ΔV = V1 - V2 									(2)

Using equations 1 and 2, the new separation distance at time tm, S(tm), is given by

S(t) = D1(t) – D2(t)      where  t > to

= S(to) + V1(t - to) – V2(t - to)

= S(to) + (V1 - V2 ) (t - to)

= S(to) + ΔV (t - to)  						(3)

For each increment of speed difference, ΔV, it takes  to erode the initial separation, S(to).  Therefore, for an overtake to occur by some time t, where  t > to, the time to erode the initial separation must be less than or equal to the time interval between consecutive position reports and the ATC intervention buffer;

The ATC resolution buffer is denoted as τ.  Therefore, the probability of an overtake is the probability that τ is greater than or equal to the time for the remaining separation to be eroded at the end of the 14 minute reporting interval:

(4)

Rearranging terms in equation (4):

(5)

The components for the ATC resolution buffer, τ, are provided in a study titled “Collision Risk Model Based on Reliability Theory that Allows for Unequal RNP Navigation Accuracy.”  Under normal ADS operation, an allowance of 4 minutes is assumed for the value of τ.  In the case where the periodic ADS reports are received and a response to the CPDLC uplink is not received in 3 minutes, an allowance of 10½ minutes is assumed for the value of τ.  The study referenced above also provides components for τ when the ADS periodic report is lost or takes longer than 3 minutes, these components are listed in Table 1.  The total allowance provided for the ATC resolution buffer in this case is 810 seconds or 13½ minutes.

| Component | Value (seconds) |
| --- | --- |
| Controller wait for ADS report | 180 |
| Controller message composition | 15 |
| CPDLC uplink and wait for response | 90 + α |
| HF communication | 300 |
| Pilot reaction | 30 |
| Aircraft inertia plus climb | 75 |
| Extra allowance | 30 |
| Total | 720 + α |

Table 1.  Components of τ when ADS periodic report takes longer than 3 minutes

Three minutes after an ADS position report is overdue, a request for a position report will be sent by ATC via ADS or CPDLC.  The study makes a conservative assumption that this request will always fail, the original time allowance for this request is 180 seconds for the CPDLC uplink and wait for response.  The time allotted for the CPDLC uplink was 90 seconds, the remaining 90 seconds was the time allotted for the controller to wait for the response.  The controller will re-attempt to contact the aircraft via HF, a 300 second allowance is provided for this in Table 2.

Transit time data for uplink CPDLC messages were collected from the Oakland OAC over the eight month period of February through July 2008.  These data show a large range for CPDLC uplink transit times.  A total of 290,178 data values were available during this time period.  The maximum delay time observed was over 45 minutes (45:32 minutes).  These data were fit to a mixture of two exponential distributions, with parameters λ1= 15.73 sec, λ2 = 240.01 sec and, ρ = 0.015.

where 0 < ρ < 1, and 0 < λ1 < λ2

The CPDLC uplink time is modeled to the fitted data.  The α value in Table 1 represents the transit time for CPDLC uplink messages observed in the Oakland OAC data.

It is desired to compute the maximum change in longitudinal distance between the aircraft pair if one or both of the aircraft change their airspeed.  To do this, the worst case scenario is examined.  Here, the initial longitudinal distance, S(to), between A1 and A2 is close to the minimum of 30 nm, and ATC expects the aircraft to maintain the same Mach number, although for this scenario a Mach number assignment has not been given to either aircraft.  The ADS periodic reporting interval is 14 minutes.

There are nine possible scenarios to consider for the change in airspeed, in some cases the magnitude of the airspeed change by aircraft A1 and/or A2 determines whether an overtake is possible or not.  Table 2 contains the nine possible speed change scenarios.

|  | Aircraft A1 Increases Speed | Aircraft A1 Decreases Speed | Aircraft A1 Maintains Constant Speed |
| --- | --- | --- | --- |
| Aircraft A2 Increases Speed | Possible Risk of Overtake | Risk of Overtake | Risk of Overtake |
| Aircraft A2 Decreases Speed | No Risk of Overtake | Possible Risk of Overtake | No Risk of Overtake |
| Aircraft A2 Maintains Constant Speed | No Risk of Overtake | Risk of Overtake | No Risk of Overtake |

Table 2.  Speed Change Scenarios for the Lead Airplane, A1, and the Trailing Aircraft, A2, Over a 14 Minute Interval

In the worst case scenario, the lead aircraft, A1, experiences a decrease in airspeed, while the trailing aircraft, A2, experiences an increase in airspeed.

Between FL250 and FL450, the ratio of Mach number to knots is approximately 0.01 to 6 knots.  This assumption was validated using the ICAO Standard Atmosphere for FL250 through FL450.

It is also assumed that the aircraft report simultaneously because this increases the interval of uncertainty in the positions, thus increasing the amount of potential separation change between the aircraft pair.  Therefore, the change in longitudinal distance over the 14 minute periodic interval is examined.

If both airplanes share a common initial speed, then ΔV in equation (2) is equal to the difference in the change of speed between the two airplanes.  Let time tm be the time of the end of the 14 minute reporting interval.  Then the new separation distance at time tm, S(tm), is given by equation (3).  The initial separation distance, S(to), is equal to the minimum allowed, 30 nm.  The difference between the end time and the start time, (tm - to), is the ADS periodic reporting interval of 14 minutes.  It is assumed the reporting times are synchronized in the worst case scenario.  Therefore S(tm) becomes

S(tm) =  S(to) + ΔV (tm - to)

= 30 nm + ΔV (14 min)

(6)

Assuming the airplanes hold the new speed, equation (6) gives the longitudinal separation between the airplanes at the end of the 14 minutes reporting interval.  Let tb be the time at the end of the ATC resolution buffer.  Then, the amount of time before an overtake occurs is the amount of ATC resolution buffer time before the longitudinal separation equals 0 nm. Let S(tb) be the separation at time tb, where tb > tm > to.

S(tb) = D1(tb) – D2(tb)      where  tb > tm

= S(tm) + V1(tb - tm) – V2(tb - tm)

= S(tm) + (V1 - V2 ) (tb - tm)

= S(tm) + ΔV (tb – tm)  						(7)

An overtake occurred when the longitudinal distance between the airplanes at the end of the ATC resolution buffer, S(tb), is 0 nm.  The amount of ATC resolution buffer time available before an overtake occurs is found by setting S(tb) = 0 nm.

S(tb) =  S(tm) + ΔV (tb – tm)

0 =  S(tm) + ΔV (tb – tm)

(8)

Assuming the worst case scenario, at least one of the ADS periodic reports will be lost.  Using the τ when an ADS periodic report takes longer than 3 minutes, Table 3 presents the longitudinal distances after the 14 minute periodic report interval using equation (3) in column 2.  Given the speed changes indicated in column1, column 3 of Table 3 presents the separation distance still to be eroded for an overtake to occur using equation.  The 4th column of Table 3 uses equation (8) to determine the size of the ATC resolution buffer needed for an overtake to occur.  After removing the static portions of the ATC resolution buffer contained in Table 2, the last column in Table 3 contains the probability that the ATC resolution buffer time would equal or exceed the minimum τ needed for an overtake.  This value is given by the data fitted to a mixture of two exponential distributions observed for CPDLC uplink messages in Oakland OAC.

| Combined Speed Difference ΔV (Mach) | Separation Decrease After 14 Minutes (nm) | Distance Still to Be Eroded After 14 Minutes Elapsed for an Overtake to Occur (nm) | Min τ Needed for an Overtake to Occur (minutes) | P(ATC Resolution Buffer ≥ Min τ Needed for an Overtake) |
| --- | --- | --- | --- | --- |
| -0.08 | 11.2 | 18.8 | 23.50 | 8.463 x 10-4 |
| -0.07 | 9.8 | 20.2 | 28.86 | 2.218 x 10-4 |
| -0.06 | 8.4 | 21.6 | 36.00 | 3.719 x 10-5 |
| -0.05 | 7.0 | 23.0 | 46.00 | 3.053 x 10-6 |
| -0.04 | 5.6 | 24.4 | 61.00 | 7.181 x 10-8 |
| -0.03 | 4.2 | 25.8 | 86.00 | 1.387 x 10-10 |

Table 3.  Probability that the ATC Resolution Buffer ≥ the Minimum τ Needed for an Overtake to Occur

A study entitled “The Rate of Collisions Due to the Loss of Distance-Based Longitudinal Separations” provides an estimate of collision risk as:

P{pair collides} = P{pair collides | overtake occurs} x P{overtake occurs}

A partial form of the collision risk model is:

(9)

The P{τ ≥ Minimum τ needed for an overtake} is substituted for the P{overtake occurs} in equation (9) for this worst case scenario.  The estimate of the probability of an overtake comes from the given change in airspeed, the remaining separation distance to be eroded for an overtake to occur, the CPDLC performance data, and the length of the ATC resolution buffer time needed for an overtake to occur.

Table 4 contains the parameter definitions and values assumed for risk estimation using equation (9).

| Parameter | Description | Value | Source |
| --- | --- | --- | --- |
| Nax | Collision risk of an aircraft pair on the same route at the same flight level whose nominal separation is x (NM). |  |  |
| Py(0) | Lateral overlap probability. Probability that airplanes assigned to the same route have laterally overlapping positions. | 0.669 | Value estimated for pairs of GPS-GPS aircraft (Ref 10) |
| Pz(0) | Vertical overlap probability. Probability that airplanes assigned to the same flight level have vertically overlapping positions. | 0.538 | Value used in Pacific Vertical Risk Estimate |
| T | Reporting interval of ADS position report. | 14 minutes | Requirement for ADS-based separation (Ref 7) |
| x | Average aircraft length (nm) | 0.0364 nm | Value used in Pacific Vertical Risk Estimate |
| y | Average aircraft width (wingspan) (nm) | 0.0321 nm | Value used in Pacific Vertical Risk Estimate |
| z | Average aircraft height (nm) | 0.0101 nm | Value used in Pacific Vertical Risk Estimate |
|  | Average relative speed at which an airplane overtakes and passes another airplane assigned to the same route and flight level (kts) | Varies by scenario | =ΔV in Table 3 converted to kts |
|  | Average relative speed at which airplanes assigned to the same route laterally wander past each other (kts) | 20 kts | Value used in Ref 10 |
|  | Average relative speed at which airplanes assigned to the same flight level vertically wander past each other (kts) | 1.5 kts | Value used in Ref 10 |

Table 4.  Collision Risk Model Parameter Definitions and Estimates

The Collision Risk Model Based on Reliability Theory that Allows for Unequal RNP Navigation Accuracy study used a weighted risk for the collision risk estimation for same track longitudinal separation.  The weight given to the ATC resolution buffer corresponding to the components given in Table 1 was 0.05, this means it was assumed that 5 percent of the time the ADS periodic position report would take longer than 3 minutes and the controller would eventually resort to HF communication.  Table 5 provides the collision risk estimates for each scenario presented in Table 3.  Table 5 also provides the “weighted” collision risk values assumed for this worst case scenario as it would apply to the overall risk of the system.

| Combined Speed Difference ΔV (Mach) | Combined Speed Difference ¦ΔV¦ (kts) | P(ATC Resolution Buffer ≥ Min τ Needed for an Overtake) | Collision Risk Estimate (Where τ = Minimum τ Needed for an Overtake to Occur) | Weighted Collision risk = 5% of Collision Risk Estimate |
| --- | --- | --- | --- | --- |
| -0.08 | 48 | 8.463 x 10-4 | 3.057 x 10-4 | 1.529 x 10-5 |
| -0.07 | 42 | 2.218 x 10-4 | 8.015 x 10-5 | 4.008 x 10-6 |
| -0.06 | 36 | 3.719 x 10-5 | 1.345 x 10-5 | 6.725 x 10-7 |
| -0.05 | 30 | 3.053 x 10-6 | 1.105 x 10-6 | 5.526 x 10-8 |
| -0.04 | 24 | 7.181 x 10-8 | 2.603 x 10-8 | 1.302 x 10-9 |
| -0.03 | 18 | 1.387 x 10-10 | 5.039 x 10-11 | 2.519 x 10-12 |

Table 5.  Effect on the Weighted Portion of Risk for RNP 4 ADS Separation

The combined difference in airspeed, ΔV, presented in columns 1 and 2 of Table 5, represents the difference in airspeed of A1 and A2.  The smallest combined speed difference, ΔV, with a collision risk estimate below the Target Level of Safety (TLS) is 0.04 Mach or 24 knots.

This result supports the recommendation for pilots to notify ATC when an airspeed change of 0.02 Mach or more is expected from the first speed entry in Item 15 of the FPL.

f)	Proposed implementation date of the amendment:

Upon approval by the Council.

g)	Action by the Secretary General:

The proposal has been circulated to the following States and international organizations.

XXX

h)	Secretariat’s comments:

If the magnitude of the speed increase of airplane A1 is less than the magnitude of the speed increase of airplane A2 there is a risk of overtake, otherwise no risk of overtake

If the magnitude of  the speed decrease of airplane A1 is greater than the magnitude of the speed decrease of airplane A2 there is a risk of overtake, otherwise no risk of overtake