THE THIRTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/38)

(Mountain View, California, USA, 22-26 April 2013)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Nomination for ASPIRE-Daily Program

(Presented by Civil Aviation Bureau of Japan)

SUMMARY

In January 2011, Asia Pacific Initiative to Reduce Emissions (ASPIRE) partners established the ASPIRE-Daily program to increase awareness and utilization of best practices in the Asia-Pacific region by identifying and publishing ASPIRE-Daily city pairs where three or more fuel-saving best practices are available on a daily basis.

Since then, ten city pairs have been validated and published. However, no city pair associated with Japan could not been nominated. This paper presents the information regarding ongoing efforts for the first nomination from JCAB in collaboration with FAA and Japan airlines (JAL).

- Introduction

  - In January 2011, ASPIRE partners launched the ASPIRE-Daily program, which identifies and publishes the most efficient city-pair routes in the Asia-Pacific region. The objective of ASPIRE-Daily is to increase awareness and utilisation of ASPIRE-Daily Best Practices on a daily basis.

  - ASPIRE-Daily Best Practices include User Preferred Routes (UPRs), Dynamic Airborne Reroute Procedure (DARP), 30/30 Reduced Oceanic Separation, Time-Based Arrivals Management, Arrivals Optimization, Departure Optimization, Surface Movement Optimization, and Network Optimization –Collaborative Decision Making.

  - Based on ASPIRE-Daily star rating system, only city-pair routes where three or more ASPIRE-Daily Best Practices are available will be qualified as ASPIRE-Daily city pairs.

  - Each nominated city pair is assessed and validated in consultation with IATA Asia Pacific Office. After the city pair is validated and published, participating airlines associated with the ASPIRE-Daily city pair are requested to report of their successful utilisation rates of the best practices on monthly basis. Thus, ASPIRE-Daily program is characterized by its collaborative initiative among ASPIRE partner ANSPs, IATA Asia Pacific Office and airlines.

  - The partners have already validated and published 10 city pairs:

 Auckland, NZ – San Francisco, CA

 Los Angeles, CA – Singapore

 Los Angeles – Melbourne, AUS

 Sydney, AUS – San Francisco

 Singapore – Melbourne

 Melbourne – Singapore

 Singapore – Sydney

 Sydney – Singapore

 Melbourne – Los Angeles

 Sydney – Los Angeles .

- Discussion

  - JCAB has been seeking possible candidates for the ASPIRE-Daily city pair for nomination since the program has launched. However, it was difficult to find in any city pair associated with Japan in which airlines can utilize or be applied three or more best practices. This is partly because the aircraft operated by possible participating airlines are not applicable to 30/30 Reduced Oceanic Separation due to lack of RNP 4 certification.

  - However, JAL finally decided to apply for RNP 4 certification to their all B767, B777, and B787 operated for international oceanic flights. They were certified by JCAB in March 2013. Now they have submitted the application to FAA and are waiting for the certification.

  - JAL also showed JCAB their willingness to involve in ASPIRE-Daily program, which led JCAB to decide that Tokyo (Haneda) – San Francisco operated by JAL (flight number “JAL2”) should be the first nominating city pair associated with Japan.

  - Three ASPIRE-Daily best practices, which are UPR, 30/30 Reduced Oceanic Separation, and Arrivals Optimization (Tailored Arrivals at San Francisco), can be utilized by JAL2. JCAB and FAA are coordinating to jointly nominate Tokyo (Haneda) - San Francisco for an ASPIRE-Daily city pair when JAL receives RNP 4 certification.

  - JCAB, in cooperation with Electronic Navigation Research Institute (ENRI) and JAL, estimated the amount of reduction of fuel consumption and CO2 emissions derived from JAL2’s utilization of UPR, 30/30 and Tailored Arrivals.

  - JAL2 departs Tokyo in the middle of night and has relatively low density traffic; therefore, it is assigned its optimum altitude in many cases. Nevertheless, ENRI’s estimation shows that additional 70 lbs/flight fuel can be reduced by applying 30/30 to JAL2 with RNP4, comparing to existing 50/50 application based on RNP10.  In addition, based on JAL’s estimation, 200 lbs/flight fuel savings can be expected by JAL2’s utilization of UPR, and 1,000 lbs/flight by Tailored Arrivals. In total, it is estimated that JAL2 can save more than 212,000lbs/year by taking into account the utilization of these best practices for the past 12 months. This means that even JAL2 flight only, one-way flight from Tokyo to San Francisco in low density traffic, can contribute about 300,000 kg of CO2 emissions per year by utilizing these three ASPIRE-Daily best practices.

  - Furthermore, if all aircraft, flying adjacent to JAL2, are certified with RNP 4, JAL2 can save about 220 lbs/flight, compared to 70 lbs/flight in case of only JAL with RNP4. At the same time, similar fuel savings can be shared with other related traffic with RNP 4.  This convinces us that the expected fuel savings are much greater when all aircraft are certified with RNP4 rather than limited airlines/aircraft are certified with RNP4.

  - JCAB will continue to work closely with FAA, other ASPIRE ANSPs, partner airlines and IATA Asia Pacific Office to expand ASPIRE-Daily city pairs to increase other airlines awareness and utilization of ASPIRE-Daily best practices, which contributes to reduce fuel consumption and the environmental footprint in the Asia Pacific Region.

- Conclusion

  - The meeting is invited to:

a)  note the information provided in this paper; and

b)  be encouraged to utilize ASPIRE-Daily best practices.