THE THIRTY-EIGHTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/38)

(Mountain View, California, USA, 22-26 April 2013)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Update on the progress of the ICAO ITP Circular and associated amendments to Annex 10 Volume II, PANS-ATM and PANS-OPS

(Presented by the Federal Aviation Administration)

SUMMARY

This information paper provides an update on the progress of the ICAO ADS-B ITP Circular and associated amendments to Annex 10 Volume II, PANS-ATM, and PANS-OPS.

- Introduction

  - At the Tenth Meeting of the Separation and Airspace Panel (SASP) Working Group of the Whole (SASP WG/WHL/10) held in November 2006, the panel was advised that the Air Navigation Commission had approved activities of the Aeronautical Surveillance Panel (ASP) and requested that SASP, among other panels, be directly involved in assisting in the progression of a work programme focused on airborne surveillance applications.

  - Of particular interest to SASP was that the programme included development of ADS-B in-trail procedure (ITP) that would enable a reduced separation minimum to be used in planned flight trials. The use of airborne surveillance applications such as ITP would facilitate enroute climbs and descents to enable better use of optimal flight levels in environments where a lack of air traffic service (ATS) surveillance service or the application of large separation minima were a limiting factor.

  - RTCA and EUROCAE developed a concept of operations for ITP and the requirements for the ITP equipment are specified in RTCA DO-312/EUROCAE ED-159, Safety Performance and Interoperability Requirements Document for the In-Trail Procedure in Oceanic Airspace Application and Supplement. Furthermore, the avionics standards for ITP can be found in RTCA DO-317A /EUROCAE 
ED-194, Minimum Operational Performance Standards (MOPS) for Aircraft Surveillance Applications (ASA) System and Supplement.

  - It has been determined that the best safe way to exchange information and clearances between flight crews and controllers is using Controller-Pilot Data Link Communications (CPDLC).  CPDLC messages have been developed for ITP.  Bearing in mind the challenges and costs related to the update of the aircraft message set, a series of standardized free text messages were developed.

- Discussion

  - The introduction of standardized free text messages requires an amendment to Annex 10 — Aeronautical Telecommunications, Volume II — Communication Procedures including those with PANS status.

  - The introduction of ADS-B ITP procedures requires an amendment to the Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM, Doc 4444).

  - ITP itself does not require pilots to monitor the ADS-B traffic display during the ITP manoeuvre (i.e. the climb or descent).  However, before starting an ITP maneuver, flight crew may use the ADS-B traffic display to improve situational awareness.  Since there is no provision in an ICAO document for the use of a cockpit traffic display by pilots, an amendment to the Procedures for Air Navigation Services — Aircraft Operations, Volume I — Flight Procedures (PANS-OPS, Doc 8168) is also proposed.

  - The ICAO Secretariat has prepared a paper proposing amendments to Annex 10 — Aeronautical Telecommunications, Volume II — Communication Procedures, Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM, Doc 4444) and the Procedures for Air Navigation Services — Aircraft Operations, Volume I — Flight Procedures (PANS-OPS, Doc 8168) on the subject of ADS-B (ITP) to facilitate enroute climb and descent in oceanic and remote continental airspace where the lack of ATS surveillance coverage is a limiting factor.

  - The ICAO Secretariat is proposing that these amendments be considered for final review by the Air Navigation Commission in the 194th Session, with an applicability date of 13 November 2014.

  - It will be up to the ATS unit to determine in which part of its airspace, if any, ADS-B ITP will be authorized.  For those ATS units planning to use ADS-B ITP, controller training will be required, including the use of standardized free text CPDLC messages related to ADS-B ITP.  System upgrades may be needed for ATS units, especially for those wanting to provide controllers with pre-formatted CPDLC messages, automatic clearance composition and automatic analyses of ADS-B ITP criteria.

- Conclusion

  - The meeting is invited to note the information provided.