THE FORTIETH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/40)

THE TWENTY-SEVENTH MEETING OF THE FANS

INTEROPERABILITY TEAM   (FIT/27)

(Washington, DC, USA  11 September 2014)

Agenda Item 3:  	Reports on the relevant activities

The results of the FOM analysis in Fukuoka FIR

(Presented by Civil Aviation Bureau of Japan)

SUMMARY

This paper presents the analysis based on FANS 1/A Operations Manual (FOM) regarding the aggregated communication performance of whole fleet in Fukuoka Flight Information Region (FIR).

- Introduction

  - This paper provides results of the performance evaluations of whole fleet based on FANS 1/A Operations Manual (FOM) in Fukuoka Flight Information Region (FIR). The purpose of this paper is to present the most recent observed performance of them.

  - The performance data observed from the Controller Pilot Data Link Communications (CPDLC) and Automatic Dependent Surveillance -Contract (ADS-C) at the Data Link Center System (DLCS) are measured against the appropriate criteria.( e.g. Round Trip Performance, Unsuccessful Rate)

  - This paper presents the aggregated communication performance of whole fleet in Fukuoka FIR.

- Discussion

  - The FOM provides guidance material describing the required data from the FANS 1/A aircraft communications addressing and reporting system (ACARS) messages. The FOM also describes the calculation process for the End-to-end round trip time for uplinks (Round Trip), End-to-end one way time for downlinks (One Way), and Uplink unsuccessful rate (Unsuccessful Rate).

  - Figure 1 presents the Round Trip measurement against FOM criteria in the Fukuoka FIR. The collection period is July 2013 to June 2014. The numbers of samples included in the analysis are 642,079 CPDLC messages and 1,558,650 ADS-C messages.

  - Figure 2 presents the One Way measurement against FOM criteria in the Fukuoka FIR. The collection period is July 2013 to June 2014. The numbers of samples included in the analysis are 739,577 CPDLC messages and 3,205,447 ADS-C messages.

  - Figure 3 presents the Unsuccessful Rate measurement against FOM criteria in the Fukuoka FIR. The collection period is July 2013 to June 2014.The numbers of samples included in the analysis are 646,465 CPDLC messages and 1,568,553 ADS-C messages.

Figure 1: Round Trip Performance

Figure 2: One Way Performance

Figure 3: Unsuccessful Rate

- Conclusion

  - The results of the performance evaluations comfortably meet FOM criteria.

  - The meeting is invited to note the information provided.

END