THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 6:  Evaluation of Costs and Benefits

North Pacific Airspace Cost Effectiveness (NPACE) Study – Mixed ADS Scenario Results

(Prepared by the FAA Technical Center and Rutgers University)

# Summary

This paper presents a summary of the activities related to the North Pacific Airspace Cost Effectiveness (NPACE) Study.  At IPACG/22 the NPACE Study Advisory Group (NSAG) determined that the next step for the NPACE study would be to examine two scenarios, one to determine the benefits of 30nm lateral separation, and the second to examine a mixed environment in which all aircraft are either RNP-10 or RNP-4 capable, using the current track system.  The focus of this paper is on the second scenario.

- Introduction

  - At IPACG/17, the meeting accepted a proposal for a study to investigate future changes to North Pacific airspace.  A similar study was conducted for the North Atlantic (NAT) airspace by the US Federal Aviation Administration (FAA) Technical Center and Rutgers University.  The meeting noted that it would be necessary to establish an advisory group consisting of airspace users and operational experts to provide advice and data sources necessary for successful completion of the NPACE study.  The meeting agreed that additional aspects of flight, not considered in the NAT Study, such as schedule integrity and operational performance outside oceanic airspace, would need to be included in the study (reference 1).  The system modeling for the NPACE study is being conducted by Rutgers University in conjunction with the FAA Technical Center.

  - At IPACG/18, the FAA presented the proposed terms of reference and work program for the NPACE Study (reference 2).  A list of proposed future separation reductions and system scenarios for the North Pacific (NOPAC) routes was also presented.  It was anticipated that the proposed separation and system scenarios would change and/or new ones be added as technology advances and the requirements of ATS providers and airspace users change.

  - At IPACG/19 (reference 3), it was requested that the North Pacific simulation model be used to study the effects of implementing separate tracks for automatic dependent surveillance (ADS)-equipped aircraft and non-ADS-equipped aircraft. This study is important because of the growing population of ADS-capable aircraft utilizing the Pacific airspace, and their increased communication capabilities that make it possible to reduce lateral and longitudinal separation standards. It was also requested that the simulation model be used to study the effects of having high-speed and low-speed tracks where aircraft are assigned to these tracks based on their cruising speeds. The results of the high-speed low-speed track study were presented at IPACG/20.  It was shown that there could be a small overall fuel savings and flight time savings if this scenario were to be implemented.  The meeting noted that the savings would be relatively small and that most of the aircraft operating in NOPAC would be assigned to tracks with their requested speeds (reference 4).

  - Regarding ADS and non-ADS equipped aircraft, it was decided at IPACG/20 that separate tracks should not be considered further and that instead the NPACE model should be modified and used to determine at what point implementing ADS technology will be necessary for all airlines (reference 4).  An advisory group was formed to provide technical and operational guidance for modifying the existing models developed by the NPACE Study.

  - The first NPACE Study Advisory Group (NSAG) meeting was held in Livermore, California on 13 January 2004 following an Oceanic Work Group (OWG) meeting.  Three specific simulation scenarios were established for the NPACE study. Additionally, information was provided by the group regarding air traffic control (ATC) procedures to be incorporated into the revised flight tracking program.

  - The second NSAG meeting was held in Livermore, California on 21 July 2004 following an OWG meeting. The modifications made to the model were presented and the preliminary results were discussed.

  - The third NSAG meeting was held in Livermore, California on 13 October 2004 following an OWG meeting. The final model modifications were presented and further results were discussed, including the supplementary results for the addition of the required navigation performance (RNP)-4 scenario.

  - Two conference calls were held on 10 January 2005 and 13 January 2005 where members of NSAG provided further input and discussed the results of the ADS scenarios.

1.9.	At IPACG/22, the results for the requested scenarios, involving the longitudinal separation standards available with RNP-10 and RNP-4 aircraft capabilities were presented. The results showed that there is a noticeable improvement in system performance when RNP-4 separation standards are used; as the percentage of ADS flights increases and the traffic level increases, the impact of RNP-4 separation standards compared with RNP-10 becomes more profound.  It was concluded by the IPACG group that the initial request had been fulfilled and that the results were acceptable, but it was also expressed by several of the members of the group that it would be desirable to expand the study to include the 30nm lateral separation standards available with the RNP-4 aircraft capability.

1.10.	The fourth NSAG meeting we held during the week of IPACG/22.  The purpose of the meeting was to define the future scenarios for the NPACE study.  Two scenarios were defined, one to determine the benefits of 30nm lateral separation, and the second to examine a mixed environment in which all aircraft are either RNP-10 or RNP-4 capable.

1.11.	The simulation scenarios and investigation procedure are outlined in section two of this paper, the results are presented in section three, and the summary and conclusions are discussed in section four.

2.	ADS Investigation

2.1.	The scenario established to investigate the North Pacific airspace system performance with varying percentages of RNP-4 and RNP-10 and future traffic densities is described as follows:

Scenario: Investigate the system performance under the forecasted traffic for the years 2005, 2010, 2015, 2020, and 2025, shown in Table 1, with 0%, 50%, 75%, and 100% of the flights as 30 nm ADS-equipped (RNP-4) and the remaining flights as 50 nm ADS-equipped (RNP-10).  Longitudinal separation standards are described in Table 2.

Table 1. Scenario Summary

| Year | 0% RNP-4 100% RNP-10 | 50% RNP-4 50% RNP-10 | 75% RNP-4 25% RNP-10 | 100% RNP-4 0% RNP-10 |
| --- | --- | --- | --- | --- |
| 2005 | 415 flights per day |  |  |  |
| 2010 | 530 flights per day |  |  |  |
| 2015 | 675 flights per day |  |  |  |
| 2020 | 850 flights per day |  |  |  |
| 2025 | 1000 flights per day |  |  |  |

Table 2. Longitudinal Separation Standards for RNP-10 and RNP-4 Capable Aircraft

(Lateral separation of 50 nm is accounted for by the placement of the tracks)

| In-trail Aircraft | Minimum Aircraft Separation |
| --- | --- |
| RNP-10/RNP-10 | 50 nm longitudinal |
| RNP-4/RNP-4 | 30 nm longitudinal |
| RNP-4/RNP-10 | 50 nm longitudinal |
| RNP-10/RNP-4 | 50 nm longitudinal |

2.2.	Flight events are first generated for a month of flights in the North Pacific airspace, utilizing the flight events program (reference 5). The simulated flight events utilize the historical data for the Pacific oceanic airspace for the year 2002, provided by the Anchorage and Oakland Air Route Traffic Control Centers (ARTCCs), referred to as Enhanced Traffic Management System (ETMS) data, and the Japan Civil Aviation Bureau (JCAB). The flight events program produces a specified number of flights closely imitating actual flights occurring during the same interval of time, and it can be adjusted to reflect a decrease or a growth in flight traffic for particular regions or origin-destination pairs based on traffic forecasting, as well as to account for the introduction of new aircraft types and withdrawal of older types.

2.3.	The first step of flight planning is to determine which track, out of a maximum of five possible for each flight, will be used in the subsequent processing. To accomplish this, each flight is run through the flight planning program for each possible track with the altitudes set at a constant 35,000 feet for the entire flight on all tracks to ensure an unbiased comparison. The track affording the least fuel consumption is chosen as the final track for that flight.

2.4.	The second step is the actual optimal flight planning in which the optimal altitude combination is found for the optimal track, where optimal denotes least fuel consumption. An altitude restriction subroutine in the flight planning program ensures that the maximum altitudes for the different aircraft types are not exceeded at oceanic entry or in-flight.

2.5.	The first step of flight tracking deals with the conflicts at oceanic entry. The flights are first arranged in order of entry time, and then the conflicts are detected based on the defined separation standards for each respective scenario and the Mach Number Technique (MNT).  Conflicts are resolved by adjusting the entry flight level, delaying the flight, or a combination of these two adjustments.

2.6.	The second step of flight tracking deals with the in-flight conflicts between in-trail flights. All occurrences of multiple flights arriving at the same waypoint, whether in-trail on the same track or crossing on different tracks, are considered in time order. Separation standards are established and calculated for in-trail flights using the specific scenario separation standards and the MNT, and a required separation time of 10 minutes is used for flights on different tracks crossing at the same waypoint. When a conflict is found, various combinations of flight level changes are considered to resolve the conflict while attempting to preserve the optimal flight plans of the flights involved as much as possible.

2.7.	After all conflicts have been detected and resolved using these methods, an algorithm similar to the second step of flight planning is run to evaluate the final flight plans and calculate the resulting fuel consumption and flight time.  The difference in these recalculated values provides an indication of the effect of the changing separation standards for increasing ADS-capable aircraft densities. In addition, statistics are generated, indicating the percent of flights altered by entry flight level changes and/or flight delays.

- Results

3.1.	Table 3 shows the results for the 2005 scenario, Table 4 shows the results for the 2010 scenario, Table 5 shows the results for the 2015 scenario, Table 6 shows the results for the 2020 scenario, and Table 7 contains the results for the 2025 scenario. Table 8 illustrates the increasing benefits of 100% RNP-4/ 0% RNP-10 versus 0% RNP-4/ 100% RNP-10 as the traffic densities increase.  The numbers represent the improvement in the respective performance measures between the two aforementioned RNP scenarios.

  - Figure 1 shows the percentage of flights delayed for all scenarios, Figure 2 shows the average delay time for all scenarios, Figure 3 shows the percentage of flights with entry level changes for all scenarios, Figure 4 shows the percentages of flights with in-flight level changes for all scenarios, and Figure 5 shows the average increase in fuel over the optimally projected fuel consumption, for all scenarios.

Table 3.  System performance results for 2005 (415 flights per day)

|  | 0% RNP-4 100% RNP-10 | 50% RNP-4 50% RNP-10 | 75% RNP-4 25% RNP-10 | 100% RNP-4 0% RNP-10 |
| --- | --- | --- | --- | --- |
| % Delayed | 12.9% | 12.4% | 11.3% | 10.2% |
| Average delay (min) | 5.5 | 5.1 | 4.7 | 4.0 |
| % With entry level change | 9.9% | 9.0% | 7.4% | 5.5% |
| % Deviating from optimal profile | 38.6% | 36.5% | 33.1% | 29.0% |
| Average fuel increase over optimal (gallons) | 143.5 | 137.9 | 131.7 | 121.9 |

Table 4.  System performance results for 2010 (530 flights per day)

|  | 0% RNP-4 100% RNP-10 | 50% RNP-4 50% RNP-10 | 75% RNP-4 25% RNP-10 | 100% RNP-4 0% RNP-10 |
| --- | --- | --- | --- | --- |
| % Delayed | 17.2% | 15.9% | 13.9% | 11.8% |
| Average delay (min) | 6.5 | 5.9 | 5.4 | 4.2 |
| % With entry level change | 11.5% | 10.5% | 9.0% | 6.6% |
| % Deviating from optimal profile | 43.7% | 41.7% | 38.1% | 32.3% |
| Average fuel increase over optimal (gallons) | 153.3 | 146.2 | 140.4 | 121.8 |

Table 5.  System performance results for 2015 (675 flights per day)

|  | 0% RNP-4 100% RNP-10 | 50% RNP-4 50% RNP-10 | 75% RNP-4 25% RNP-10 | 100% RNP-4 0% RNP-10 |
| --- | --- | --- | --- | --- |
| % Delayed | 21.4% | 20.1% | 18.9% | 15.8% |
| Average delay (min) | 8.7 | 7.8 | 6.8 | 5.1 |
| % With entry level change | 14.4% | 13.1% | 10.8% | 9.1% |
| % Deviating from optimal profile | 49.3% | 48.2% | 44.6% | 39.7% |
| Average fuel increase over optimal (gallons) | 163.4 | 155.8 | 148.0 | 134.9 |

Table 6.  System performance results for 2020 (850 flights per day)

|  | 0% RNP-4 100% RNP-10 | 50% RNP-4 50% RNP-10 | 75% RNP-4 25% RNP-10 | 100% RNP-4 0% RNP-10 |
| --- | --- | --- | --- | --- |
| % Delayed | 28.8% | 26.9% | 24.2% | 19.8% |
| Average delay (min) | 12.2 | 11.3 | 8.9 | 6.4 |
| % With entry level change | 16.6% | 15.2% | 13.6% | 10.4% |
| % Deviating from optimal profile | 54.2% | 52.4% | 48.2% | 43.7% |
| Average fuel increase over optimal (gallons) | 176.6 | 169.2 | 154.0 | 140.1 |

Table 7.  System performance results for 2025 (1000 flights per day)

|  | 0% RNP-4 100% RNP-10 | 50% RNP-4 50% RNP-10 | 75% RNP-4 25% RNP-10 | 100% RNP-4 0% RNP-10 |
| --- | --- | --- | --- | --- |
| % Delayed | 33.5% | 31.2% | 28.2% | 23.9% |
| Average delay (min) | 16.4 | 15.3 | 10.8 | 7.7 |
| % With entry level change | 18.0% | 17.0% | 15.3% | 12.7% |
| % Deviating from optimal profile | 57.7% | 55.2% | 52.2% | 47.8% |
| Average fuel increase over optimal (gallons) | 195.5 | 185.5 | 167.8 | 148.5 |

Table 8.  Performance improvement from 0% to 100% RNP-4 for all traffic densities

|  | 2005 | 2010 | 2015 | 2020 | 2025 |
| --- | --- | --- | --- | --- | --- |
| % Delayed | 2.7% | 5.4% | 5.6% | 9.0% | 9.6% |
| Average delay (min) | 1.4 | 2.2 | 3.6 | 5.9 | 8.8 |
| % With entry level change | 4.4% | 4.9% | 5.4% | 6.2% | 5.4% |
| % Deviating from optimal profile | 9.6% | 11.4% | 9.6% | 10.6% | 9.9% |
| Average fuel increase over optimal (gallons) | 21.7 | 31.5 | 28.6 | 36.5 | 47.0 |

Figure 1.  Percent of flights delayed

Figure 2.  Average delay time in minutes

Figure 3.  Percent of flights with entry level changes

Figure 4.  Percent of flights deviating from in-flight optimal profile

Figure 5.  Average fuel increase over optimal

4.	Summary and Conclusions

4.1	The information contained in this paper provides an update of the status of the NPACE study.  An advisory group was formed to provide technical and operational guidance to the NPACE Study team.  The most recent NSAG meeting was held in January 2005 during the week of IPACG/22.  At this meeting, the simulation scenarios were established for the NPACE study

4.2.			The results of the mixed ADS scenario show that for an environment of exclusively RNP-10 and RNP-4 ADS-capable aircraft, the percentage of flights changed from their optimal profile, through delays, entry level changes, and in-flight level changes, decreases as the percentage of RNP-4 ADS aircraft increases (while the percentage of RNP-10 ADS aircraft decreases). Because the 0% scenario corresponds to 100% RNP-10 ADS aircraft and the 100% scenario corresponds to 100% RNP-4 ADS aircraft, the magnitude of the improvement for the increasing RNP-4 aircraft is not substantial when compared with the results for the previously investigated scenarios involving non-ADS aircraft (reference 6), but clearly shows that with more RNP-4 ADS aircraft, the airspace experiences fewer overall changes.

4.3.			As shown in Table 8, the benefits experienced, for the 100% RNP-4/ 0% RNP-10 scenario versus the 0% RNP-4/ 100% RNP-10 scenario, increase as the traffic density increases for the percentage of flights delayed, the average delay time, and the average increase in fuel, but not for the entry and in-flight level change performance measures. However, the level change performance measures do not appear to affect the overall system performance, as the average increase in fuel, which is related to the level changes and is the more critical performance measure, demonstrates improvement. This can be explained by the fact that the level change performance measures merely reflect that a flight was altered, not how many changes were made to alter that flight.

4.4.	As illustrated through in Tables 3 through 7, in order to maintain the current levels of the percent of flights delayed, which is approximately 13%, the system would have to move to a 100% RNP-4 status in 2010 and maintain that status thereafter with the addition of further system enhancements.

5.	Recommendation

5.1.	The meeting is invited to note the information presented in this paper.

# References

- Summary of Discussions for the Seventeenth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/17), 15-19 April, 2002; San Francisco, CA, USA.

- Summary of Discussions for the Eighteenth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/18), 8-11 October 2002; Tokyo, Japan.

- Summary of Discussions for the Nineteenth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/19), 15-17 July 2003; Tokyo, Japan.

- Summary of Discussions for the Twentieth Meeting of the Informal Pacific Air Traffic Control Coordination Group (IPACG/20), 6-10 October 2003; Honolulu, Hawaii, USA.

- Flight Events Generation for Air Traffic Over the Northern Pacific Ocean, Final Report, Rutgers University and The Federal Aviation Administration Technical Center, 30 June 2003.

- Northern Pacific Airspace Cost Effectiveness (NPACE) Study: Modeling and Simulation of the Impact of Automatic Dependence Surveillance (ADS) Communication Technology, Final Report, Rutgers University and the Federal Aviation Administration Technical Center, 19 January 2005.