THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 2:  Air Traffic Management (ATM) issues

Implementation of 30nm Lateral and 30nm Longitudinal Separation in

US-controlled Flight Information Regions

Presented by the Federal Aviation Administration

SUMMARY

This paper provides information about the status of FAA 30nm lateral and 30nm longitudinal implementation in oceanic Flight Information Regions (FIRs) where the US provides air traffic services (ATS).

- Discussion

  - As reported at IPACG/22, the FAA plans to implement 30nm lateral/30nm longitudinal separation (30/30) throughout oceanic airspace where the US provides air traffic services (ATS), beginning with operational trials in portions of Oakland’s oceanic airspace, then expanding throughout the US-controlled Pacific and Atlantic FIRs.

  - To implement 30/30 in US controlled oceanic airspace, a task force has been formed and a task list, similar to that used for implementation of reduced vertical separation minimum (RVSM) has been developed.  Steps on the task list include completion of a required navigation performance (RNP)-4 operational approval process, completion of an appropritate safety assessment, and ICAO approval of the application in a specific airspace.

  - Implementation will begin with operational trials in the Oakland Air Route Traffic Control Center (ARTCC) oceanic sector 3 which spans that airspace roughly between California and Australia/New Zealand (see attachment).  Though the following Pacific Organized Track System (PACOTS) tracks in sector 3 are no longer routinely published, they are available on request.

Routes							 Track Designators

California to Australia/New Zealand  						W & X

Australia/New Zealand to California						20 & 21

  - Application of 30/30 in other oceanic airspace is beyond the scope of the initial operational trials and will not be authorized until further safety assessments are completed.  The 30/30 Task Force will continue to work on expanding the implementation of reduced horizontal separation in other airspace as appropriate.

  - Aircraft approved to operate with an RNP value of RNP-4 will immediately benefit from this reduction to current horizontal separation criteria.  Ocean21 will be able to verify which aircraft are equipped for various separation minima so that Oakland ARTCC will be able to apply 30/30 separation between pairs of RNP-4 approved aircraft.  Oakland ARTCC will also accommodate aircraft that are not approved for RNP-4 operations throughout its FIR, to include oceanic sector 3.  By improving the efficiency for some (e.g. RNP-4 approved) airspace users, other airspace users should benefit from the increased availability of efficient altitudes/routes/trajectories.

- Recommendation

  - The meeting is invited to note the information provided in this paper.