THE TWENTY-THIRD MEETING OF

THE INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

Tokyo, Japan, 11-15 July 2005

Agenda Item 2:	Air Traffic Management (ATM) Issues

ATC contingency procedures to be used

during failure of datalink in oceanic control airspace

(Presented by Loss of Data Link Task Force)

This paper provides information about draft ATC contingency procedures to be used during failure of datalink in the Pacific FIRs

1.	Background

1.1	At the IPACG/19 meeting, held in July 2003, JCAB requested to consider developing ATC contingency procedures to be mutually applied during failure of datalink within the Anchorage, Oakland and Tokyo FIRs.  The IPACG/19 agreed to develop such procedures, and a task force composed of representatives from Anchorage Center, Oakland Center, Tokyo Center, FAA and JCAB was formed to develop procedures.

2.      Summary

2.1     At the IPACG/21 meeting, JCAB presented an updated version of the draft ATC contingency procedures to be used during failure of data link in the Pacific FIRs and a draft amendment proposal to ICAO Regional Supplementary Procedures, Doc.7030.

2.2     The IFALPA representative questioned using the two-fold approach of a 500 feet level change and a lateral offset. He noted that there were different procedures for different environments and this would add another layer of confusion to pilots.

2.3     The IPACG/21 meeting suggested that the task force should continue discussion on this draft which reflected comments given at the ISPACG/18 meeting in February 2004 in Fiji, and should report a further update to IPACG.

2.4	It is considered crucial that such contingency procedures shall be agreed upon mutually by all concerned and be applied over the Pacific airspace in a harmonious way. Consultation with ICAO was initiated by presenting draft contingency procedures at the APANPIRG/15 in August 2004, and JCAB will continue in the necessary process of amendment of Doc.7030 once agreement is reached.

2.5     The further updated draft procedures were reviewed and approved by the IPACG/22 meeting in January 2005.

2.6     The proposed draft procedure were reviewed and approved by the ISPACG/19 meeting in March 2005.

2.7	JCAB submitted to ICAO this proposal amendment as attached to this paper on 26 April 2005.

3.	Recommendation

3.1     The meeting is invited to note the progress being made in the development ATC procedures to be used during failure of data link in oceanic control airspace in this paper.

--- END ---

Attachment to IPACG/22 WP/11

## Proposal for Amendment of

Regional Supplementary Procedures - Doc 7030/4

(Serial No. APAC-S xx/x)

| a) Regional Supplementary Procedures, Doc 7030/4: | MID/ASIA and PAC, as Amended No. 206 dated 26 September 2003 |
| --- | --- |
| b) Proposing State(s): | Japan |
| c) Proposed Amendment: | 1. On page MID/ASIA/RAC-5 dated 13/2/00, Add the following additional paragraphs. 4.4 ATC contingency procedures to be used during failure of datalink in oceanic control airspace 4.4.1 When failure of datalink communication is recognized, ATC and pilot shall: - Try to establish communication by other means. - If attempts to establish communication by other means are unsuccessful, the pilot shall follow the provisions detailed in Section 4.0 COMMON PROCEDURES FOR RADIO COMMUNICATIONS FAILURE (total loss of communication procedures being proposed by Australia, Fiji, France, Japan, New Zealand, the Philippines and the Unite States). - If communication by other means is established, and loss of datalink communication has resulted, or will result, in a traffic confliction: 1) ATC shall apply an alternate form of separation. 2) If an alternate form of separation cannot be applied immediately, a. ATC shall at a minimum: i) Assign a speed restriction to the pilot and, ii) Issue essential traffic information for all affected aircraft |
|  | 2. On page PAC/RAC-5 dated 30/11/01, Add the following additional paragraphs. 4.4 ATC contingency procedures to be used during failure of datalink in oceanic control airspace 4.4.1 When failure of datalink communication is recognized, ATC and pilot shall: - Try to establish communication by other means. - If attempts to establish communication by other means are unsuccessful, the pilot shall follow the provisions detailed in Section 4.0 COMMON PROCEDURES FOR RADIO COMMUNICATIONS FAILURE (total loss of communication procedures being proposed by Australia, Fiji, France, Japan, New Zealand, the Philippines and the Unite States). - If communication by other means is established, and the loss of datalink communication has resulted, or will result, in a traffic confliction: 1) ATC shall apply an alternate form of separation. 2) If an alternate form of separation cannot be applied immediately, a. ATC shall, at a minimum: i) Assign a speed restriction to the pilot and, ii) Issue essential traffic information for all affected aircraft |
| d) Proposer’s reasons for amendment: | a) ICAO procedures for the application of 50NM and 30NM longitudinal separation minima utilizing a datalink system (ADS and CPDLC) are detailed in the PANS-ATM, Doc 4444. The procedures require ATC intervention, via alternate means of communication, in the event that an ADS periodic report or waypoint change event is not received within a certain time (paragraph 5.4.2.6.4.3.3). Since the standard horizontal separation minima using other means of communication are larger than the minima using a datalink system, it is possible that alternate means of separation may not be immediately available, especially in a busy traffic environment. b) Failure of datalink communications may occur due to several reasons including; ground earth station (GES) failure, network failure and avionics failure. If a GES fails, aircraft will try to logon to another GES. According to a simulation conducted by the Electric Navigation Research Institute in Japan, it took, depending on the number of aircraft who try to logon, several dozen minutes to re-establish logon with another GES. c) There may be a case where an aircraft, (or several aircraft) suffering from a datalink system communications failure, may cross into the adjacent ACC’s FIR without standard separation It is considered therefore, that ATC contingency procedures should be uniformly applied in airspace where ATC datalink systems are being utilized and reduced horizontal separation minima are being applied. Japan, the United States and other Pacific States plan to implement reduced horizontal separation minima using datalink systems within their oceanic control airspace. The Informal Pacific ATC Coordinating Group (IPACG) and Informal South Pacific ATS coordinating Group (ISPACG) have agreed to the procedures contained in this proposal. |
| e) Proposed implementation date of the amendment: | Upon approval by the Council |
| f) Proposal circulated to the following Sates and International Organizations: |  |
| g) Secretariat comments: |  |