THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 2:	Air Traffic Management (ATM) Issues

Interim ATS Inter-facility Data Communications (AIDC) Functionality Between Tokyo ACC ODP and Oakland ARTCC Ocean21

(Presented by the Federal Aviation Administration)

SUMMARY

This paper describes the interim AIDC interface developed to support a legacy AIDC message service between the Oakland ARTCC Ocean21 system and the Tokyo ACC ODP system.

1.		Background

  - ATS Inter-facility Data Communications (AIDC) provides an automated means of flight data exchange between Area Control Centers (ACC). The AIDC service between Oakland Air Route Traffic Control Center (ARTCC) and Tokyo ACC was initiated in 1998 based on Version 1 of the Asia/Pacific AIDC Interface Control Document (ICD). During the development of this service both the JCAB and the FAA encountered technical problems in implementing the full ICD so the two agencies worked together to make changes to support each other. As a result of this cooperation the first automated data interface between foreign flight information regions (FIRs) was established and it has run successfully for over seven years.

  - The FAA will decommission the current flight data processor, Oceanic Display and Planning System (ODAPS), and transition to the replacement system, Ocean21, in 2005.

  - The Ocean21 system does not provide native support for the unique AIDC interface that is used between Oakland ARTCC, Anchorage ARTCC, and Tokyo ACC. In order to maintain this essential interface after transitioning to Ocean21 the FAA has developed a “translator” that is in the Ocean21 – Oceanic Data Processing system (ODP) data path.

- Discussion

  - The solution for this interface will be a new processor, ATS Interfacility Data Communications-Tokyo (AIDC-T).  The FAA has ported the current AIDC software to the new platform and modified the code so that it can maintain the currently deployed reduced message set and functionality.  AIDC-T will be positioned in the data path downstream from the Ocean21 as illustrated in Figure 1.

Figure 1.   Ocean21 - ODP Interim AIDC Data Path

  - The functional requirement for AIDC-T is that it shall present an interface to the Tokyo ODP that exactly matches the one that is in operation today.  This means that the messages exchanged for operational use will not change: CPL, EST, MAC, ACP, LAM, and LRM.

    - AIDC-T will have limited support for the MIS and EMG messages. AIDC-T can send and receive these messages but they will be displayed on a terminal that is not constantly monitored so they should only be used for test purposes.  Any need to send the information that would be contained in an MIS or EMG message during operational use should be relayed via voice circuits.  This follows current practice so it should not be viewed as a change.

  - During recent e-mail exchanges with JCAB the FAA has gained a more thorough understanding of the ODP AIDC interface and as a result has set further functional requirements for AIDC-T.

    - The AIDC-T system will not attempt to send ABI, CDN, REJ, TOC, AOC, or PAC messages.

    - Messages from AIDC-T will not contain ICAO subfields 7b (SSR mode), 7c (SSR code), or 16c (alternate aerodrome).

    - AIDC-T will not transmit the letter “D” as part of ICAO subfield 10b (surveillance equipment).

  - When data is required to be exchanged between Ocean21 and ODP the AIDC-T will provide conversion services between ICAO format and NAS format.  Additionally, AIDC-T will perform certain connection management functions.

    - For flights outbound from the Oakland FIR, 80 minutes prior to the estimated boundary crossing, Ocean21 will send AIDC-T the flight profile information in the FAA National Airspace System (NAS) format.  AIDC-T will then convert this data into a CPL message that is formatted to the same specification that Oakland ARTCC and Tokyo ACC are using today.  AIDC-T will transmit this CPL to Tokyo ACC (RJTGZOZO) and wait for an LAM or LRM.  If it receives a LAM it will also wait for an ACP.  If AIDC-T receives an ACP it will then transmit a LAM to Tokyo ACC. The AIDC-T will send status information to Ocean21 regarding this CPL/LAM/LRM/ACP messaging using the NAS protocol.

      - If there are any profile changes (e.g. altitude, time) for this flight Ocean21 will send the information to AIDC-T.  AIDC-T will compare the new profile information with the previous profile information to determine what has changed.

        - If the change involves only an altitude (14c) or boundary time estimate (14b) then AIDC-T will convert the data into an EST message that is formatted to the same specification that Oakland ARTCC and Tokyo ACC are using today.  AIDC-T will transmit this EST to Tokyo ACC (RJTGZOZO) and wait for an LAM or LRM.  If it receives a LAM it will also wait for an ACP.  If AIDC-T receives an ACP it will then transmit a LAM to Tokyo ACC. The AIDC-T will send status information to Ocean21 regarding this EST/LAM/LRM/ACP messaging using the NAS protocol.

        - If the profile change involves fields other than 14b or 14c AIDC-T will merge the new profile data with the previous profile data and convert the information to a CPL and follow the sequence described above (2.4.1).

        - One final possibility for this profile change information is that it indicates that the flight will no longer be entering the Tokyo FIR.  If this is the case then AIDC-T will transmit a MAC message that is formatted to the same specification that Oakland ARTCC and Tokyo ACC are using today.  AIDC-T will wait for an LAM or LRM and send status information to Ocean21 regarding this MAC/LAM/LRM messaging using the NAS protocol.

    - For flights inbound to the Oakland FIR AIDC-T will receive the initial CPL message from TYO ACC and convert the information to flight profile information in NAS format and send it to Ocean21.  If Ocean21 indicates that the data was acceptable AIDC-T will reply to TYO ACC with a LAM and ACP.  If Ocean21 indicates that the data was not acceptable AIDC-T will reply to TYO ACC with a LRM.

      - If AIDC-T receives additional CPL messages or any EST messages for this flight it will compare the data in the message with stored profile data from previous messages and convert the changed information into an amendment message in NAS format and send it to Ocean21.  Based on the reply from Ocean21, the AIDC-T processor will transmit a LAM/ACP message sequence or LRM message to the Tokyo ACC ODP.

      - If AIDC-T receives a MAC message for this flight it will convert this to flight profile amendment (cancel) data into NAS format and send it to Ocean21.  Based on the reply from Ocean21, the AIDC-T processor will transmit a LAM or LRM message to the Tokyo ACC ODP.

      - All of the reply messages sent from AIDC-T to the Tokyo ACC ODP will be formatted to the same specification that Oakland ARTCC and Tokyo ACC are using today.

  - The FAA has developed a transition plan to move the existing AIDC data streams from the current system to either the Ocean21 External Communications Server or the AIDC-T.  This plan will not require non-FAA organizations to make any modifications to their systems coincident to the cutover of the interfaces.

    - JCAB has been requested to change the address that the Tokyo ODP uses to transmit AIDC messages to Oakland ARTCC from the current “KZOAZOZF” to “KZOAAIDC”.  The FAA has made this request so that Tokyo ACC can make the address change to the ODP at a time that is convenient to them.  This change also allows Oakland ARTCC more flexibility regarding the exact time that the transition to Ocean21 is made.

      - The FAA AFTN gateway has been configured to presently send messages addressed to “KZOAZOZF” and “KZOAAIDC” to the ODAPS AIDC station as illustrated in Figure 2. During the transition these data streams will be split and redirected to different stations as illustrated in Figure 3.

Figure 2.   Pre-Transition Data Path

Figure 3.   Post-Transition Data Path

      - At the moment that Oakland ARTCC transitions the AIDC interface to the Ocean21 system the AFTN gateway technicians will be notified to implement a real-time modification to the station directing tables.  The “KZOAAIDC” message traffic (which would be from Tokyo ACC) will be redirected from the ODAPS station to the Ocean21 AIDC-T station.  The “KZOAZOZF” message traffic (which would be from all other ACCs) will be redirected from the ODAPS station to the Ocean21 External Communications Server to be handled by the native AIDC function.  If circumstances arise that require Oakland ARTCC to fallback to ODAPS this redirection can be easily reversed.

      - During recent e-mail exchanges with JCAB the FAA has been informed that the Tokyo ACC ODP requires the Originator Indicator (ICAO Doc. 4444 paragraph 11.2.1.2.5) on incoming messages to match the address that the ODP uses to send messages to that facility.

        - The FAA has modified the current AIDC software so that a simple adaptation change can be made to the Tokyo ACC parameters to set the Originator Indicator to “KZOAAIDC”.  The FAA will make this change to adaptation at the same time that Tokyo ACC implements the address change.

        - The FAA encourages the JCAB to make this modification at their earliest convenience.

- Recommendation

  - The meeting is invited to note the contents of this paper.