THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 2: Air Traffic Management (ATM) issues

Implementation of ADS 50NM longitudinal separation minimum in the Tokyo FIR

(Presented by Tokyo ACC)

## Summary

This paper reports two months research of implementation of ADS 50NM longitudinal separation minimum during level changes (step climb/descent) in Tokyo ACC oceanic airspace.

- Background

  - At the IPACG/22 meeting, held in Los Angeles in January 2005, Tokyo Area Control Center (ACC) presented the implementation of ADS 50NM longitudinal separation minimum in the Tokyo Oceanic Control Airspace.

  - Tokyo ACC has started to apply ADS 50NM longitudinal separation minimum during level changes (step climb/descent) on two NOPAC routes in the Tokyo Oceanic Control Airspace on 11 April, 2005.

  - Applicable ATS routes : R220 (include in R217) / R580 (except converging to or crossing the airways)

Applicable aircraft: Data Link (ADS and CPDLC capable) aircraft

  - The total number of traffic in oceanic airspace, Tokyo FIR is approximately 440 per day, the number of traffic account for 20.3% on R220, 5.1% on R580 respectively.

  - The ratio of Data Link(ADS and CPDLC capable) aircraft

1.5.1  The ratio of Data Link in the Tokyo Oceanic Control Airspace is 43%, 190 flight

per day.

    - The ratio of Data Link is 46.9% on R220, 74.2% on R580 respectively.

（As of April ’05）

- Observations

  - The survey regarding ADS50NM application was conducted during two months, from 11 April to 10 June, 2005.

  - The data were collected manually by controller at the Oceanic sector when they cleared to change altitude by using ADS50NM longitudinal separation minimum.

  - The total number of recorded cases during two months is 56; the average of application is one case per day.

(See Attachment-1)

  - The cases of applying ADS50NM minimum are as follows;

2.4.1  Longitudinal separation is less than 10 minutes

2.4.2  Longitudinal separation is between 10 minutes to 14 minutes, however it is difficult to utilize Mach Control procedure due to preceding aircraft is much slower than succeeding aircraft.

(For example; the preceding aircraft is B767 (M081) and the following aircraft is B747 (M085))

(See Attachment-2)

- Comment

  - The meeting is invited to note the information in this paper and comment on it.

  - The case of applying ADS50NM is expected to increase when the implementation of ADS50NM minimum expands to other routes and cruising situation as well as the increase of ratio of Data Link aircraft.