# THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 2: Air Traffic Management (ATM) issues

International Route Reservation Service (IRRES)

Study Report

(Presented by Civil Aviation Bureau, Japan)

Summary

This paper presents the information on our approach and study for applying International Route Reservation Service (IRRES) to eastbound flights in North and Central Pacific, planned as a service enhancement of ATM Center that will be established in October 2005.

1. Purpose of this study

This study is designed for the purpose of enhancement of the capacity on the NOPAC Composite Routes System and PACOTS (Pacific Organized Track System) as well as eases the workload of controllers by commutating the flow in the said Route/ Track System with full utilization of ATM Center’s systems

2. Recent Enhancement Procedures

2.1 Typical problem for oceanic control in Tokyo FIR are in the following points:

- altitude selected in the flight plan to cross over gateway fix tends to be almost the same

for flights having the same destination and,

- even though those estimated time over the gateway fix are concentrating into a short

period, they are dispersed considerably(Poisson distribution) and,

- controllers find it difficult for effective altitude assignment in such situation and,

- inefficient use of airspace starts with unnecessary longitudinal separation.

2.2 Introduction of RVSM in this airspace in the year of 2000 mitigated constraint of traffic congestion up to a certain level.

But still controllers are tackling a problem that aircraft are concentrating into the same altitudes within a short period because they are overflights converging into same gateway fix from different routes/FIRs, and pilots are not completely satisfied with their altitude clearance and delay.

2.3 Future implementation of ADS separation will be a key for better solutions.

But still there will be a remaining problem of mixed performance environment(equipage　of ADS) that will make controllers applying complex separation standard, and inefficiency will still be inevitable.

2.4 Our study is, in addition to RVSM and ADS separation, to find effective use of airspace by

providing clear-cut answers to problems in 2.2 and 2.3 above.

3. Forecast

3.1 (Figure 1)

Figure 1

shows the computed forecast of

theoretical delay time when every flight

concentrates into the optimal altitude at each

gateway fix designated for eastbound NOPAC

and PACOTS route system with standard 15

minutes longitudinal separation.

The rate is calculated by following form:

longitudinal separation × number of flight per 1 hr. 60 min.

It means that delay time increases rapidly while rising rate of using. To solve this, two methods are devised.

Figure 2

3.2 One is a method of reducing the longitudinal

separation. As for this, reduction of the

longitudinal separation will be prepared by

operating MTSAT (Multi-functional Transport

Satellite). And we will examine shorter

longitudinal separation by using ADS (Automatic

Dependent Surveillance) afterwards in the future.

(Figure 2) shows same forecast in (Figure 1) with

longitudinal separation of 8 minutes.

Delay will be decreased by the effect of shortening the time interval compared with (Figure 1). However, it means that the steep curve (increase of delay) will sooner or later occur when the rate rises also.

And the model shown in (Figure 2) is limited under condition that all aircraft should meet

the ADS requirement applicable for playing these effects to its maximum extent with the

reduced longitudinal separation minima between each other.

We predict several years of transitional period might be required for the airline operators to

equip with ADS capable avionics or to introduce new fleet for exhibition of the maximum

effect of new separation standard. For the time being, the situation in North Pacific expected

to be rather (Figure 1) than to (Figure 2).

Figure 3

3.3 we also forecast an increase of the traffic demand in the future (Figure 3). We counted the average number of aircraft at the peak time based on the number of flight in 2004 over each gateway fix to NOPAC and PACOTS: OATIS, PABBA, SCORE, VACKY, SMOLT and FORDO.  Each line indicates the growth in 15 years on the said routes based on ICAO Demand Forecast of "7.0%" in Asia Pacific Region.

In conjunction with (Figure 1.) and (Figure 2.), it is expected that the number of flight, whose assigned altitude is different from their proposed altitude,        will increase considerably,

or they need to accept en-route delay and/or departure delay as long as they stick to the

original plans because of fuel considerations, especially for those flights through PABBA

and SCORE. And it is expected the similar situation will be observed over the other 4 fixes

when traffic demand reaches such level as at PABBA and SCORE in the near future.

3.4 Another approach is to commutating the flow of the aircraft such as in Track Advisory

Service in Oakland ARTCC. For the provision of the service, the important factor is how we

can reduce unnecessary spacing between aircraft because of random arrival over gateway

fix as we observed in present daily operations. To make full use of optimal altitude or

altitude next to it, it is necessary to make aircraft fly in accordance with the arranged

gateway fix passing plan as precise as possible by forecasting the traffic flow beforehand

with ATM Center’s system development and sophisticated CDM tools, because in-flight

aircraft has limited means for adjusting the time over gateway fix except for consumption

of unnecessary fuel.

4 Requirement for system development and operational procedures

4.1 Our study group, that consist with ATC specialists, technical providers, research institute

and several airline operations staffs, concluded in its intermediate report that the

development of the system and operational procedure should be made in the following

manner:

- to meet and reflect the intentions and requirements of the airline operators as much as

possible on both fuel considerations and on time performance.

- to introduce the reservation procedure of 4th Dimension to fly efficiently.

- the number of aircraft that has optimal altitude/s can be increased by the procedure, and as a result, it should be able to answer a lot to the demand of the airspace customers

- to lift the controllers’ workload to satisfactory level.

4.2 We will furnish any result to concerning parties for further discussion when ongoing study

produces any fruit.

5. Conclusion

5.1 We believe that the IRRES will be the answer to enhance the capacity of the oceanic airspace.

5.2 We also believe that it is the responsibility as the ATM Service Provider of the said airspace

to study and set up such alternative/supplementary procedures in preparation for the

situation when the traffic demand exceeds in a manner shown in (Figure 1) or (Figure 2).

5.3 Our study is focusing on the operational aspect to seek a higher probability of flight operations to meet with the result calculated by advanced and planned system.