THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

(Los Angeles, USA, 25-28 January 2005)

Agenda Item 2:	Air Traffic Management (ATM) Issues

Lateral Offset and Expansion of Exclusive RVSM Airspace to FL410

(Presented by the Federal Aviation Administration)

SUMMARY

This paper provides an overview of the Notice to Airmen (NOTAM) that FAA published on 23 Dec 2004, concerning the strategic lateral offset procedure and the expansion of exclusive reduced vertical separation minima (RVSM) airspace to FL410 in Pacific oceanic flight information regions (FIRs) where FAA provides air traffic services.

1.		Background

1.1	FAA published the attached NOTAM that describes the strategic lateral offset procedure and expansion of exclusive RVSM airspace in US-controlled Pacific oceanic airspace, effective 20 Jan 2005.  This NOTAM, published on 23 Dec 2004, is at Attachment 1, and also available in the FAA NOTAM publication at http://www.faa.gov/ntap/NTAP04DEC23/PART3_SEC2.HTM

1.2	The strategic lateral offset procedure has been in use in North Atlantic airspace (see “Operational Policy and Procedures for the West Atlantic Route System (WATRS), the New York Oceanic FIR, the San Juan FIR and Atlantic High Offshore Airspace” in NOTAM publication cited above) and will be applied in other Pacific oceanic FIRs in March 2005.

2.	Discussion

2.1	The strategic lateral offset procedure and expansion of exclusive RVSM airspace will also be published in the Pacific and Atlantic Chart Supplements.

2.2	A concern about the implementation of the strategic lateral offset procedures was published in the 4-10 Jan 2005 edition of Flight International, where editor David Learmount wrote the following:

“Pilots employing lateral offsets on North Atlantic tracks (NAT) have been wrongly using individual interpretations of how the International Civil Aviation Organisation procedure should be flown, says the UK Civil Aviation Authority in a flight operations department communication. The CAA says it has not been made clear that the prescribed offset distances are designed to be followed precisely, not approximately at pilots' discretion.

Strategic lateral offset procedures (SLOP) are used to reduce conflict risks introduced by the sheer accuracy of modern navigation aids, which tend to put on-track aircraft precisely above or below each other, increasing the risk of collision if either aircraft diverges from its cleared height. The CAA has recommended that airlines review their operations manuals to ensure SLOP instructions are clear, and that the procedures should be ‘incorporated in all initial, conversion and recurrent training’.

Only three choices are laid down by ICAO for correct SLOPs, says the CAA: on-track; 1nm (1.85km) right of track; or 2nm right of track. In choosing which to employ, pilots can use visual means, traffic alert and collision avoidance system information or communication with the other aircraft, but they must stick to the exact prescribed offset distances and never go left of track nor further right than the 2nm maximum. If a flight management system cannot be programmed for automatic offset, the pilot must fly the centreline, says the rule.”

2.3	Exclusive RVSM airspace has already been applied in Atlantic oceanic airspace from FL290-FL410 inclusive.  With implementation of RVSM from FL290-FL410 throughout North, South and Central America on 20 Jan 2005, as well as RVSM implementations elsewhere within the Asia Pacific Region, the Japan Civil Aviation Bureau requested that FAA convert FL410 to exclusive RVSM airspace.  After coordination with national and international business aircraft organizations, the main proponents of the exclusion agreed with the change.

3.	Recommendation

  - The meeting is invited to note the information in this paper.

SPECIAL NOTICE - RVSM AIRSPACE IN THE PACIFIC REGION (PAC)

a. Australia, Fiji, New Zealand, Tahiti, the United States, Japan, Philippines, Indonesia, and Papua New Guinea have implemented RVSM within specified areas of their Flight Information Regions (FIR) at specified levels.

b. RVSM airspace is prescribed within the Oakland Oceanic FIR and Anchorage Oceanic FIR within controlled airspace between FL290 and FL410 inclusive. The flight level orientation scheme (FLOS) is single alternate, per ICAO Annex 2, Appendix 3a.

c. Airworthiness and Operational Approval and Monitoring

1. Operators must obtain operational approval from the State of Registry or State of the Operator, as appropriate, to conduct RVSM operations. On behalf of the Pacific Air Traffic Service Providers, the FAA is maintaining a website containing documents and policy for RVSM approval.

The address is: www.faa.gov/ats/ato/rvsm1.htm. In the Pacific RVSM Documentation section, "Documents and Process for Pacific RVSM Aircraft and Operator Approval" provides an outline of approval process events with references to related documents.

2. Airborne Collision Avoidance System II (ACAS II). (TCAS II, Version 7.0 meets the ICAO ACAS II standard).

(a) U.S. operators flying in airspace where RVSM is applied must comply with Part 91 Appendix G (Operations in RVSM Airspace). Appendix G states that unless otherwise authorized by the FAA, aircraft equipped with TCAS II and used in RVSM operations must incorporate Version 7.0 or a later version. For operations within other countries,

Part 91 Section 91.703 requires U.S. operators to "...comply with the regulations relating to flight and maneuver of aircraft there in force".

(b) Non-U.S. Operators should confirm ACAS II equipage requirements applicable to them with the responsible State authority. Many countries have adopted the ICAO Annex 6, Part I (International Commercial Air Transport Airplanes) standard: from 1 January 2005, turbine-engined airplanes with a maximum certificated take-off mass in excess of 5,700 kg or authorized to carry more than 19 passengers shall be equipped with ACAS II.

3. An essential part of the implementation of RVSM is the ability to monitor aircraft height to ensure that the aircraft height-keeping performance standard is being met. The Asia Pacific Approvals Registry and Monitoring Organization (APARMO) will process the results of monitoring. For further information on RVSM monitoring, the APARMO web site is: www.tc.faa.gov/act500/rvsm/aparmo_intro.html.

d. In-flight Procedures Within RVSM Airspace

1. Before entering RVSM airspace, the pilot should review the status of required equipment. (See Appendix 4 of FAA Guidance 91-RVSM for pilot RVSM procedures). The following equipment should be operating normally:

(a) Two primary altimetry systems.

(b) One automatic altitude-keeping device.

(c) One altitude-alerting device.

2. The pilot must notify ATC whenever the aircraft:

(a) Is no longer RVSM compliant due to equipment failure.

(b) Experiences loss of redundancy of altimetry systems.

(c) Encounters turbulence that affects the capability to maintain flight level.

(See Appendix 5 of FAA Guidance 91-RVSM for pilot and controller actions in such contingencies).

3. During cleared transition between levels, the aircraft should not overshoot or undershoot the assigned FL by more than 150 ft. (45 m).

4. Pilot Level Call. Except in an ADS or radar environment, pilots shall report reaching any altitude assigned within RVSM airspace.

e. Special Procedures for In-flight Contingencies. This paragraph and paragraphs f, g, and h below contain procedures for in-flight contingencies that have been updated for RVSM operations.

General Procedures

1. The following general procedures apply to both subsonic and supersonic aircraft and are intended as guidance only. Although all possible contingencies cannot be covered, they provide for cases of inability to maintain assigned level due to weather, aircraft performance, pressurization failure and problems associated with high-level supersonic flight. They are applicable primarily when rapid descent and/or turn-back or diversion to an alternate airport are required. The pilot's judgment shall determine the sequence of actions taken, taking into account specific circumstances.

2. If an aircraft is unable to continue flight in accordance with its air traffic control clearance, a revised clearance shall, whenever possible, be obtained prior to initiating any action, using a distress or urgency signal, as appropriate.

3. If prior clearance cannot be obtained, an ATC clearance shall be obtained at the earliest possible time and, until a revised clearance is received, the pilot shall:

(a) If possible, deviate away from an organized track or route system.

(b) Establish communications with and alert nearby aircraft by broadcasting, at suitable intervals: flight identification, flight level, aircraft position (including the ATS route designator or the track code), and intentions on the frequency in use, as well as on frequency 121.5 MHz (or, as a back-up, the VHF inter-pilot air-to-air frequency 123.45).

(c) Watch for conflicting traffic both visually and by reference to ACAS (if equipped).

(d) Turn on all aircraft exterior lights (commensurate with appropriate operating limitations).

f. Special Procedures for Subsonic Aircraft Requiring Rapid Descent and/or Turn-back or Diversion To an Alternate Airport Due to Aircraft System Malfunction or other Contingencies

Initial Action

1. If unable to comply with the provisions of paragraph e.2 to obtain a revised ATC clearance, the aircraft should leave its assigned route or track by turning 90 degrees right or left whenever this is possible. The direction of the turn should be determined by the position of the aircraft relative to any organized route or track system; e.g., whether the aircraft is outside, at the edge of, or within the system. Other factors to consider are terrain clearance and the levels allocated to adjacent routes or tracks.

Subsequent Action

2. AIRCRAFT ABLE TO MAINTAIN LEVEL. An aircraft able to maintain its assigned level should acquire and maintain, in either direction, a track laterally separated by 25 NM from its assigned route or track and once established on the offset track, climb or descend 500 ft. (150 m).

3. AIRCRAFT UNABLE TO MAINTAIN LEVEL. An aircraft NOT able to maintain its assigned level should, whenever possible, minimize its rate of descent while turning to acquire and maintain in either direction a track laterally separated by 25 NM from its assigned route or track. For subsequent level flight, a level should be selected which differs by 500 ft. (150 m) from those normally used.

4. DIVERSION ACROSS THE FLOW OF ADJACENT TRAFFIC. Before commencing a diversion across the flow of adjacent traffic, the aircraft should, while maintaining the 25 NM offset, expedite climb above or descent below levels where the majority of Pacific oceanic traffic operates (e.g., to a level above FL400 or below FL290) and then maintain a level which differs by 500 ft. (150 m) from those normally used. However, if the pilot is unable or unwilling to carry out a major climb or descent, the aircraft should be flown at a level 500 ft. above or below levels normally used until a new ATC clearance is obtained.

5. EXTENDED RANGE OPERATIONS BY TWO-TURBINE POWER UNITS (ETOPS) AIRCRAFT. If these contingency procedures are employed by a twin engine aircraft as a result of an engine shutdown or a failure of an ETOPS critical system, the pilot should advise ATC as soon as practicable of the situation, reminding ATC of the type of aircraft involved and requesting expeditious handling.

g. Weather Deviation Procedures for Oceanic-Controlled Airspace

General

1. The following procedures are intended to provide guidance. All possible circumstances cannot be covered. The pilot's judgment shall ultimately determine the sequence of actions taken and ATC shall render all possible assistance.

2. If the aircraft is required to deviate from track to avoid weather and prior clearance cannot be obtained, an air traffic control clearance shall be obtained at the earliest possible time. In the meantime, the aircraft shall follow the procedures detailed in paragraph g.8 below.

3. The pilot shall advise ATC when weather deviation is no longer required, or when a weather deviation has been completed and the aircraft has returned to the centerline of its cleared route.

Obtaining Priority from ATC when Weather Deviation Is Required

4. When the pilot initiates communications with ATC, rapid response may be obtained by stating "WEATHER DEVIATION REQUIRED" to indicate that priority is desired on the frequency and for ATC response.

5. The pilot still retains the option of initiating the communications using the urgency call "PAN PAN PAN" (preferably spoken three times) to alert all listening parties to a special handling condition which will receive ATC priority for issuance of a clearance or assistance.

Actions To Be Taken when Controller-Pilot Communications Are Established

6. The pilot notifies ATC and requests clearance to deviate from track, advising, when possible, the extent of the deviation expected. ATC will take one of the following actions:

(a) If there is no conflicting traffic in the horizontal dimension, ATC will issue clearance to deviate from track, or

(b) If there is conflicting traffic in the horizontal dimension, ATC will separate aircraft by establishing vertical separation, or

(c) If there is conflicting traffic in the horizontal dimension and ATC is unable to establish vertical separation, ATC shall:

(1) Advise the pilot unable to issue clearance for requested deviation.

(2) Advise pilot of conflicting traffic.

(3) Request pilot's intentions.

PHRASEOLOGY-
"Unable (requested deviation), traffic is (call sign, position, altitude, direction), advise intentions."

7. The pilot will take the following actions:

(a) Advise ATC of intentions by the most expeditious means available.

(b) Comply with air traffic control clearance issued, or

(c) Execute the procedures detailed in para 8(a) below. (ATC will issue essential traffic information to all affected aircraft.)

(d) If necessary, establish voice communications with ATC to expedite dialogue on the situation.

Actions To Be Taken if a Revised Air Traffic Control Clearance Cannot Be Obtained:

8. The pilot shall take the actions listed below under the provision that the pilot may deviate from rules of the air (e.g., the requirement to operate on route or track centerline unless otherwise directed by ATC), when it is absolutely necessary in the interests of safety to do so.

(a) If a revised air traffic control clearance cannot be obtained and deviation from track is required to avoid weather, the pilot shall take the following actions:

(1) If possible, deviate away from an organized track or route system.

| Route center line track | Deviations >10 NM | Level change |
| --- | --- | --- |
| EAST (000-179 magnetic) | LEFT RIGHT | DESCEND 300 ft CLIMB 300 ft |
| WEST (180-359 magnetic) | LEFT RIGHT | CLIMB 300 ft DESCEND 300 ft |

NOTE-
Subparagraphs 8(a)(2) and 8(a)(3) below call for the pilot to: broadcast aircraft position and pilot's intentions, identify conflicting traffic and communicate air-to-air with near-by aircraft. If the pilot determines that there is another aircraft at or near the same FL with which his aircraft might conflict, then the pilot is expected to adjust the path of the aircraft, as necessary, to avoid conflict.

(2) Establish communication with and alert nearby aircraft by broadcasting, at suitable intervals: flight identification, flight level, aircraft position (including the ATS route designator or the track code), and intentions (including the magnitude of the deviation expected) on the frequency in use, as well as on frequency 121.5 MHz (or, as a back-up, the VHF inter-pilot air-to-air frequency 123.45).

(3) Watch for conflicting traffic both visually and by reference to ACAS (if equipped).

(4) Turn on all aircraft exterior lights (commensurate with appropriate operating limitations).

(5) For deviations of less than 10 NM, aircraft should remain at the level assigned by ATC.

(6) For deviations of greater than 10 NM, when the aircraft is approximately 10 NM from track, initiate a level change based on the criteria in the table below.

(7) If contact was not established prior to deviating, continue to attempt to contact ATC to obtain a clearance. If contact was established, continue to keep ATC advised of intentions and obtain essential traffic information.

(8) When returning to track, be at its assigned flight level, when the aircraft is within approximately 10 NM of centerline.

h. STRATEGIC LATERAL OFFSETS IN OCEANIC AIRSPACE TO MITIGATE WAKE TURBULENCE AND TO MITIGATE COLLISION RISK

1. Pilots should use the Strategic Lateral Offset Procedure as standard operating practice in the course of normal oceanic operations to mitigate collision risk and wake turbulence. The Strategic Lateral Offset Procedure will be applied throughout the Oakland and Anchorage oceanic FIRs. This procedure is to be used for both wake vortex encounters, and to mitigate the heightened risk of collision when non-normal events such as operational altitude deviation errors and turbulence induced altitude deviations occur.

2. Strategic Lateral Offset Procedures will be applied using the following guidelines:

(a) Strategic lateral offsets executed to mitigate collision risk and those executed to mitigate the effects of wake turbulence are to be made to the right of a route or track;

(b) In relation to a route or track, there are three positions that an aircraft may fly: centerline, 1 NM or 2 NM right; and,

(c) Offsets are not to exceed 2 NM right of centerline.

3. The intent of this procedure is to reduce risk (increase the safety margin) by distributing aircraft laterally and equally across the three available positions. In this connection, pilots must take account of the following:

(a) Aircraft without automatic offset programming capability must fly the centerline;

(b) Aircraft capable of being programmed with automatic offsets may fly the centerline or offset 1 NM or 2 NM right of centerline to obtain lateral spacing from nearby aircraft;

(c) Pilots should use whatever means are available (e.g. communications, visual acquisition, GPWS or TCAS/ACAS) to determine the best flight path to fly;

(d) Any aircraft overtaking another aircraft is to offset within the confines of this procedure, if capable, so as to create the least amount of wake turbulence for the aircraft being overtaken;

(e) For wake turbulence purposes, pilots are also to fly one of the three positions at 2b above and never offset to the left of centerline nor offset more than 2 NM right of centerline;

NOTE. It is recognized that the pilot will use his/her judgment to determine the action most appropriate to any given situation and has the final authority and responsibility for the safe operation of the aeroplane. The use of air-to-air channel, 123.45, may be used to co-ordinate the best wake turbulence offset option.

(f) Pilots may apply an offset outbound at the oceanic entry point but must return to centerline at the oceanic exit point.

(g) Aircraft transiting radar-controlled airspace (e.g. Guam or Vancouver Center) may remain on their established offset positions but must advise the radar controller on initial contact of their offset status;

(h) There is no ATC clearance required for this procedure and, except as stated in paragraph (g), above it is not necessary that ATC be advised; and,

(i) Voice position reports are to be based on the current ATC route/course clearance and not the exact co-ordinates of the offset position.

i. Flight Planning

1. RVSM approval is required for aircraft to operate within RVSM airspace. The operator must determine that the appropriate State authority has approved the aircraft and will meet the RVSM requirements for the filed route of flight and any planned alternate routes. The letter "W" shall be inserted in item 10 (Equipment) of the ICAO standard flight plan to indicate RVSM approved aircraft.

2. Non-RVSM Aircraft.

Effective January 20, 2005 at 0901 UTC, Non-RVSM civil aircraft may not flight plan between FL290 and FL410 except under the conditions detailed below. Until that date, Non-RVSM civil aircraft may not flight plan between FL290 and FL390 inclusive within RVSM airspace except when unable to fly to an appropriate destination at or below FL280 and unable to fly at or above FL430 provided that the aircraft:

(1) Is being initially delivered to the State of Registry or Operator.

(2) Was formerly RVSM approved but has experienced an equipment failure and is being flown to a maintenance facility for repair in order to meet RVSM requirements and/or obtain approval.

(3) Is being utilized for mercy or humanitarian purposes.

(4) Is transporting a spare engine mounted under the wing.

3. Aircraft operators requesting approval as above shall:

(a) If departing within Oakland FIR or Anchorage FIR, obtain approval from the appropriate Oceanic Control Center normally not more than 12 hrs. and not less than 4 hrs. prior to the intended departure time.

(b) If transiting Oakland FIR or Anchorage FIR, notify the appropriate Oceanic Control Center after approval is received from the first affected Center and prior to departure. (Note that filing of the flight plan is not appropriate notification).

(c) Include the remarks "APVD non-RVSM" in Field 18 of the ICAO Flight Plan.

4. Contact details for approval request or notification are as follows:

Oakland ARTCC Telephone: 1-510-745-3342
AFTN: KZOAZRZX
FAX: 1-510-745-3411

Anchorage ARTCC Telephone: 1-907-269-1108
AFTN: PAZAZQZX
FAX: 1-907-269-1343

5. Non-RVSM aircraft operating in the RVSM stratum will be separated from all other aircraft by a minimum 2,000 ft vertical separation.

6. This approval process is intended exclusively for the purposes indicated above and not as a means to circumvent the normal RVSM approval process.

j. State Aircraft That Are Not RVSM Compliant

Operators of State aircraft (military, customs, police service) that are not RVSM compliant may flight plan within Oakland or Anchorage Oceanic FIR RVSM airspace. "State non-RVSM" should be added to the remarks section of the flight plan.

k. Operation of Non-RVSM Aircraft Within RVSM Airspace

1. Vertical separation applied. It should be noted that RVSM approved aircraft will be given priority for level allocation over non-RVSM approved aircraft. The vertical separation minimum between non-RVSM aircraft operating in the RVSM stratum and all other aircraft is 2,000 ft.

2. Climb and descent through RVSM airspace. Non-RVSM compliant aircraft may be cleared to climb to and operate at or above FL430 or descend to and operate at or below FL280 provided that they:

(a) Do not climb or descend at less than standard rate.

(b) Do not level off at an intermediate level while passing through the RVSM stratum.

l. Procedures for Suspension of RVSM

Air traffic services will consider suspending RVSM procedures within affected areas of the Oakland FIR or Anchorage FIR when there are pilot reports of greater than moderate turbulence. Within areas where RVSM procedures are suspended, the vertical separation minimum between all aircraft will be 2,000 ft. (ATO-E Oceanic Standards Branch 11/24/04)