THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

(Los Angeles, CA, USA, 24 - 28 January 2005)

Agenda Item 6:  Evaluation of Costs and Benefits

North Pacific Airspace Cost Effectiveness (NPACE) Study – ADS Results

(Prepared by the FAA Technical Center and Rutgers University)

# Summary

This paper presents a summary of the activity related to the North Pacific Airspace Cost Effectiveness (NPACE) Study.  Following the requests made at IPACG/20 regarding the integration of ADS aircraft into North Pacific airspace, an advisory group, entitled the NPACE Study Advisory Group (NSAG), was formed to help define the investigation scenarios and provide technical guidance for the model. The scenario definitions have evolved to include both the 50 nm separation standards (RNP-10 scenario) and the 30 nm separation standards (RNP-4 scenario), for the increasing percentages of ADS-capable aircraft of 0%, 35%, 65%, and 100%, and the increasing traffic densities for the years from 2005 to 2025.  This paper presents a description of the scenarios and the results.

- Introduction

  - At IPACG/17, the meeting accepted a proposal for a study to investigate future changes to North Pacific airspace.  A similar study was conducted for the North Atlantic (NAT) airspace by the US Federal Aviation Administration (FAA) Technical Center and Rutgers University.  The meeting noted that it would be necessary to establish an advisory group consisting of airspace users and operational experts to provide advice and data sources necessary for successful completion of the NPACE study.  The meeting agreed that additional aspects of flight, not considered in the NAT Study, such as schedule integrity and operational performance outside oceanic airspace, would need to be included in the study (reference 1).  The system modeling for the NPACE study is being conducted by Rutgers University in conjunction with the FAA Technical Center.

  - At IPACG/18, the FAA presented the proposed terms of reference and work program for the NPACE Study (reference 2).  A list of proposed future separation reductions and system scenarios for the North Pacific (NOPAC) routes was also presented.  It was anticipated that the proposed separation and system scenarios would change and/or new ones be added as technology advances and the requirements of ATS providers and airspace users change.

  - At IPACG/19 (reference 3), it was requested that the North Pacific simulation model be used to study the effects of implementing separate tracks for automatic dependent surveillance (ADS)-equipped aircraft and non-ADS-equipped aircraft. This study is important because of the growing population of ADS-capable aircraft utilizing the Pacific airspace, and their increased communication capabilities that make it possible to reduce lateral and longitudinal separation standards. It was also requested that the simulation model be used to study the effects of having high-speed and low-speed tracks where aircraft are assigned to these tracks based on their cruising speeds. The results of the high-speed low-speed track study were presented at IPACG/20.  It was shown that there could be a small overall fuel savings and flight time savings if this scenario were to be implemented.  The meeting noted that the savings would be relatively small and that most of the aircraft operating in NOPAC would be assigned to tracks with their requested speeds (reference 4).

  - Regarding ADS and non-ADS equipped aircraft, it was decided at IPACG/20 that separate tracks should not be considered further and that instead the NPACE model should be modified and used to determine at what point implementing ADS technology will be necessary for all airlines (reference 4).  An advisory group was formed to provide technical and operational guidance for modifying the existing models developed by the NPACE Study.

  - The first NPACE Study Advisory Group (NSAG) meeting was held in Livermore, California on 13 January 2004 following an Oceanic Work Group (OWG) meeting.  Three specific simulation scenarios were established for the NPACE study. Additionally, information was provided by the group regarding air traffic control (ATC) procedures to be incorporated into the revised flight tracking program.

  - The second NSAG meeting was held in Livermore, California on 21 July 2004 following an OWG meeting. The modifications made to the model were presented and the preliminary results were discussed.

  - The third NSAG meeting was held in Livermore, California on 13 October 2004 following an OWG meeting. The final model modifications were presented and further results were discussed, including the supplementary results for the addition of the required navigation performance (RNP)-4 scenario.

  - Two conference calls were held on 10 January 2005 and 13 January 2005 where members of NSAG provided further input and discussed the results of the ADS scenarios.

  - The simulation scenarios and investigation procedure are outlined in section two of this paper, the results are presented in section three, and the summary and conclusions are discussed in section four.

2.	ADS Investigation

2.1.	The following two scenarios were established to investigate the North Pacific airspace system performance under future traffic densities and separation standards.

RNP-10 Scenario: Investigate the system performance under the forecasted traffic for the years 2005, 2010, 2015, 2020, and 2025, shown in Table 1, under the separation standards capable of being used for RNP-10, listed in Table 2, for the increasing ADS aircraft percentages 0%, 35%, 65%, and 100%

RNP-4 Scenario: Investigate the system performance under the forecasted traffic for the years 2005, 2010, 2015, 2020, and 2025, shown in Table 1, under the separation standards capable of being used for RNP-4, listed in Table 2, for the increasing ADS aircraft percentages 0%, 35%, 65%, and 100%

Table 1. Traffic forecast based on APANPIRG Average Traffic Forecast (August 2003)

| Year | Approximate Number of Flights in North Pacific per Day |
| --- | --- |
| 2002 | 400 |
| 2005 | 415 |
| 2010 | 530 |
| 2015 | 675 |
| 2020 | 850 |
| 2025 | 1000 |

2.2.	Flight events are first generated for a month of flights in the North Pacific airspace, utilizing the flight events program (reference 5). The simulated flight events utilize the historical data for the Pacific oceanic airspace for the year 2002, provided by the Anchorage and Oakland Air Route Traffic Control Centers (ARTCCs), referred to as Enhanced Traffic Management System (ETMS) data, and the Japan Civil Aviation Bureau (JCAB). The flight events program produces a specified number of flights closely imitating actual flights occurring during the same interval of time, and it can be adjusted to reflect a decrease or a growth in flight traffic for particular regions or origin-destination pairs based on traffic forecasting, as well as to account for the introduction of new aircraft types and withdrawal of older types.

Table 2. Longitudinal Separation Standards for RNP-10 and RNP-4 Scenarios

(Lateral separation of 50 nm is accounted for by the placement of the tracks)

| In-trail Aircraft | Minimum Aircraft Separation RNP-10 | Minimum Aircraft Separation RNP-4 |
| --- | --- | --- |
| ADS/ADS | 50 nm longitudinal | 30 nm longitudinal |
| ADS/Non-ADS | 10 min longitudinal with MNT | 10 min longitudinal with MNT |
| Non-ADS/Non-ADS | 10 min longitudinal with MNT | 10 min longitudinal with MNT |

2.3.	The first step of flight planning is to determine which track, out of a maximum of five possible for each flight, will be used in the subsequent processing. To accomplish this, each flight is run through the flight planning program for each possible track with the altitudes set at a constant 35,000 feet for the entire flight on all tracks to ensure an unbiased comparison. The track affording the least fuel consumption is chosen as the final track for that flight.

2.4.	The second step is the actual optimal flight planning in which the optimal altitude combination is found for the optimal track, where optimal denotes least fuel consumption. An altitude restriction subroutine in the flight planning program ensures that the maximum altitudes for the different aircraft types are not exceeded at oceanic entry or in-flight.

2.5.	The first step of flight tracking deals with the conflicts at oceanic entry. The flights are first arranged in order of entry time, and then the conflicts are detected based on the defined separation standards for each respective scenario and the Mach Number Technique (MNT).  Conflicts are resolved by adjusting the entry flight level, delaying the flight, or a combination of these two adjustments.

2.6.	The second step of flight tracking deals with the in-flight conflicts between in-trail flights. All occurrences of multiple flights arriving at the same waypoint, whether in-trail on the same track or crossing on different tracks, are considered in time order. Separation standards are established and calculated for in-trail flights using the specific scenario separation standards and the MNT, and a required separation time of 10 minutes is used for flights on different tracks crossing at the same waypoint. When a conflict is found, various combinations of flight level changes are considered to resolve the conflict while attempting to preserve the optimal flight plans of the flights involved as much as possible.

2.7.	After all conflicts have been detected and resolved using these methods, an algorithm similar to the second step of flight planning is run to evaluate the final flight plans and calculate the resulting fuel consumption and flight time.  The difference in these recalculated values provides an indication of the effect of the changing separation standards for increasing ADS-capable aircraft densities. In addition, statistics are generated, indicating the percent of flights altered by entry flight level changes and/or flight delays.

- Results

  - Table 3 contains the results for the 2005 scenario, Table 4 contains the results for the 2010 scenario, Table 5 contains the results for the 2015 scenario, Table 6 contains the results for the 2020 scenario, and Table 7 contains the results for the 2025 scenario.

  - Figures 1 and 2 compare the percentage of flights delayed over all ADS percentages for the five scenarios under RNP-10 and RNP-4, respectively; Figures 3 and 4 compare the average delay time over all ADS percentages for the five scenarios under RNP-10 and RNP-4, respectively; Figures 5 and 6 compare the percentage of flights with entry level changes over all ADS percentages for the five scenarios under RNP-10 and RNP-4, respectively; Figures 7 and 8 compare the percentages of flights with in-flight level changes over all ADS percentages for the five scenarios under RNP-10 and RNP-4, respectively; and Figures 9 and 10 compare the average increase in fuel over the optimally projected fuel consumption, over all ADS percentages for the five scenarios under RNP-10 and RNP-4, respectively.

Table 3.	System performance results for 2005 (415 flights per day)

|  | 0% |  | 35% |  | 65% |  | 100% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 |
| % Delayed | 16.5% | 16.5% | 15.6% | 15.2% | 15.1% | 13.1% | 12.9% | 10.2% |
| Average delay (min) | 8.4 | 8.4 | 8.0 | 7.9 | 7.1 | 6.1 | 5.5 | 4.0 |
| % With entry level change | 13.5% | 13.5% | 13.1% | 12.8% | 12.0% | 10.8% | 9.9% | 5.5% |
| % With in-flight level change | 45.6% | 45.6% | 45.1% | 44.1% | 43.0% | 39.8% | 38.6% | 29.0% |
| Average fuel increase over optimal (gallons) | 162.3 | 162.3 | 158.8 | 155.0 | 153.5 | 145.3 | 143.5 | 121.9 |

Table 4.	System performance results for 2010 (530 flights per day)

|  | 0% |  | 35% |  | 65% |  | 100% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 |
| % Delayed | 21.5% | 21.5% | 21.1% | 19.8% | 19.7% | 17.6% | 16.6% | 12.2% |
| Average delay (min) | 10.5 | 10.5 | 9.8 | 9.4 | 8.8 | 7.7 | 6.3 | 4.4 |
| % With entry level change | 16.0% | 16.0% | 15.2% | 15.0% | 14.5% | 12.9% | 11.1% | 6.8% |
| % With in-flight level change | 50.6% | 50.6% | 49.8% | 48.6% | 47.8% | 44.7% | 42.1% | 33.5% |
| Average fuel increase over optimal (gallons) | 176.8 | 176.8 | 173.4 | 171.4 | 168.5 | 159.1 | 148.0 | 126.9 |

Table 5.	System performance results for 2015 (675 flights per day)

|  | 0% |  | 35% |  | 65% |  | 100% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 |
| % Delayed | 28.4% | 28.4% | 28.0% | 26.9% | 25.0% | 22.2% | 21.4% | 15.8% |
| Average delay (min) | 14.8 | 14.8 | 13.8 | 13.6 | 11.9 | 10.2 | 8.7 | 5.1 |
| % With entry level change | 18.9% | 18.9% | 18.3% | 18.4% | 17.1% | 15.1% | 14.4% | 9.1% |
| % With in-flight level change | 56.9% | 56.9% | 55.7% | 55.3% | 53.7% | 50.3% | 49.3% | 39.7% |
| Average fuel increase over optimal (gallons) | 206.3 | 206.3 | 192.8 | 192.9 | 182.2 | 169.2 | 163.4 | 134.9 |

Table 6.	System performance results for 2020 (850 flights per day)

|  | 0% |  | 35% |  | 65% |  | 100% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 |
| % Delayed | 37.2% | 37.2% | 36.2% | 34.8% | 33.6% | 29.6% | 28.8% | 19.8% |
| Average delay (min) | 20.5 | 20.5 | 19.2 | 18.7 | 17.7 | 15.0 | 12.3 | 6.4 |
| % With entry level change | 21.0% | 21.0% | 20.4% | 19.7% | 19.4% | 17.5% | 16.6% | 10.4% |
| % With in-flight level change | 60.6% | 60.6% | 60.0% | 58.9% | 58.0% | 54.2% | 54.2% | 43.7% |
| Average fuel increase over optimal (gallons) | 219.6 | 219.6 | 209.5 | 204.9 | 202.6 | 181.4 | 176.6 | 140.1 |

Table 7.	System performance results for 2025 (1000 flights per day)

|  | 0% |  | 35% |  | 65% |  | 100% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 | RNP 10 | RNP 4 |
| % Delayed | 42.1% | 42.1% | 41.6% | 40.5% | 38.6% | 34.2% | 33.5% | 23.9% |
| Average delay (min) | 22.6 | 22.6 | 21.3 | 21.3 | 19.3 | 17.6 | 16.4 | 7.7 |
| % With entry level change | 22.4% | 22.4% | 22.1% | 21.6% | 20.8% | 19.4% | 18.0% | 12.7% |
| % With in-flight level change | 63.8% | 63.8% | 63.1% | 62.1% | 61.5% | 58.1% | 57.7% | 47.8% |
| Average fuel increase over optimal (gallons) | 239.8 | 239.8 | 242.7 | 227.7 | 226.6 | 203.5 | 195.5 | 148.5 |

Figure 1.	Percent of flights delayed for RNP-10

Figure 2.	Percent of flights delayed for RNP-4

Figure 3.	Average delay time in minutes for RNP-10

Figure 4.	Average delay time in minutes for RNP-4

Figure 5.	Percent of flights with entry level changes for RNP-10

Figure 6.	Percent of flights with entry level changes for RNP-4

Figure 7.	Percent of flights deviating from optimal for RNP-10

Figure 8.	Percent of flights deviating from optimal for RNP-4

Figure 9.	Average fuel increase from optimal for RNP-10

Figure 10.	Average fuel increase from optimal for RNP-4

- Summary and Conclusions

  - The information contained in this paper provides an update of the status of the NPACE study.  An advisory group was formed to provide technical and operational guidance to the NPACE Study team.  The first NSAG meeting was held in January 2004.  At this meeting, the simulation scenarios were established for the NPACE study. Additional information was provided by the NSAG regarding ATC procedures to be incorporated into the NPACE simulation model.

  - The simulation scenarios involve forecasted airspace traffic volume for the years 2005, 2010, 2015, 2020, and 2025, and the separation standards available with both the RNP-10 and RNP-4 classifications for various percentages of ADS equipped aircraft.

  - This paper provided a summary of the results for the originally established scenarios as well as those for additional years of forecasted traffic volumes and the inclusion of the RNP-4 scenario.

4.4.	The conclusions of the NPACE investigation regarding the benefits of ADS based on the ADS scenario analyses are:

- The percentage of flights changed from their optimal entry time and level decreases as the percentage of ADS-designated flights increases.

- The average delay time of those flights that are delayed decreases as the percentage of ADS-designated flights increases.

- The percentage of flights changed from their optimal in-flight profile to resolve oceanic conflicts decreases as the percentage of ADS-designated flights increases.

- The average increase in fuel consumption over the projected optimal of those flights that deviate from the optimal profile decreases as the percentage of ADS-designated flights increases.

- The higher the traffic level, the greater the number of conflicts and consequently more entry level changes, in-flight level changes, and entry time changes are required by the system, and the greater the magnitude of the decrease in necessary changes as the percentage of ADS flights increases.

- There is a noticeable improvement in system performance when RNP-4 separation standards are used; as the percentage of ADS flights increases and the traffic level increases, the impact of RNP-4 separation standards compared with RNP-10 becomes more profound.

- The NPACE model assumes that the separation standards are uniform across the oceanic airspace and does not consider the possibility of the Oakland Flight Information Region (FIR) implementing the RNP-10 or RNP-4 separation standards while the Tokyo FIR maintains the current separation standards.

- The NPACE model captures only the random possibility that the ADS-equipped flights for the 35% and 65% scenarios will travel through the oceanic airspace in-trail; these in-trail conditions would further the benefits of the reduced separation standards experienced by the ADS-equipped flights as well as the system. (It was noted that this is a considerable possibility on certain tracks in the PACOTS.)

5.	Recommendation

5.1.	The meeting is invited to note the information presented in this paper.

# References

- Summary of Discussions for the Seventeenth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/17), 15-19 April, 2002; San Francisco, CA, USA.

- Summary of Discussions for the Eighteenth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/18), 8-11 October 2002; Tokyo, Japan.

- Summary of Discussions for the Nineteenth Meeting of the Informal Pacific ATC Coordinating Group (IPACG/19), 15-17 July 2003; Tokyo, Japan.

- Summary of Discussions for the Twentieth Meeting of the Informal Pacific Air Traffic Control Coordination Group (IPACG/20), 6-10 October 2003; Honolulu, Hawaii, USA.

- Flight Events Generation for Air Traffic Over the Northern Pacific Ocean, Final Report, Rutgers University and The Federal Aviation Administration Technical Center, 30 June 2003.

- Flight Events Generation for Selected 16 OD Pairs Over the Northern Pacific Ocean, Final Report, Rutgers University and The Federal Aviation Administration Technical Center, 30 June 2003.