#### THE TWENTY-SECOND MEETING OF THE

#### INFORMAL PACIFIC ATC COORDINATION GROUP

(IPACG/22)

Los Angeles, USA, 24-28 January 2005

Agenda Item 2: Air Traffic Management (ATM) Issues

Implementation of Strategic Lateral Offsets Procedures in Tokyo FIR and Naha FIR

(Presented by Japan Civil Aviation Bureau)

SUMMARY This paper presents the information concerning the different points of procedures of strategic lateral offsets in Tokyo FIR and Naha FIR from FAA procedures. Ｊ

- Introduction

  - ICAO distributed a State Letter, AN 13/11.6-04/85, on 27 August 2004, regarding revised guidelines on the use of strategic lateral offsets as a safety measures to reduce the risk of collision in the event of loss of vertical separation. The ICAO revised guidelines on the use of strategic lateral offsets to be applied by pilots are as follows:

a)  offsets shall only be applied in airspace where this has been approved by the appropriate ATS authority;

b)  offsets shall be applied only by aircraft with automatic offset tracking capability;

c)  the decision to apply a strategic lateral offsets is the responsibility of the flight crew;

d)  the offsets shall be established at a distance of one or two nautical miles to the right of the center line relative to the direction of flight;

e)  the strategic lateral offset procedure has been designated to include offsets to mitigate the risk of wake turbulence of preceding aircraft. If wake turbulence needs to be avoided, one of the three options (centerline, 1NM or 2NM right offset) shall be used;

f)  in airplane where the use of lateral offsets has been authorized, pilots are not required to inform air traffic control (ATC) that an offset is being applied; and

g)  aircraft transiting areas of radar coverage in airspace where offsets tracking is permitted may initiate or continue an offset.

  - The Fifteenth meeting of the Asia/Pacific Air Navigation Planning and Implementation Regional Group (APANPIRG/15, 23-27 August 2004) urged all States who were responsible for the provision of air traffic services in such airspace to authorize the use of strategic lateral offsets in accordance with the guidelines at the earliest opportunity. To ensure simultaneous implementation over contiguous airspaces, APANPIRG/15 agreed that the Asia/Pacific should

adopt a coordinated approach to implementing the offset procedures and formulated the following conclusion.

Conclusion 15/8   -   Implementation of 2 NM lateral offset procedures in the Asia/Pacific region

That, States in the Asia/Pacific region implement the 2 NM lateral offset procedures to the right of center line in accordance with ICAO guidance on a common AIRAC date to be coordinated by the ICAO Regional Office with States, ATS Coordination Groups and users concerned.

1.3          Accordingly, States are requested to make suitable arrangements to implement the 2 NM lateral offset to the right of route center line procedures on AIRAC date 20 January 2005.

2.　        Strategic Lateral Offsets Procedures in Tokyo FIR and Naha FIR

2.1          A NOTAM on implementing the 2 NM lateral offsets procedures to the right of the center line in Tokyo FIR and Naha FIR had been issued on 13 January 2005, reproduced as Appendix A, and AIP amendment will be issued on AIRAC date 17 February 2005.

2.2          A typical different point of procedures of strategic lateral offsets in Tokyo FIR and Naha FIR from FAA procedures 3. (f) and (g), reproduced as Appendix B, is 4 f):

f)   When radar identification is established, pilots shall fly on the centerline.

Note) After radar identification established, if wake turbulence needs to be avoided the offset procedure shall be approved as deviation with ATC clearance.

2.3          The reasons are as follows:

a)   JCAB has no oceanic entry/exit point defined clearly;

b)   JCAB has no transiting radar-controlled airspace;

c)   ATC shall provide safety separation between under radar controlled aircraft; and

- Civil Aeronautical Regulations of Japan, article 207, states:

“Any aircraft flying under instrument flight rules shall, while flying along an airway within a control area or control zone, fly along the center of the airway, except in unavoidable cases.”

3.             Recommendation

3.1           The meeting is invited to note the information provided in this paper.

Attachments:

A   -   JCAB Strategic Lateral Offsets Procedures in Tokyo FIR and Naha FIR

B   -   FAA Strategic Lateral Offsets in the Oceanic Airspace to Mitigate Wake Turbulence and to Mitigate Collision Risk

Attachment A

Strategic Lateral Offsets Procedures in Tokyo FIR and Naha FIR

1.  	This procedure is to be used for both wake vortex encounters, and to mitigate the heightened risk of collision when non-normal events such as operational altitude deviation errors and turbulence induced altitude deviations occur.

2.  	Offsets are only applied in the oceanic control area over the Pacific Ocean within the Tokyo and the Naha FIRs.

3.  	Offsets are applied only by aircraft with automatic offset tracking capability.

4.  	The following requirements apply to the use of the offset:

a)    The decision to apply a strategic lateral offset is the responsibility of the flight crew.

- The offset shall be established at a distance of one or two nautical miles to the right of the center line relative to the direction of flight.

- The strategic lateral offset procedure has been designed to include offsets to mitigate the effects of wake turbulence of preceding aircraft. If wake turbulence needs to be avoided, one of the three available options (centerline, 1NM or 2NM right offset) shall be used.

- There is no ATC clearance required for this procedure and, above it is not necessary that ATC be advised.

- Voice position reports are to be based on the current ATC route/course clearance and not the exact co-ordinates of the offset position.

- When radar identification is established, pilots shall fly on the centerline.

Effective: January 20, 2005, 0901UTC

Note) AIP ENR3.6 Additional articles for ATS routes 10.Implementation of RVSM in Tokyo FIR and Naha FIR 10.7Special Procedures to mitigate Wake turbulence encounters and distracting aircraft system alerts is withdrawn on January 20, 2005, 0901UTC.

Attachment B

# Strategic Lateral Offsets In Oceanic Airspace To Mitigate Wake Turbulence And To

# Mitigate Collision Risk

1. 	Pilots should use the Strategic Lateral Offset Procedure as standard operating practice in the course of normal oceanic operations to mitigate collision risk and wake turbulence.  The Strategic Lateral Offset Procedure will be applied throughout the Oakland and Anchorage oceanic FIRs. This procedure is to be used for both wake vortex encounters, and to mitigate the heightened risk of collision when non-normal events such as operational altitude deviation errors and turbulence induced altitude deviations occur.

2.  	Strategic Lateral Offset Procedures will be applied using the following guidelines:

(a) Strategic lateral offsets executed to mitigate collision risk and those executed to mitigate the effects of wake turbulence are to be made to the right of a route or track;

(b) In relation to a route or track, there are three positions that an aircraft may fly: centerline, 1 NM or 2 NM right; and,

(c) Offsets are not to exceed 2 NM right of centerline.

3. 	The intent of this procedure is to reduce risk (increase the safety margin) by distributing aircraft laterally and equally across the three available positions. In this connection, pilots must take account of the following:

(a) Aircraft without automatic offset programming capability must fly the centerline;

(b) Aircraft capable of being programmed with automatic offsets may fly the centerline or offset 1 NM or 2 NM right of centerline to obtain lateral spacing from nearby   aircraft;

(c) Pilots should use whatever means are available (e.g. communications, visual acquisition, GPWS or TCAS/ACAS) to determine the best flight path to fly;

(d) Any aircraft overtaking another aircraft is to offset within the confines of this procedure, if capable, so as to create the least amount of wake turbulence for the aircraft being overtaken;

(e) For wake turbulence purposes, pilots are also to fly one of the three positions at 2b above and never offset to the left of centerline nor offset more than 2 NM right of centerline;

NOTE.   It is recognized that the pilot will use his/her judgment to determine the action most appropriate to any given situation and has the final authority and responsibility for the safe operation of the aeroplane. The use of air-to-air channel, 123.45, may be used to co-ordinate the best wake turbulence offset option.

(f) Pilots may apply an offset outbound at the oceanic entry point but must return to centerline at the oceanic exit point.

(g) Aircraft transiting radar-controlled airspace (e.g. Guam or Vancouver Center) may    remain on their established offset positions but must advise the radar controller on initial contact of their offset status;

(h) There is no ATC clearance required for this procedure and, except as stated in paragraph (g), above it is not necessary that ATC be advised; and,

(i) Voice position reports are to be based on the current ATC route/course clearance and not the exact co-ordinates of the offset position.