THE TWENTY-SECOND MEETING OF

THE INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/22)

Los Angeles, U.S.A., 24-28 January 2005

Agenda Item 2: Air Traffic Management (ATM) Issues

# The establishment of Air Traffic Management Center (ATMC)

# and

# the related changes on

# oceanic ATC, data link operations and addressing of flight plan message

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This paper presents the information about the timeline on the merger of JCAB oceanic ATC functions to the planned Air Traffic Management Center in Fukuoka, the expansion of data link serviced airspace and the change of addressing of flight plan message to the Japanese FIRs.

Action by the meeting is in paragraph 5.

- INTRODUCTION

  - JCAB introduced Air Traffic Management Center (ATMC) plan with the IP/7 titled “The establishment of Air Traffic Management Center in Japan” at the IPACG/20 in Honolulu, Hawaii, 2003. The ATMC is the development of the existing Air Traffic Flow Management Center (ATFMC) in Fukuoka by adding Air Space Management (ASM) function and Oceanic ATC function (ATS).

  - As stated in the IP/7 at IPACG/20 in Honolulu, the oceanic ATC functions, which are currently provided by Tokyo ACC and Naha ACC, will be merged to the ATMC step by step. The merger will begin on October 27, 2005 and be over on February 16, 2006.

  - On completion of the merger of the oceanic ATC functions to the ATMC, the Tokyo and Naha FIRs will be consolidated to a single FIR, of which name will be Fukuoka FIR.

  - Data link services, ADS/CPLDC, will be available in all the oceanic airspace of the unified FIR, which improves the aircraft operations in the oceanic airspace.

- MERGER OF OCENIC ATC FUNCTIONS TO THE ATMC

Sapporo ACC

Tokyo ACC

Fukuoka ACC

Naha ACC

ATMC will be located in Fukuoka and responsible for ATC service in all the oceanic controlled airspace of the Fukuoka FIR.

North A/B (NOPAC) 2005/10/27

East 2005/12/8

South B  (Naha Ocean) 2006/2/16

South A 2005/12/8

Figure 1.   ATC facilities and their responsible areas

| New oceanic Sectors at ATMC | 2005 |  |  |  |  |  |  |  |  | 2006 |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 1 | 2 | 3 |  |
| North A/B |  |  |  |  |  |  | 27th |  |  |  |  |  | From Tokyo ACC |
| East |  |  |  |  |  |  |  |  | 8th |  |  |  |  |
| South A |  |  |  |  |  |  |  |  | 8th |  |  |  |  |
| South B |  |  |  |  |  |  |  |  |  |  | 16th |  | From Naha ACC |
| Inauguration of ATMC |  |  |  |  |  |  | 1st |  |  |  |  |  |  |
| Single FIR |  |  |  |  |  |  |  |  |  |  | 16th (TBD) |  | Fukuoka FIR |

Table 1.   Timeline on the merger of oceanic ATC functions to the ATMC

  - October 27, 2005

The oceanic sectors North A and North B of Tokyo ACC, which are responsible for NOPAC routes, are moved to the ATMC.

  - December 8, 2005

The oceanic sector East and South of Tokyo ACC, the sector East responsible for PACOTS routes and the sector South responsible for the routes to Guam/Saipan and Oceania region, are moved to the ATMC. The South sector will be renamed as South A after the move to the ATMC. With the completion of the transfer of oceanic sectors North A/B, East and South (South A in the ATMC), there will be no sectors responsible for oceanic ATC service in Tokyo ACC.

  - February 16, 2006

    - The ATMC will be responsible for the oceanic ATC service in the Naha FIR. This concludes the merger of all the oceanic ATC functions to the ATMC.

    - FIR unification … With the completion of the merger of oceanic ATC functions to the ATMC, the Tokyo and Naha FIRs will be unified and renamed as Fukuoka FIR. The unification of the two FIRs improves the efficiency of aircraft operations in the oceanic airspace.

- ADS/CPDLC DATA LINK SERVICES

  - Tokyo ACC has been providing data link services, ADS/CPDLC, since 1997.

  - Upon the merger of the oceanic ATC functions in Naha oceanic airspace to the ATMC, data link services, ADS/CPDLC, will be available in all the oceanic airspace of the unified FIR.

  - Currently the AFN log-on address to the data link system of the Tokyo FIR is “RJTG”. After the completion of the merger of the oceanic ATC functions to the ATMC, which is scheduled on February 16, 2006, the AFN log-on address to the data link system of the unified FIR will be changed to “RJJJ”, the location indicator of the ATMC.

  - As for HF communication, the Tokyo radio and the Naha radio have been providing their HF communication services to Tokyo FIR and Naha FIR respectively. The Naha radio continues their service until they are merged to the Tokyo radio, which is expected in 2007.

- ADDRESSING OF FLIGHT PLAN MESSAGE

  - On February 16, 2006, the flight data processing (FDP) system of Japan, which is currently at Tokyo ACC (RJTG), will be replaced by the new one in the ATMC (RJJJ).

  - The replacement of the FDP system by the new one in the ATMC will change the addressing of flight plan message to Japan from “RJTGZQZX” to “RJJJZQZX”, effective as of February 16, 2006.

ATM Center

in Fukuoka

(RJJJ)

Tokyo ACC

(RJTG)

- ACTION BY THE MEETING

  - The meeting is invited to note the information in this paper.