THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

Los Angeles, USA, 24-28 January 2005

Agenda Item 2:  Air Traffic Management (ATM) issues

Implementation of ADS 50NM longitudinal separation minimum in the Tokyo FIR

(Presented by Tokyo Area Control Center)

SUMMARY

This paper describes the introduction of ADS 50NM longitudinal separation minimum during level changes (step climb/descent) on R220/R580 in the Tokyo Oceanic Control Airspace.

１.	Background

1.1	At the IPACG/21 meeting, held in Tokyo in June 2004, JCAB presented a paper on the implementation plan for ADS 50NM longitudinal separation minimum in the Tokyo Oceanic Control Airspace. At the meeting, JCAB reported that in early 2005 ADS 50NM longitudinal separation minimum during level changes (step climb/descent) would be applied prior to 50NM longitudinal separation at cruise.

２.	Discussion

2.1	According to the implementation plan of JCAB, Tokyo Area Control Center (ACC) made the procedures to implement ADS 50NM separation in the Tokyo Oceanic Control Airspace in consideration of the current of airspace/airways usage, occupation of ADS capable aircraft, training of the controllers and so on.

2.2	Procedures of the limited operation are below;

・Applicable ATS route : R220 (incl.R217)/R580 (except converging to or crossing the airways)

・Applicable aircraft : ADS capable aircraft

・Implementation date : the middle of April, 2005

2.3	Tokyo ACC decided to implement the ADS 50NM separation initially in the limited airways such as R220/R580 because the airways R220/R580, main routes from the US to Asian countries are congested with westbound flights. The population of ADS capable aircraft on these airways is approximately 50% or more. That’s why the benefit of introduction would be expected on the airways.

2.4	After the evaluation on R220/R580, JCAB will consider the implementation plan to expand applicable airspace/airways.

2.5	ODP3 was upgraded to add the function of measuring on 18 January 2005 and will be modified in March 2005.

３.	Recommendation

3.1 	The meeting is invited to note the information provided in this paper.