THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 2:	Air Traffic Management (ATM) Issues

Enhanced ATS Inter-facility Data Communications (AIDC) Functionality Between Tokyo ACC, Anchorage ARTCC, and Oakland ARTCC

(Presented by the Federal Aviation Administration)

SUMMARY

This paper proposes the adoption of Version 2 of the Asia/Pacific AIDC Interface Control Document (ICD) as the basis of inter-facility data communications between Tokyo ACC, Anchorage ARTCC, and Oakland ARTCC. The paper further proposes a phased implementation of the provisions and guidance material contained within the ICD.

1.		Background

  - ATS Inter-facility Data Communications (AIDC) provides an automated means of flight data exchange between Area Control Centers (ACC). The AIDC service between Oakland Air Route Traffic Control Center (ARTCC) and Tokyo ACC was initiated in 1998 based on Version 1 of the Asia/Pacific AIDC Interface Control Document. During the development of this service both the JCAB and the FAA encountered technical problems in implementing the full ICD so both agencies worked together to make changes to support each other. As a result of this cooperation the first automated data interface between foreign FIRs was established and it has run successfully for over seven years.

  - Subsequent to this, several other air traffic service providers within the region established and operated AIDC interfaces with surrounding flight information regions (FIRs).  Decision 13/9 of the 13th Meeting of the Asia Pacific Air Navigation Planning and Implementation Regional Group (APANPIRG/13) reactivated the AIDC Task Force and charged the group with reviewing and updating Version 1 of the ICD.  As a result of these activities Version 2 of the document was published 28 March 2003.

  - One major change introduced in version 2 of the Asia/Pacific AIDC ICD that is pertinent to this paper was the incorporation of recent proposed changes to PANS-ATM (Doc 4444) and Doc 9694, regarding additional optional sub-fields in ICAO Field 14.  Version 2 of the ICD also defines three additional message types that are of interest to this paper, namely the Application Status Monitor (ASM), the FANS Application Notification (FAN) and the FANS Completion Notification (FCN).

- Discussion

  - The FAA proposes the adoption of Version 2 of the Asia/Pacific AIDC Interface Control Document as the basis of inter-facility data communications between Tokyo ACC, Anchorage ARTCC, and Oakland ARTCC. FAA further proposes a three stage implementation of the provisions and guidance material contained within the ICD.  The stages will be defined by technical complexity.  This approach will help minimize risk to schedule and cost.  Another advantage to the staged approach is that it will provide the changes needed for the elimination of routine verbal coordination at an earlier date.

    - Stage 1 contains two main tasks.  The first is the implementation of core message use and sequencing in accordance with Appendix D of the ICD, Implementation Guidance Material (IGM).  The other is to employ the field 14 sub-field enhancements.

    - Stage 2 will improve end-to-end communication reliability and reduce user communication costs.  This will be accomplished by the use of the three newly defined messages; ASM, FAN, and FCN.

    - Stage 3 will enhance safety by providing greater visibility of cross- and near-boundary flight operations and, further reduction of user communication costs.  Accomplishment of this stage will require the development of a definition for the text field of the Surveillance ADS (ADS) message.

  - Stage 1, Guidance material implementation.  The IGM describes three phases of flight to help define appropriate use and sequence of AIDC messages.  These three phases are notification, coordination, and transfer of control.

    - During the notification phase the adjacent FIR receives initial information on a flight that will enter its airspace or be operating in an Area of Common Interest (ACI).  The information is provided beginning some period of time prior to the aircraft reaching the boundary, typically 60 or 80 minutes.  This advisory notification is accomplished using the Advance Boundary Information (ABI) message which provides boundary, level and route information on the flight. Any changes to the aircraft’s flight profile before the initial coordination message are conveyed by transmitting another ABI.

    - The next phase, coordination, can be subdivided to initial coordination and renegotiation.  Initial coordination marks the transition of the flight from the notification to the coordination phase.  The initial coordination is a one time event used to establish a “contract” between the facilities regarding the flight profile parameters.  This event is initiated when the controlling FIR transmits a Current Flight Plan (CPL) message to the receiving FIR to specify the proposed boundary crossing conditions.  If the specified conditions are acceptable to the receiving FIR it will transmit an Acceptance (ACP) message.  If the receiving FIR requires some modification to proposed conditions contained in the CPL it will reply to the controlling FIR with a Coordination (CDN) message to propose a different set of boundary crossing conditions.  If these conditions are acceptable to the controlling FIR it will transmit an ACP message.  If not it will transmit a CDN.  The initial coordination is completed when one of the FIRs transmits an ACP message.  It is important to note that the use of the Rejection (REJ) message is not permitted during initial coordination.

    - The completion of the initial coordination marks the beginning of the renegotiation portion of the coordination phase.  Renegotiation will not be required for most flights but if either FIR deems it necessary to modify the “contracted” conditions (e.g., boundary crossing time estimate changes by 4 minutes) it can initiate a renegotiation by transmitting a CDN message.  If the other FIR finds these conditions to be acceptable it transmits an ACP message otherwise it transmits a CDN message to propose an alternate set of conditions or a REJ message.  A renegotiation event is terminated when one of the FIRs transmits either an ACP or a REJ message.  If a renegotiation event is terminated with a REJ message then the previously “contracted” conditions remain in effect.

    - If at any point during the notification or coordination phases the flight profile parameters change such that the flight is no longer relevant to the receiving FIR the controlling FIR transmits a Coordination Cancellation (MAC) message.

    - The transfer of control phase begins when the controlling FIR transmits a Transfer of Control (TOC) message to offer executive control of a flight to the receiving FIR.  The phase (and entire process) is completed when the receiving FIR transmits an Assumption of Control (AOC) message to indicate acceptance of executive control of the flight.

  - Stage 1, ICAO field 14 sub-field enhancements implementation.  One of the major changes made to the ICD in Version 2 was making provisions for the use of additional subfields to ICAO field 14, estimate data. These subfields are used to convey information regarding block altitudes, boundary crossing fix altitude restrictions, weather deviations, route offsets, and Mach number assignment. The exchange of this information via the automated data interface improves air traffic management (earlier detailed insight to inbound flight profiles) and enhances safety (eliminates verbal misunderstandings and reduces incomplete coordination errors).

    - AIDC ICD Version 1 functionality only provides for the inclusion of a single flight level in coordination messages.  By Letter of Agreement the upper level of a block altitude clearance is the altitude that is transmitted in field 14c, cleared level.  The full information regarding the block levels is then passed by voice coordination.  Under Version 2 of the ICD all information regarding the block level can be relayed via data communications, further reducing the need for voice coordination.

    - While Version 1 of the ICD did not specifically prohibit the use of fields 14d and 14e the AIDC implementations within the region tended to not include them in the processing flow.  Version 2 specifically allows for the inclusion of boundary crossing fix altitude restrictions in any message that contains field 14 (e.g., ABI, CPL) to convey a restricted climbing or descending situation.  The crossing restriction is limited to a single level but can be used in conjunction with block altitude data to indicate that an aircraft will be climbing or descending to reach a cleared block level when it crosses the boundary.

    - The boundary estimate data may contain additional clearance information to described route offsets and weather deviations.  When one of the conditions will exist when the aircraft crosses the boundary field 14 will contain a subfield that describes (1) the nature of the divergence, offset or weather deviation, (2) the off track distance of the divergence and (3) the direction of the divergence, left, right, or either.  When this condition is included the contents of field 14a, boundary point, shall be based on the aircraft’s nominal route rather than route divergence.  If route divergence information has been previously transmitted and the aircraft subsequently returns to its nominal route this information will be conveyed to the receiving FIR by sending the appropriate message (e.g., ABI, CDN) with the boundary estimate data containing no route divergence data.

    - When an aircraft has been issued a Mach number assignment this information is conveyed in a field 14 subfield.  The subfield will indicate whether the aircraft will be maintaining (1) the notified Mach number or less, (2) the notified Mach number or greater, (3) or exactly the notified Mach number; and the notified Mach number.

  - Stage 2 will put into service three optional messages that were added to the AIDC message set in Version 2 of the ICD.  The overall goal of this stage is to improve end-to-end communication reliability and reduce user communication costs.  The three messages to be deployed during this stage are the Application Status Message (ASM), the FANS Application Message (FAN), and the FANS Completion message (FCN).

    - AIDC messaging between Oakland ARTCC and Tokyo ACC has been in operation since 1998.  While the actual coordination is presently accomplished via voice, the automated exchange of the boundary crossing has been incorporated into the operating procedures to such a degree that the service is now virtually essential.  After Stage 1 of this proposal has been implemented and the requirement for verbally coordinating every flight has been eliminated the ability to detect outages of the interface in a timely manner will be vital.

      - The ASM message is a simple message consisting only of ICAO field 3a, message type designator.  It is sent to an adjacent center to confirm that the adjacent center’s AIDC application is online and reachable. It is transmitted when no other application messages have been received from that center within an adaptable period of time.  The failure to receive a Logical Acknowledgement Message (LAM) in response to the ASM can alert technical and operational personnel to the loss of the interface.

      - Message Format

| ATS Fields | Description |
| --- | --- |
| 3 | Message type |

      - Example

(ASM)

    - When aircraft using CPDLC or ADS are transiting from one FIR to another FIR their data communication connection must be transferred to the gaining ATS unit.  The addition of two messages, FAN and FCN, to the Asia/Pacific AIDC message set enables the use of ground-ground communications to facilitate the transfer of a FANS-1/A data communication connection.

      - This connection forwarding is currently accomplished using the complex ATS Facilities Notification (AFN) procedure outlined below and illustrated in Figure 1.

Figure 1– CPDLC Connection Transfer Using AFN

- The controlling ATS unit (ATS Unit #1) uplinks a Next Data Authority (NDA) message which contains the facility designator of the next ATS unit (ATS Unit #2).

- ATS Unit #1 then uplinks an AFN Contact Advisory (FN_CAD) message which contains the network address of ATS Unit #2.

- The aircraft automatically responds to ATS Unit #1 with a downlink AFN Response (FN_RESP) message indicating intent to perform the AFN logon with ATS Unit #2.

- The aircraft then downlinks an AFN Contact (FN_CON) message to ATS Unit #2 which provides aircraft addressing information, airborne ATS applications addressing information, and ATS application version numbers.

- ATS Unit #2 then uplinks a positive AFN Acknowledgement (FN_AK) message indicating that the AFN logon process has been successfully completed.

- The aircraft then downlinks an AFN Complete (FN_COMP) message to ATS Unit #1 indicating a successful AFN logon with ATS Unit #2.

- ATS Unit #2 uplinks a Connect Request (CR1) message to initiate a CPDLC connection with the aircraft.

- Upon receipt of the CR1 message the aircraft downlinks a Connect Confirm (CC1) message to ATS Unit #2 to establish the CPDLC connection (inactive).

      - The use of air-ground communications to accomplish the connection transfer has proven adequate though it does incur some overhead and timing constraints that could be eliminated through the use of ground-ground communications to accomplish the task.  The complexity and costs associated with the current AFN procedure can be reduced through the use of the FAN and FCN messages.  The connection forwarding scheme using AIDC messaging is outlined below and illustrated in figure 2.

Figure 2– CPDLC Connection Transfer Using AIDC

- The controlling ATS unit (ATS Unit #1) uplinks a NDA message which contains the facility designator of the next ATS unit (ATS Unit #2).

- ATS Unit #1 transmits a FAN message to ATS Unit #2 which provides aircraft addressing information, airborne ATS applications addressing information, and ATS application version numbers.

- ATS Unit #2 uplinks a CR1 message to initiate a CPDLC connection with the aircraft.

- Upon receipt of the CR1 message the aircraft downlinks a CC1 message to ATS Unit #2 to establish the CPDLC connection (inactive).

- ATS Unit #2 transmits a FCN message to ATS Unit #1 indicating the outcome of the connection attempt with the aircraft.

      - The FAN message is transmitted by the controlling ATS unit to provide the receiving ATS unit with the required Context Management information necessary to establish CPDLC and/or ADS connections.  If the message contains information for a CPDLC connection it shall not be transmitted until 60 seconds after sending a Next Data Authority message element to the aircraft.

      - FAN Message Format.

| ATS Fields | Description |
| --- | --- |
| 3 | Message type |
| 7 | Aircraft identification |
| 13 | Departure aerodrome |
| 16 | Destination aerodrome |
| Text | Application and address data |

      - Application and Address Data. Structured text elements are used in this message to transfer aircraft registration (REG), airborne ATS applications addressing (ADD), CPDLC/ADS application name and version numbers (APP), and ICAO 24-bit aircraft address (SRN) information.  Structured text elements must be separated from each other by a space.  The data unit(s) contained in a text element shall be separated from the element identifier by the slash ‘/’ character. Multiple data units within a text element shall be separated from each other with a space.

        - Aircraft Registration. This mandatory text element is preceded by the identifier ‘REG’.  When an aircraft registration contains a hyphen ‘-’ character the sending ATS unit shall transmit this character as an asterisk ‘*’ character (e.g., B-HXL is transmitted as B*HXL) in order to avoid interpretation of the hyphen mark as a Start-of-Field signal.  The receiving ATS unit shall translate an asterisk ‘*’ character received in an aircraft registration to a hyphen ‘-’ character.  The data for this element can be obtained from the FN_CON message header (AEEC 622 section 3.5.3) if the aircraft initially logged onto the controlling ATS unit using AFN messaging or from the FAN message received from the previous controlling ATS unit if the connection transfer to the current controlling ATS unit utilized AIDC messaging.

        - Airborne ATS Applications Addressing.  This mandatory text element is preceded by the identifier ‘ADD’.  The data unit of the text element shall consist of three alpha characters.  The data for this element can be obtained from the ACARS network header line 3 of the ground-ground message (AEEC 620 section 3.2.1) that contained the FN_CON message if the aircraft initially logged onto the controlling ATS unit using AFN messaging or from the FAN message received from the previous controlling ATS unit if the connection transfer to the current controlling ATS unit utilized AIDC messaging.

        - CPDLC/ADS Application Name and Version Numbers.  These mandatory text elements are preceded by the identifier ‘APP’.  The data unit of the text element shall consist of three alpha characters, ATC or ADS, to represent the application name immediately followed (no intervening spaces) by two numeric characters to represent the application version number.  If a transferring ATS unit wishes to transmit a FAN message to permit a downstream ATS unit to establish only ADS contracts, the ATC text element is sent with an application version number of “00”.  The data for these elements can be obtained from the FN_CON message (AEEC 622 section 3.5.4) if the aircraft initially logged onto the controlling ATS unit using AFN messaging or from the FAN message received from the previous controlling ATS unit if the connection transfer to the current controlling ATS unit utilized AIDC messaging.

        - ICAO 24-bit Aircraft Address.  This optional text element is preceded by the identifier ‘SRN’.  The data unit of the text element shall consist of the ISO-5 character translation of the 24-bit aircraft address.

        - Example

(FAN-UAL853-KSFO-RJAA-REG N171UA/ADD FML/APP ATC01/APP ADS01)

      - The FCN message is transmitted by the receiving ATS unit to the transferring ATS unit as an operational response to a FAN message. It is used to provide assurance to the transferring unit that a successful CPDLC transfer should occur.

      - FCN Message Format.

| ATS Fields | Description |
| --- | --- |
| 3 | Message type |
| 7 | Aircraft identification |
| Text | Connection status data |

      - Connection Status Data. This mandatory text element is preceded by the identifier ‘RMK’. The data unit contained in the text element shall be separated from the element identifier by the slash ‘/’ character. The data unit is set to zero if the receiving ATS unit was unable to establish a CPDLC connection with the aircraft; otherwise it is set to one.

      - Example

(FCN-UAL882-RMK/1)

(FCN-COA-RMK/0)

    - As stated earlier, the messages necessary for the use of ground-ground communications to facilitate the transfer of a FANS-1/A data communication connection first appeared in Version 2 of the ICD.  The technical details of message content, timings, and constraints were not included the implementation guidance material of the document.  As part of the work plan to implement the use of these messages the group should include tasks to submit the necessary ICD appendix D additions to the ICAO regional office for circulation and approval by the States within the region.  This will not only allow other ATS units to benefit from the work of this group but also further the goal of harmonization.

  - Stage 3 will put into service the sharing of surveillance data for cross- and near-boundary flight operations via ground-ground messaging.  Accomplishment of this stage will require the development of a definition for the text field of the Surveillance ADS (ADS) message.

    - ATS units are required to provide flight plan and flight progress information to adjacent ATS units when flights under their control are operating in close proximity to FIR boundaries.  This information is currently provided via voice and appropriate AIDC messages (e.g., CPL).  The nature of these messages is that of a snapshot of estimates of events occurring in the future.

    - The data obtained through an ADS contract with aircraft operating near their boundaries could be used by an ATS unit to supplement the flight plan and flight progress data provided to the adjacent ATS unit during the period of proximate boundary operations.  While the same information could be obtained by the adjacent ATS unit by establishing a separate ADS contract with the aircraft it would effectively double the communication charges incurred by the user since the aircraft would be down-linking similar data twice, once to each of the ATS units.

    - The additional charges could be avoided by having only one ADS contract in effect and forwarding the surveillance information via ground-ground messaging.  The ICD includes an optional surveillance data transfer message, Surveillance ADS (ADS), to be used in the transfer of ADS data over ground-ground links.  The ADS message includes a text field to provide “to be determined” ADS data.

      - This data field could consist of the bit-oriented portion of any ADS message received by the controlling ATS unit.  This data would begin with the Imbedded Message Identifier (in this case “ADS”) and end with the cyclical redundancy check (CRC).  An advantage to this method is that it would provide the most complete information.  A disadvantage to the scheme is that it is possible that the receiving ATS unit’s ADS function might not support some of the ADS groups contained in the message which could cause problems with decoding and interpreting the data.

      - Another possibility for the data field is to use the decoded data from the Basic ADS Group.  This group is included in every message sent by the aircraft and provides the aircraft’s latitude, longitude, and altitude.  An advantage to this format is that it can be assumed that all ATS units that have an AIDC function would be able utilize this data within their flight data processing function.  A disadvantage to this approach is that it excludes the aircraft intent data that is included in most ADS messages.

      - One use of the data unit that must be provided for in the design is the ability to indicate that the sharing of surveillance data for a flight is being discontinued.

    - An early part of the work plan developed to implement the sharing of surveillance data needs to be the design and definition of the surveillance data unit of the ADS message as well as implementation guidance materials.  These can then be submitted to the ICAO Regional Office as additions to appendices A and D of the ICD for circulation and approval by the States within the region.

- Recommendation

  - The meeting is invited to:

    - consider the contents of the working paper;

    - review the proposed course of action, and provide modification or enhancement;

    - adopt the proposal as an Action Item for the IPACG; and

    - consider the establishment of a task force to implement the proposal.

These sub-fields are described in the ICD as authorized exceptions to compliance with the field 14 definition contained in ICAO PANS-ATM when used for AIDC messaging within the Asia/Pacific Region.