THE TWENTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/23)

(Tokyo, Japan, 11-15 July 2005)

Agenda Item 2:  Air Traffic Management (ATM) issues

HF Regression in Pacific Oceanic Airspace

Presented by the Federal Aviation Administration

SUMMARY

This paper introduces the need to evaluate current data link capabilities to determine how existing and near term data link capabilities can reduce the reliance on HF voice for pilot/controller communications.  It proposes using work already in progress in the North Atlantic (NAT) to help determine priorities and develop a roadmap for HF regression (phasing out of parts of current HF infrastructure and replacement by data link technologies) in Pacific oceanic airspace.

- Background

1.1  	Iceland and Ireland recently signed an agreement on HF Voice Services in the NAT, which describes their joint response to the challenges raised by changes in the provision of North Atlantic (NAT) ATS voice services.  Other NAT ATS providers and airspace users are actively seeking means to reduce reliance on expensive and inefficient pilot/controller HF voice communications.

1.2	The NAT will use the following assumptions as the basis for planning for HF regression:

a.	there will be no increase in current HF voice capacity requirements in the NAT Region for ATS purposes because of the migration to data link applications, and any substantial decline in the requirements is unlikely before 2008;

b.	by 2008 data link will be approved for use on an operational trial basis, as a primary means of communications (subject to regulatory approval), with voice available for backup requirements only;

c.	by 2010, the majority of NAT Region movements will be conducted by aircraft that have a data link capability;

d.	by 2010, data link will be approved for use as a primary  means of communication, with voice requirements as described in f below;

e.	data link will be the mandated communications medium for ATS purposes in portions of the NAT Region by 2015;

f.	there will be a general requirement for voice communications in the datalink environment for emergencies and non-routine events, which could be satisfied by a number of technologies; and

g.	a decline in HF voice capacity requirements in the NAT is unlikely before 2006.

1.3	The NAT ATS providers and airspace operators recognised that a number of issues concerning HF regression would need to be resolved.  Some of the issues included exit costs, pension obligations, environmental issues and the necessary infrastructure costs needed to provide limited HF services.  In addition, the group strongly endorsed the notion of bilateral discussions such as those taking place between Iceland and Ireland.

2.0	Discussion

2.1	HF voice communications make up close to 30% of FAA’s oceanic operations costs, while providing a service that does not provide dependable availability, reliability or quality.

2.2	With all Pacific ATS providers and an increasing number of aircraft equipped/authorized to communicate via data link, it may be appropriate to:

    - describe the operational requirement for air/ground communications subject to  performance and functional criteria;

    - quantify benefits of use of data link as a replacement for HF voice;

    - determine infrastructure changes needed to support HF voice service reduction;

    - consider possible needs for communications service providers to re-equip during transition period(s);

    - identify costs to include the cost of communications service provider reductions; and

    - consider regulatory requirements.

3.0	Recommendation

3.1	The meeting is invited to

- note the initiatives taken in the NAT toward addressing the issues raised by the operator community concerning HF inefficiencies and high costs; and

- determine if there is a need to address the changing communications environment in the Pacific, and if so, prioritise this issue for future work.