THE TWENTY-SECOND MEETING OF

THE INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

Los Angeles, USA, 24-28 January 2005

Agenda Item 2:	Air Traffic Management (ATM) Issues

Implementation of Reduced Longitudinal Separation Minimum

in the NOPAC and CENPAC airspace using ADS/CPDLC

(Presented by Civil Aviation Bureau, Japan)

This paper presents information regarding JCAB’s implementation plan for the reduced longitudinal separation minimum in the North and Central Pacific airspace using ADS/CPDLC.

# 1.	Introduction

1.1	At the IPACG/21 meeting, held in Tokyo in June 2004, JCAB indicated that the Multifunctional Transport Satellite (MTSAT) would need to be online for Tokyo FIR to apply 50NM longitudinal separation minimum between aircraft at cruise.

1.2	JCAB also indicated that ADS waypoint reporting and step climb and descent with 50NM longitudinal separation may be implemented prior to the commissioning of MTSAT.

2.	ADS Waypoint Reporting

2.1	Over NOPAC and CENPAC, datalink capable aircraft currently conduct position reporting by CPDLC. JCAB intends to introduce ADS waypoint reporting by replacing CPDLC waypoint reporting after completion of comparative evaluation of them. ADS waypoint reporting data will be evaluated for six months from March 2005.

2.2     ADS waypoint reporting shall be implemented in Tokyo ACC, Anchorage and Oakland ARTCCs at the same time in order to eliminate pilot’s confusion or misunderstanding regarding position reporting. Implementation date of ADS waypoint reporting depends on when the three parties concerned would be prepared.

2.3     It is considered that the introduction of ADS waypoint reporting will significantly reduce workload for controllers and pilots. It will also reduce the communication cost.

3.      Application of 50NM longitudinal separation

3.1    JCAB also plans to introduce 50NM longitudinal separation minimum during level changes (step climb/descent) in the oceanic airspace of Tokyo FIR in April 2005. It is considered that following the introduction of step climb/descent, more aircraft will have opportunities to climb to optimum levels. JCAB intends to introduce the application of 50NM longitudinal separation minimum at cruise using ADS on the same date in a harmonious manner with Anchorage and Oakland FIRs.

4.	Amendment to ICAO Doc 7030

4.1	The approved amendment to the ICAO Regional Supplementary Procedures, Doc 7030 allowing the application of reduced separation minima using ADS/CPDLC (50NM and 30NM longitudinal and 30NM lateral) within the Pacific FIRs became effective by the ICAO State Letter T3/02.8-AP028/04 (ATM) dated 22 April 2004.

5.	Action by IPACG/22

5.1	The IPACG/22 is requested to:

1) Note the JCAB’s plan for replacing the current CPDLC position reporting with ADS waypoint reporting and for the introduction of 50NM longitudinal separation minimum during level changes (step climb and descent) in 2005; and

2) Develop an implementation programme for Anchorage, Oakland and Tokyo FIRs.

--- END ---