THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

(Los Angeles, USA, 25-28 January 2005)

Agenda Item 3:	Communications/Navigation/Surveillance (CNS) Issues

POSITION REPORTING DEFICIENCIES

(Presented by the Federal Aviation Administration)

SUMMARY

This working paper provides an update to the position report deficiencies that have occurred in the Oakland Flight Information Region (FIR) and corrective actions that have been taken.

1.		Background

1.1  	During late 2002, Oakland ARTCC controllers reported an increase in the number of overdue position reports.

1.2	Data was collected to validate this information.  The data collected during November and December 2002 and conversations with airline representatives indicated that additional information would be helpful in identifying possible software problems with controller pilot data link communication (CPDLC) position reporting.

1.3	Oakland ARTCC has been working to reduce the occurrence of “overdue” position reports within the Oakland FIR.  Briefings with statistical data have been provided at the meetings of the Oceanic Work Group (OWG), Informal South Pacific ATS Coordination Group (ISPACG), and IPACG in an effort to reduce the frequency of overdue position reports.

1.4	The FANS Interoperability Team (FIT) Central Reporting Agency (CRA) has been monitoring the overdue CPDLC reports and has reported finding a problem in the FANS software on the B777 (fixed in BP 01), B747 (wind direction reported as 512 degrees), and B767 (current position latitude or longitude uses 60 minutes rather than next whole degree) that would account for some of the missing reports.

2.0	Discussion

2.1	According to ICAO Doc 4444 (PANS/ATM), Section 4.11.1.1, and ICAO Annex 2, Section 3.6.3.1, flight crews should make position reports as soon as possible after passing a compulsory reporting point.  Air traffic service units, in accordance with ICAO Doc. 4444, Section 9.2.2.1, must take action to obtain position reports that are not received within a reasonable period of time in order to provide alerting services if necessary.  The FAA has designated 10 minutes as the reasonable period of time before taking action to obtain a report.

2.2	Overdue position reports continue to be an issue.  The most recent analysis covering the period of 1-20 May 2004, indicates that the number of overdue position reports has declined only slightly since Oakland ARTCC began tracking and reporting on this issue at ISPACG/17 in 2003.

2.3	Several detailed analyses have been conducted since this issue was first identified.  See Attachment 1 for specific data.  During our most recent tracking period from 1 December 2004 to 20 December 2004, there were 603 overdue reports.  Twenty two of these were attributed to known problems with the B744 and the B767.  Of the remaining 581 overdue reports, 312 were CPDLC (approx. 54%) and 269 were HF (approx. 46%).  The daily average was 29.5 (16.5 CPDLC/ 13.5HF).  This is an increase over our previous report in which we were averaging approximately 24 per day (51% CPDLC/ 49% HF).  Time of day does not seem to be a factor.

2.4	One tracked item is the elapsed time from when a controller makes a request for a position report that is considered overdue to the time that the controller receives the report.  The average response time for flight crews using CPDLC was two minutes and for HF was six minutes.  In all cases, once the flight crew was queried, a position report was received.

3.0	Recommendation

3.1	Aircraft operators are urgently requested to explain the importance of this problem to flight crews, remind flight crews of the requirements for position reporting, and to communicate any technical problems to the FANS Implementation Team (FIT).

| Time Period | Total Overdue | Average per Day | CPDLC Reports | HF Reports |
| --- | --- | --- | --- | --- |
| 1-20 May 2004 | 486 | 24.3 | 250 (51.4%) | 236 (48.6%) |
| 1-20 December 2004 | Total Overdue(Non-technical) 581 | 29.5 (16.5 CPDLC 13.5 HF) | 312 (54%) | 269 (46%) |

| Date | Total | CPDLC | Tech | HF |
| --- | --- | --- | --- | --- |
| 12/1/2004 | 29 | 18 | 5 | 11 |
| 12/2/2004 | 23 | 16 | 2 | 7 |
| 12/3/2004 | 32 | 18 | 3 | 14 |
| 12/4/2004 | 32 | 16 | 1 | 16 |
| 12/5/2004 | 33 | 20 | 1 | 13 |
| 12/6/2004 | 43 | 24 | 0 | 19 |
| 12/7/2004 | 21 | 13 | 1 | 8 |
| 12/8/2004 | 36 | 20 | 0 | 16 |
| 12/9/2004 | 25 | 17 | 2 | 8 |
| 12/10/2004 | 29 | 18 | 1 | 11 |
| 12/11/2004 | 28 | 18 | 0 | 10 |
| 12/12/2004 | 23 | 15 | 0 | 8 |
| 12/13/2004 | 25 | 10 | 0 | 15 |
| 12/14/2004 | 40 | 23 | 1 | 17 |
| 12/15/2004 | 24 | 14 | 0 | 10 |
| 12/16/2004 | 22 | 10 | 1 | 12 |
| 12/17/2004 | 32 | 12 | 1 | 20 |
| 12/18/2004 | 44 | 19 | 0 | 25 |
| 12/19/2004 | 30 | 17 | 3 | 13 |
| 12/20/2004 | 32 | 16 | 0 | 16 |