THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/22)

Los Angeles, United States, 24 – 28 January 2005

Agenda Item 3: CNS Issues

USAGE OF DL#68 [FREE TEXT] (DISTRESS) MESSAGE IN CPDLC

(Presented by Civil Aviation Bureau, Japan)

# SUMMARY

This paper prepares present for a discussion on the usage of DL#68 [free text] (Distress) by

pilots and the reaction taken by the controller who received the message.

- Introduction

1.1 　　The Oceanic Control Data Processing (ODP) systems used at Tokyo ACC alerts controllers, when receiving the DL#68 Free Text (Distress) from aircraft. The system is designed to draw the attention of controllers with the message displayed on the aircraft situational display in the attractive red colored letters and at the same time with a letter of ‘E’ displayed on the electric flight progress strips in red color.

1.2	In such cases, the controllers should be required of taking reaction with special handling in precedence over other tasks. However, FANS 1/A Operations Manual does not prescribe the handling priority of this message.

- Discussions

2.1	The controller at Tokyo ACC, who received a DL#68 (Distress) free text message requesting a weather deviation via CPDLC, took the reaction with the manner of confirming the emergency situation.

2.2	There is no detailed description on the usage of DL#68 Free Text (Distress)　in FOM, and the discussion may arise among pilots what CPDLC Text should be used in the urgent situation such as a request of weather deviation.

2.3	Controllers may have a question what reaction should be taken, when they received the message of DL#68 [free text] (Distress), because the text should be used by pilots with the intent of ‘Distress or Urgency’, usually following the prefixed words MAYDAYs or PANs. Therefore the controller who received this message may consider the aircraft to be in emergency and, when having a doubt, may verify the situation. Some controllers may think if it would be possible for pilots to use DL#67 [free text] (Normal) when they request a weather deviation.

3.  Recommendation

3.1	The meeting is recommended to discuss about the detailed explanation on the usage of DL#68 (Distress) by pilots and its responding procedures by controllers to be prescribed in FANS 1/A Operations Manual.