THE TWENTY SECOND MEETING OF THE

INFORMAL PACIFIC ATC COORDINATING GROUP

(IPACG/22)

### (Los Angeles, California, 25-28 January 2005)

Agenda Item 2 – Air Traffic Management (ATM) Issues

OCEANIC IN-TRAIL CLIMB AND IN-TRAIL DESCENT USING ACAS/TCAS & POTENTIAL ADS-B IN-TRAIL Procedure

(Presented by the Federal Aviation Administration)

SUMMARY

This paper presents information about the in-trail climb/descent (ITC/ITD) procedure using Airborne Collision Avoidance System/Traffic Collision Avoidance System (ACAS/TCAS) and introduces a proposal for an oceanic Automatic Dependent Surveillance-Broadcast (ADS-B) in trail procedure.

- Background

1.1 	The Airborne Collision Avoidance System/Traffic Collision Avoidance System (ACAS/TCAS) oceanic in-trail climb and in-trail descent (ITC/ITD) procedure was published in the U.S. Pacific and Alaska Chart Supplements on a ‘test’ basis in the Oakland and Anchorage Flight Information Regions (FIRs) in 1996.

1.2	A concern was raised internally within the /FA?A that this procedure had never been published in the International Civil Aviation Organization (ICAO) Regional Supplementary Procedures.  At IPACG/21 and recent Oceanic Working Group (OWG) meetings, the International Air Transport Association (IATA) and a few airspace operators expressed interest in continuing the procedure.  However, in subsequent informal discussions with the ICAO Asia Pacific Regional Office, we learned that there may not be sufficient justification to establish ITC/ITD as a permanent procedure.

1.3  	The Airline Pilots Association (ALPA) participated in Phase I (in-trail climbs) of the ACAS/TCAS oceanic ITC/ITD program, but in late 1997 opposed Phase II (in-trail descents) because of the transponder cycling procedure and the need for the flight identification (ID) from the leading aircraft. ALPA initially considered these limitations as unacceptable for a long-term operational procedure.

1.4 	Other noted issues limiting the use of the ACAS/TCAS oceanic ITC/ITD procedure during this test period included:  trailing aircraft crew were not always able to acquire other pertinent aircraft due to the limited range of ACAS/TCAS (typically 40 NM); frequent inability to re-acquire the leading aircraft after the transponder standby/on procedure; and the requirement for the leading aircraft crew to actively cooperate.

1.5	The ITC/ITD procedure was rarely requested and eventually was dropped from some operator’s aircrew documentation and training programs.  The FAA considered canceling the test, but at the request of some operators agreed to extend the test program.

1.6	Since resumption of the test in 2004, the procedure has been used only occasionally.  Nevertheless, FAA planned to continue the ITC/ITD test until 30NM longitudinal/30NM lateral separation is initially established in the Oakland Center FIR—estimated for no later than the end of 2005.

1.7	Even if the test was discontinued, FAA planned to retain background information and data collected during the test period for potential future use with oceanic applications of ADS-B or other technologies.

2	Discussion

2.1	During 2004, U. S. Government personnel, in coordination with several operators and avionics vendors, began examining the use of Automatic Dependent Surveillance (ADS-B) to overcome some of the limitations observed with using ACAS/TCAS for the oceanic in-trail ITC/ITD procedure.

2.2	The proposed ADS-B in-trail procedure would use the same (separation and closure rate) criteria authorized by the FAA for the current ITC/ITD procedure using ACAS/TCAS.  Additionally, it will retain ACAS/TCAS as an independent safety system.

2.3	The proposed ADS-B in-trail procedure would address some issues previously noted by allowing the maneuvering (trailing or leading) aircraft to obtain the flight ID, altitude, and ground speed of the non-maneuvering (leading or trailing) aircraft from ADS-B data. This would simplify the procedure by removing the requirement for the non- maneuvering aircraft to give operational approval to participate in this procedure; removing the requirement for the non-maneuvering aircraft to cycle their transponder for a positive ID; and removing the requirement for voice communications between aircraft.

2.4	Because ADS-B has a much greater range than ACAS/TCAS, the ADS-B in-trail procedure may be initiated at greater distances between the aircraft, thereby significantly increasing the percentage of time when an aircraft could utilize this procedure.

2.5	An ADS-B in-trail procedure could be implemented on a test basis, replacing the current ITC/ITD procedure, to determine its operational suitability as a future permanent procedure.

3 	Recommendation

  - The Meeting is invited to:

- consider the information in this paper, and

- comment on the potential for testing an ADS-B in-trail procedure as a replacement for the current test of the ITC/ITD procedures using ACAS/TCAS.