THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

Los Angeles, USA, 25-28 January 2005

Agenda Item 2: Air Traffic Management (ATM) issues

THE STRUCTURE OF FIXED ROUTES

FROM WESTERN PACOTS GATEWAYS TO JAPANESE DOMESTIC AIRSPACE

（Presented by Tokyo ACC / ATFMC）

SUMMARY This working paper explains the impact of traffic flow around the boundary between domestic and oceanic airspace at Tokyo ACC when PABBA composes the part of gateway of westbound PACOTS tracks.

- Background

  - The eastbound PACOTS tracks are generated by ATFMC in Japan and the westbound by Oakland TMU in US respectively on daily basis. By using the weather information of upper-air forecast and considering flight efficiency, PACOTS are designed and made public based on MOU between ATFMC and Oakland/Anchorage ARTCC.

  - A590, one of the five (5) NOPAC routes, is exclusively used for eastbound flights bound for North America and East coast. The number of flights on A590 is the largest among the traffic flying oceanic eastbound routes. Though A590 used to serve for bi-directional traffic, the repeated critical incidents in oceanic airspace ended up designating A590 for eastbound only.

- Discussion

  - PABBA, located on the boundary between domestic and oceanic airspace, composes the part of gateways for westbound PACOTS. The procedural separation shall be applied between the westbound flight on PACOTS inbound PABBA and the northeastbound flight on A590 through PABBA because of insufficient radar coverage.

  - The derived bi-directional traffic flow over PABBA potentially deteriorates efficiency in aircraft operations. Sometimes the eastbound overflights are forced to descend temporarily, and sometimes the departures via A590 from airports in Japan are not able to take planned altitude due to westbound flights coming to PABBA.

- Recommendation

  - Tokyo ACC and ATFMC are invited to start the study on:

  - a correlation between the established PACOTS tracks and their impact on the ATC operations around PABBA.

  - a solution used by established ATS resources to change the traffic flow.

  - a solution by redesigning route structures.

  - The meeting is invited to note the information in this paper and comment.