THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

(Los Angeles, USA, 25-28 January 2005)

Agenda Item 2:	Air Traffic Management (ATM) Issues

Oakland ARTCC Update on Oakland FIR Waypoints ON PACOTS Tracks

(Presented by the Federal Aviation Administration)

SUMMARY

This paper provides an update to WP/3 (Oakland FIR Waypoints on PACOTS Tracks) that was presented at IPACG/21.

1.		Background

  - The Pacific Organized Track System (PACOTS) tracks are currently defined with waypoints at every 10 degrees of longitude.  Aircraft filed on the PACOTS tracks are required to report only the waypoints published for the track they are flying and are not required to make a flight information region (FIR) boundary report if it is not a published waypoint.

1.2	Without an FIR progress report controllers do not always have the information they need to provide additional services such as altitude changes between the FIR and the next published reporting point.

1.3	At IPACG/21 Oakland ARTCC presented a paper which recommended that a waypoint be added to the PACOTS tracks at the FIR boundary between Tokyo Area Control Center (ACC) and Oakland Air Route Traffic Control Center (ARTCC).

1.4	This was recommended in order to provide accurate and complete position report information at the FIR boundary to allow controllers to provide more additional services requiring this exact position information, such as applying exactly 10 minutes separation at the same altitude.

1.5	At IPACG/21 it was agreed that a survey would be done to identify what the 165E crossing points would be and that this information would be reported back at IPACG/22.

2.	Discussion

2.1	Since IPACG/21, Oakland ARTCC has discussed this issue several times at the Oceanic Work Group (OWG) meetings.  During these discussions it became apparent that adding waypoints to the PACOTS tracks at the FIR boundary would create dispatching and/or Flight Management System (FMS) problems for some of the customers that auto-load their flight plans.

2.2	It was also noted at the last OWG meeting that some customers were already providing complete position reports at the FIR on their own.

2.3	Due to the concerns raised, Oakland ARTCC is withdrawing is request to add FIR crossing points to the PACOTS

3.	Recommendation

3.1	The meeting is invited to:

a.	consider the information in this paper,

b.	comment as appropriate; and

c.	encourage operators to provide complete FIR position reports when possible.