THE TWENTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/22)

Los Angeles, USA, 24-28 January 2005

Agenda Item 2: Air Traffic Management (ATM) Issues

# CPDLC and HF Position Reporting Deficiencies

(Presented by Tokyo ACC)

## Summary

This paper summarizes comparative study of CPDLC and HF position report deficiencies that have occurred in Tokyo ACC oceanic airspace

- Background

  - Oakland ARTCC has been reported the evaluation about the overdue position reports on CPDLC and HF occurred in Oakland airspace at IPACG19-21.

  - At IPACG20, Tokyo ACC also has presented the CPDLC position reports’ deficiencies.

  - Following the increasing overdue position reports at Oakland ARTCC, Tokyo ACC was requested to identify how much overdue position reports actually occurred in oceanic airspace in Tokyo FIR and to report the evaluation at IPACG22.

- Observations

  - In Tokyo FIR, the ratio of aircrafts using CPDLC as primary communication means is about  35%, ones using HF is about 65 %.（As of December. ’04）

  - The survey of overdue position report at Tokyo ACC was conducted during one month, Nov. 1-30, 2004.

  - The data were collected manually by controller at the Oceanic sector. The overdue reports from both CPDLC and HF delayed more than 5 minutes were reported. Refer to Attachment 1 for specific data.

Approximately 430 flights fly in oceanic airspace per day.

Total of position report is approximately 1,290 per day. (Supposed three compulsory position reports are made per flight. The number of compulsory reporting points as follows; NOPAC--- 3, PACOTS --- 2(eastbound) ／ 3(westbound), other airways ----- 3 or 4)  The average of overdue is 13 per day. The ratio of overdue position reports is about 1%.

Attachment 1 indicates that:

1）shows the number of overdue, total of the month and average per day,

2）shows route-specific data

The pie chart 1 displays the ratio of overdue by routes

The pie chart 2 displays the ratio of total position reports by route.

Overdue of position reports on NOPAC was observed more frequent than on other routes.

3) categorizes type of aircrafts.

No specific factor of type of aircraft is identified on overdue.

The graph indicates the distribution of the overdue position report by HF and CPDLC.

  - Many overdue by HF position report seemingly include not only the elapsed time consumed by flight crews’ operation but also the elapsed time for messages being relayed to controllers.

  - According to the CPDLC performance concerning one way time for messages being down-linked, 95 % of the messages reached within 1 minute, and 99% arrives at ground system within 3 minutes.

- Procedures

3.1 　According to ICAO Doc.4444, Section 4.11.1.1 and Annex2, Section 3.6.3.1, position reports should be made as soon as possible after passing each designated compulsory reporting point.

3.2 　According to ICAO Doc.4444, Section 9.2.2.1, Air traffic service units must take action to obtain position reports that are not received within a reasonable period of time in order to provide alerting services if necessary.

3.3 	JCAB has designated 5 minutes as the reasonable amount of time before action must be taken.

- Discussion

In order to solve the overdue position reports, human factors such as CPDLC messages being sent manually by flight crews should be considered. In this perspective, the implementation of ADS waypoint reports may be effective because messages of ADS are transmitted automatically without flight crews operation. Replacing CPDLC position reporting with ADS position reporting will be able to reduce workloads for not only flight crews but also controller.

- Recommendation

5.1	Operators are encouraged to remind flight crews of the requirement for position reporting without delay.

5.2	Operators are encouraged to remind flight crews that the position reporting with next COMPULSORY WAYPOINT as next position will be desirable for ATC operation.

5.3	The meeting is invited to communicate any technical problems to the FANS Implementation Team (FIT) for the implementation of ADS position reporting for the replacement of CPDLC position reporting and to consider closing this action item.