----- Message from "COMNAVAIRSYSCOM PATUXENT RIVER MD\(uc\)" on Unknown -----

| To: | "CO MCB HAWAII\(uc\)" |
| --- | --- |
| cc: | "PEOSTRKWPNSUAVN PATUXENT RIVER MD\(uc\)", "PEOASWASM PATUXENT RIVER MD\(uc\)", "COMNAVAIRSYSCOM PATUXENT RIVER MD\(uc\)" |
| Subject: | P152000Z DEC 10 UAS CAT 3 IFC EXTENSION FOR RQ-11B RAVEN COA OPS AT KANEOHE |

PASS TO OFFICE CODES:
FM COMNAVAIRSYSCOM PATUXENT RIVER MD//4.0P//
INFO PEOSTRKWPNSUAVN PATUXENT RIVER MD//PMA263//
COMNAVAIRSYSCOM PATUXENT RIVER MD//5.0D/4.1.1.5/4.0P//
MSGID/GENADMIN/MIL-STD-6040(SERIES)/B.0.01.00/NAVAIRSYSCOM AIR-4.0P/-
/-/-/-/-/UNCLASSIFIED//
SUBJ/UAS CAT 3 IFC EXTENSION FOR RQ-11B RAVEN COA OPS AT KANEOHE
/BAY, HI//
REF/A/DESC:DOC/COMNAVIARSYSCOM/140806ZDEC2010//
REF/B/MSGID:GENADMIN/COMNAVAIRSYSCOM/312009ZAUG2010//
REF/C/DESC:DOC/FAA/04AUG2010//
REF/D/DESC:DOC/USA/28MAR2008//
REF/E/DESC:DOC/NOSSA/27SEP2007//
REF/F/DESC:DOC/NOSSA/26JAN2007//
REF/G/DESC:DOC/USA/28MAR2008//
REF/H/DESC:DOC/IEC/01JAN2007//
REF/I/DESC:DOC/USN/08JAN2008//
REF/J/DESC:DOC/USMC/18AUG2010//
REF/K/DESC:DOC/USMC/13AUG2010//
REF/L/DESC:DOC/USAF,USN,USA/02FEB2007//
REF/M/DESC:EML/NAVAIR/16AUG2010//
REF/N/DESC:DOC/NAVAIR/18MAY2006//
REF/O/DESC:NAVAIR 13034.1D/NAVAIR/15MAR2010//
NARR/REF A IS THE SUBJECT FLIGHT CLEARANCE REQUEST.
REF B IS THE PREVIOUS UAS INTERIM FLIGHT CLEARANCE 
FOR RQ-11B RAVEN COA OPS AT KANEOHE BAY, HI.
REF C IS THE CERTIFICATE OF AUTHORIZATION (COA) APPLICATION FOR
COA, APPLICATION NO. 2009-WSA-126.
REF D IS THE TM 1-1550-695-13&P, OPERATOR AND FIELD MAINTENANCE
MANUAL INCLUDING REPAIR PARTS AND SPECIAL TOOLS LIST FOR SMALL
UNMANNED AIRCRAFT SYSTEM (SUAS) RQ-11B.
REF E IS THE SAFETY APPROVAL FOR LITHIUM BATTERIES.
REF F IS THE HAZARDS OF ELECTROMAGNETIC RADIATION TO ORDNANCE SAFE
SEPARATION DISTANCES.
REF G IS THE TECHNICAL MANUAL 1-1550-695-CL, OPERATORS AND
CREWMEMBERS CHECKLIST, RQ-11B SUAS.
REF H IS THE IEC 60825-1 SAFETY OF LASER STANDARDS.
REF I IS THE LASER SAFETY REVIEW BOARD (LSRB) APPROVAL LETTER,
SER 010/LSRB.
REF J IS THE SOP FOR UAV OPERATIONS AT MARINE CORPS AIR FACILITY
(MCAF) AND MARINE CORPS TRAINING AREA BELLOWS (MCTAB).
REF K IS THE AREA OF OPERATIONS FOR MCAF AND MCTAB.
REF L IS THE TRI SERVICE MEMORANDUM OF AGREEMENT (MOA), MUTUAL
ACCEPTANCE OF AIRWORTHINESS CERTIFICATION DATA
REF M IS THE HIGHER PROBABILITY OF LOSS STATEMENT.
REF N IS THE MEMORANDUM, SER 5.0307, FROM AIR-4.0P TO THE FAA
REGARDING STATEMENTS OF AIRWORTHINESS FOR NAVY AND MARINE CORPS UAS
IN SUPPORT OF COA APPLICATIONS.
REF O IS THE FLIGHT CLEARANCE POLICY FOR AIR VEHICLES AND AIRCRAFT
SYSTEMS.//
POC/RON LYLISTON/UAS FACIILITATOR/UNIT:PATUXENT RIVER, MD  
/NAME: PATUXENT RIVER MD/TEL: 301-757-0503  
/EMAIL:RONNIE.LYLISTON.CTR@NAVY.MIL//
GENTEXT/REMARKS/1. IRT REF A AND ISO REF C, NAVAIR CATEGORY 3 (SEE
PARA 7.H) INTERIM FLIGHT CLEARANCE (IFC) EXTENSION IS GRANTED TO
THE SMALL UNMANNED AIRCRAFT SYSTEM (SUAS) RAVEN RQ-11B. THIS FLIGHT
CLEARANCE IS FOR TESTING, TRAINING, AND CONOPS DEVELOPMENT. THIS
FLIGHT CLEARANCE IS IN SUPPORT OF A FAA CERTIFICATE OF
AUTHORIZATION (COA). THE AREA OF OPERATIONS INCLUDE THE MARINE
CORPS AIR FACILITY (MCAF) AND THE MARINE CORPS TRAINING AREA
BELLOWS (MCTAB) LOCATED IN HAWAII IAW REF K. REF B IS CANCELED AND
SUPERCEDED BY THIS CLEARANCE. THIS FLIGHT CLEARANCE IS FOR BOTH 4
AND 8 CHANNEL RAVEN SYSTEMS. THIS FLIGHT CLEARANCE IS SUBJECT TO
THE FOLLOWING CONFIGURATION, LIMITS, WARNINGS, CAUTIONS AND NOTES.
2. TAKEOFF CONFIGURATION: BASELINE CONFIGURATION IAW REFS D, E
   AND AS FOLLOWS:
   A. DIMENSIONS: WINGSPAN 55 INCHES, LENGTH 36 INCHES
   B. PAYLOAD:
      (1) DUAL FORWARD AND SIDE LOOK EO CAMERA NOSE
      (2) SIDE LOOK IR CAMERA NOSE WITH CLASS 1M (IAW REF I) IR
          LASER ILLUMINATOR
   C. POWER: SINGLE DIRECT DRIVE ELECTRIC MOTOR
   D. AVIONICS: AEROVIRONMENT MINIATURE AUTOPILOT
   E. NAVIGATION: P(Y) CODE GLOBAL POSITIONING SYSTEM/SELECTIVE
      AVAILABILITY ANTI-SPOOFING MODULE (GPS/SASSM) WITH ELECTRONIC
      COMPASS
   F. DATALINK: 8 CHANNEL SELECTABLE (UPLINK/DOWNLINK):
      (1) CH 1: 395.000 MHZ/1787.5 MHZ
      (2) CH 2: 395.050 MHZ/1810.0 MHZ
      (3) CH 3: 395.100 MHZ/1840.0 MHZ
      (4) CH 4: 394.950 MHZ/1760.0 MHZ
      (5) CH 5: 371.750 MHZ/1717.5 MHZ
      (6) CH 6: 372.950 MHZ/1737.50 MHZ
      (7) CH 7: 378.150 MHZ/1752.50 MHZ
      (8) CH 8: 384.750 MHZ/1777.50 MHZ
   G. SOFTWARE
      (1) AUTOPILOT: RAVEN B AVIONICS FIRMWARE V22.02
      (2) GCS: GCU2 HUB V1.4.8, HAND CONTROLLER V1.4.8, RF UNIT
          V2.0.0
      (3) GCS LAPTOP OPERATING SYSTEM: WINDOWS XP SP2, FALCONVIEW
          TOOLBAR V2.5.0.0.
   H. ELECTRICAL:
      (1) AIR VEHICLE: LITHIUM ION BATTERY
      (2) GCS: LITHIUM BATTERY
3. LIMITATIONS: IAW REFS D THROUGH K AS FOLLOWS:
   A. GROSS WEIGHT: 4.2 LB
   B. LAUNCH ALTITUDE: 10,000 FT MSL MAXIMUM
   C. LANDING ALTITUDE: 10,000 FT MSL MAXIMUM
   D. FLIGHT ALTITUDE: 300-500 FT AGL, 10,500 FT MSL MAXIMUM
   E. FLIGHT TIME: 90 MINUTES MAXIMUM
   F. WIND SPEED: 20 KNOTS MAXIMUM
   G. RAIN: 0.25 INCHES PER HOUR MAXIMUM
   H. FLIGHTS SHALL AVOID OTHER RAVEN TRANSMITTERS BY AT LEAST 200
      METERS. GROUND CONTROL STATIONS SHALL BE LOCATED AT LEAST
      400 METERS APART.
   I. EMERGENCY PROCEDURES IAW REFS D AND G.
4. WARNINGS, CAUTIONS AND NOTES: IAW REFS D THROUGH K AND AS
   FOLLOWS:
------------------------------WARNING------------------------------
   A. PER REF H AND I, AN EYE HAZARD EXISTS IF VIEWING THE LASER
      WITH BINOCULARS WITHIN A DISTANCE OF 2.2 METERS (7.2 FEET).
      BINOCULARS SHALL NOT BE USED TO VIEW THE LASER WHEN IT IS
      OPERATING AND WITHIN 2.2 METERS. PREFLIGHT VERIFICATION OF
      LASER ILLUMINATOR FUNCTION SHALL BE DONE WHILE AIMING THE
      BEAM STRAIGHT DOWN ONTO A NON-REFLECTIVE, DIFFUSIVE SURFACE
      SUCH AS DIRT OR DRY GRASS. NEVER DIRECT THE ILLUMINATOR
      UPWARDS OR AT ANYONE DURING THIS CHECK. THE ILLUMINATOR
      MUST BE IN THE OFF STATE AT THE END OF PREFLIGHT CHECK, TO
      ENSURE NO RISK TO PERSONNEL. ALSO, DURING RETRIEVAL OF THE
      AIR VEHICLE, PERSONNEL SHALL TREAT THE LASER ILLUMINATOR AS
      IF IT WERE IN THE ON STATE AND TAKE APPROPRIATE
      PRECAUTIONS UNTIL THE AIR VEHICLES BATTERY HAS BEEN
      DISCONNECTED. FAILURE TO COMPLY MAY RESULT IN INJURY TO
      PERSONNEL.
------------------------------WARNING------------------------------
   B. THE RQ-11B RAVEN DOES NOT HAVE A SENSE AND AVOID SYSTEM.
      MID-AIR COLLISION IS A RISK. ALL FLIGHT OPERATIONS SHALL BE
      CONDUCTED TO ENSURE THAT MANNED AND UNMANNED AIRCRAFT SHALL
      NOT OCCUPY THE SAME AIRSPACE. FAILURE TO COMPLY MAY RESULT
      IN INJURY, THE LOSS OF THE AIR VEHICLE AND/OR THE LOSS OF
      PROPERTY.
------------------------------WARNING------------------------------
   C. THE RQ-11B RAVEN UAS HAS NOT UNDERGONE SOF EXPLOSIVE
      ATMOSPHERE TESTING. A SERIOUS FIRE OR EXPLOSION MAY RESULT
      IF THE AIRCRAFT IS POWERED WHILE FLAMMABLE VAPORS ARE PRESENT
      DURING GROUND OR FLIGHT OPERATIONS. DO NOT OPERATE THE
      SYSTEM IN THE PRESENCE OF FLAMMABLE VAPORS. FAILURE TO COMPLY
      MAY RESULT IN THE LOSS OF THE AIR VEHICLE AND MAY RESULT
      IN THE INJURY OF PERSONNEL AND DAMAGE TO PROPERTY.
------------------------------WARNING------------------------------
   D. CHARGING OF THE LITHIUM BATTERY SHALL ONLY BE CONDUCTED
      USING THE AV UNIVERSAL BATTERY CHARGER P/N 56500. THE
      BATTERY IS SUSCEPTIBLE TO VIOLENT FAILURE IF CHARGED ON AN
      UNMONITORED NON-RAVEN B CHARGER. EVEN PROPER MONITORING OF
      A NON-RAVEN B CHARGER DURING CHARGE AND DISCHARGE RESULTS IN
      INCREASED CELL IMBALANCES OVER SEVERAL CHARGE/DISCHARGE
      CYCLES. FAILURE TO COMPLY MAY RESULT IN INJURY AND/OR SYSTEM
      DAMAGE.
------------------------------WARNING------------------------------
   E. NOISE CHARACTERIZATION TESTING HAS NOT BEEN COMPLETED ON
      THIS SYSTEM. PROLONGED EXPOSURE TO ENGINE NOISE AT CLOSE
      PROXIMITY MAY CONTRIBUTE TO HEARING LOSS. IT IS RECOMMENDED
      THAT EAR PLUGS BE WORN WHILE LAUNCHING THE AIR VEHICLE.
      FAILURE TO COMPLY MAY RESULT IN INJURY TO PERSONNEL.
------------------------------WARNING------------------------------
   F. OPERATION SHOULD BE CONDUCTED WELL AWAY FROM ANTENNAS OF
      ACTIVE RADARS AND RADIO TRANSMITTERS. THIS IS TO AVERT
      DISRUPTION TO THE DATALINK, GPS, VIDEO, OR AIR VEHICLE
      CONTROL OF THE RQ-11B RAVEN. AIR VEHICLE LOSS IS POSSIBLE AND
      POSSIBLE INJURY TO PERSONNEL.
      (1) SPECIFIC STAND-OFF DISTANCES CANNOT BE PROVIDED WITH
          CONFIDENCE: RELEVANT TEST DATA IS ABSENT.  
      (2) BASED UPON OTHER SMALL UAS, AND IN THE ABSENCE OF TEST
          DATA, IT IS SUGGESTED THAT THE AIR VEHICLE BE FLOWN NO
          CLOSER THAN 2 NM TO VERY-HIGH-POWERED AIR-SEARCH AND
          FIRE-CONTROL RADARS. AN ADEQUATE SEPARATION DISTANCE FROM
          LOW-POWERED (100 WATT OR LESS) TRANSMITTERS WITH LOW-GAIN
          ANTENNAS, WHICH OPERATE AWAY FROM RQ-11B RAVEN DATALINK
          FREQUENCIES, WILL LIKELY BE A FEW TENS OF METERS.
------------------------------CAUTION------------------------------
   G. THE RQ-11B RAVEN UAS HAS COMPLETED LIMITED ENVIRONMENTAL
      CHARACTERIZATION TESTING. OPERATE THE SYSTEM WITHIN THE
      TEMPERATURE RANGE OF ZERO DEGREES CELSIUS TO 50 DEGREES
      CELSIUS AND BELOW 10,500 FT MSL TO THE GREATEST EXTENT
      POSSIBLE. FAILURE TO COMPLY MAY RESULT IN THE LOSS OF THE
      AIR VEHICLE.
------------------------------NOTE---------------------------------
   H. AN ELECTROMAGNETIC COMPATIBILITY (EMC) SAFETY-OF-FLIGHT TEST
      (SOFT) IS REQUIRED PRIOR TO FIRST FLIGHT WITH ANY
      CONFIGURATION CHANGE INVOLVING ELECTRONIC/ELECTRICAL
      EQUIPMENT OR WIRING. AN EMC SOFT IS NOT COMPLETE UNTIL ALL
      UNACCEPTABLE ANOMALIES HAVE BEEN CORRECTED, AS VIA
      MODIFICATIONS TO OPERATIONAL PROCEDURES. COMPLETION OF AN EMC
      SOFT IS A CHECK, BUT NOT A GUARANTEE, THAT OPERATION OF THE
      RQ-11B RAVEN WILL NOT BE DISRUPTED BY ITS OWN OR NEARBY
      ELECTRONIC EQUIPMENT.
------------------------------NOTE---------------------------------
   I. ACCIDENTAL OPERATION OF THE RQ-11B RAVEN UAS OUTSIDE OF
      RESTRICTED AIRSPACE SHALL BE IMMEDIATELY REPORTED TO AIR
      TRAFFIC CONTROL (ATC). THE OPERATOR SHALL MAKE IMMEDIATE
      ACTIONS TO CORRECT THE FLIGHT PATH AND/OR FOLLOW ATC
      DIRECTION.
------------------------------NOTE---------------------------------
   J. RADIO CHANNELS 1 THROUGH 4 SHOULD NOT BE UTILIZED FOR
      OPERATIONS CONDUCTED IN OIF.
5. TIME PERIOD: THIS INTERIM FLIGHT CLEARANCE EXPIRES 1 FEB 2012.
6. POINTS OF CONTACT:
   A. FRANK DIGEORGE NAVAIR PMA-263 IPT LEAD, 301-757-9002,
      EMAIL:FRANK.DIGEORGE@NAVY.MIL.
   B. BRAD BRANCH, AIR-4.1.1.5 SYSTEM ENGINEER SMALL UAS
      CLASS DESK, 301-342-0272, EMAIL: WILLIAM.BRANCH@NAVY.MIL
   C. RON LYLISTON, PMA-263 FACILITATOR, 301-757-0503,
      EMAIL: RONNIE.LYLISTON.CTR@NAVY.MIL
   D. AIRWORTHINESS GLOBAL SUPPORT TEAM, 301-757-0187 OR 
      E-MAIL: AIRWORTHINESS@NAVY.MIL.
7. OTHER REMARKS:
   A. THE RAVEN B LOW RATE INITIAL PRODUCTION (LRIP) HAND
      CONTROLLER IS INCOMPATIBLE WITH THE FULL RATE PRODUCTION
      (FRP) HUB. THE SYSTEM WILL NOT INITIALIZE WITH THIS
      CONFIGURATION.
   B. APPROVAL TO FLY UAS WITHIN THE US NATIONAL AIRSPACE SYSTEM
      (NAS) OUTSIDE OF RESTRICTED OR WARNING AREAS, IS SUBJECT TO
      FAA APPROVAL.
   C. IAW REFS O, THIS IFC CERTIFIES THAT THE UAS IS
      AIRWORTHY AT THE CATEGORY 3 LEVEL.
   D. COORDINATION OF CHANNEL SELECTION WITH OTHER SYSTEMS
      OPERATING IN THE AREA IS CRITICAL TO REDUCING ACCIDENT RATES.
   E. INITIATING REQUEST FOR AND ADHERING TO RANGE AND AIRSPACE
      CLEARANCE IS THE RESPONSIBILITY OF THE OPERATORS.
   F. THIS FLIGHT CLEARANCE DOES NOT ADDRESS INTEROPERABILITY WITH
      MANNED AIRCRAFT.
   G. UAV OPERATOR QUALIFICATIONS, MAINTAINER QUALIFICATIONS,
      CONFIGURATION MANAGEMENT, ESTABLISHED FLIGHT TEST PRACTICES,
      SCHEDULED/UNSCHEDULED MAINTENANCE PRACTICES, AND OTHER
      ASSOCIATED PROCEDURES ARE BEYOND THE SCOPE OF THIS FLIGHT
      CLEARANCE BUT SHOULD BE ADEQUATELY ADDRESSED BY AN
      APPROPRIATE AUTHORITY TO ENSURE AN ACCEPTABLE LEVEL OF
      SAFETY FOR THE UAS AND TO MINIMIZE RISK TO GROUND PERSONNEL
      AND PROPERTY.
   H. PER REF O, THIS FLIGHT CLEARANCE PROVIDES A NAVAIR CATEGORY 3
      AIRWORTHINESS CERTIFICATION SUBSEQUENT TO AN ENGINEERING
      REVIEW. AS DEFINED IN PARA 6.E.2.C, CATEGORY 3 FLIGHT
      CLEARANCES ARE ISSUED FOR UAS THAT ARE NOT DESIGNED TO ANY
      ACCEPTED ENGINEERING STANDARDS AND/OR DO NOT POSSESS ADEQUATE
      ENGINEERING DATA TO SUBSTANTIATE AIRWORTHINESS LEVELS
      TYPICALLY ASSOCIATED WITH MANNED AIRCRAFT. THEREFORE, THIS
      IFC CERTIFIES AIRWORTHINESS TO A LEVEL CONSISTENT WITH THE
      PROPOSED USE AND ASSOCIATED OPERATIONAL RESTRICTIONS,
      PRIMARILY TO INCREASE THE SAFETY OF ALL PERSONNEL INVOLVED IN
      FLIGHT OPERATIONS.
   I. PER REF M, THE 263 ACKNOWLEDGES HIGHER PROBABILITY
      OF LOSS OF THE UA, AS DEFINED IN PARA 6.E.3.C OF REF O.
   J. THIS IFC DOES NOT AUTHORIZE AIRCRAFT/SYSTEM MODIFICATION, NOR
      DOES IT SATISFY NAVAIR REQUIREMENTS FOR CONFIGURATION
      MANAGEMENT. AS DIRECTED BY OPNAVINST 4790.2J, REFER TO
      CNAFINST 4790.2A FOR POLICY GUIDANCE ON CONFIGURATION
      MANAGEMENT AND MOD AUTHORITY.
   K. INFORMATION REGARDING THE AIRWORTHINESS OFFICE AND PROCESS,
      INCLUDING A LISTING OF ALL CURRENT INTERIM FLIGHT CLEARANCES
      ISSUED BY NAVAIR 4.0P AND 24/7 CONTACT NUMBERS, CAN BE FOUND
      ON OUR WEBSITE: AIRWORTHINESS.NAVAIR.NAVY.MIL.
   L. EPOWER FOLDER 927599, TRACKING NUMBER 39734.//