REMOTE VIDEO TRANSCEIVER (RVT)

A second GCS can be used as an RVT. The RVT is used to view real-time video from any location within range of the AV. To operate a GCS as an RVT, leave uplink antenna disconnected and transmitter switched off.

Payloads

EO Payload

The RQ-11B EO payload is equipped with digital front and side-look cameras capable of pan, tilt and zoom functions.  The forward look camera has a resolution of 768 x 492.  The side look EO camera has a resolution of 2048 x 1536

EO Zoom Levels

When the side-look camera is selected, the following three zoom magnifications are available: 1, 1.42 and 2.85. These correspond to horizontal fields of view of 34 deg., 24 deg. and 12 deg. To change zoom level, depress and hold the payload button (see WP 0005 00, Table 3) while pushing the Menu Button to the left to cycle through successive zoom levels.  The image will change accordingly, and the zoom level indication will update. There will be a delay in the indicated zoom level because the aircraft must first downlink confirmation of the selected zoom level.

EO Pan and Tilt

When in loiter (LOIT) mode or using orbit waypoints and with mid-level or full zoom selected, the joystick moves zoomed center field of view within the constrains of non-zoomed field of view to digitally Pan and Tilt the image.

EO Image Stabilization Mode

If the AV is operating in Manual (MAN), Altitude Hold (ALT), HOME, or navigation (NAV) mode, the pan and tilt functions act to digitally stabilize the camera using inputs from the AV’s gyroscopes. The AV can be flown normally at all zoom levels while the image stabilization occurs automatically.

Pointing Mode

If the AV is flying in a circle around a loiter point or any orbit waypoint, the camera is directed to look at the center field of view coordinate. In this mode, flight control of the AV is disabled and the joystick controller allows the operator to redirect the camera. This way, the operator can adjust the center field of view onto a point of interest or reacquire the loiter point if orbit is not positioned correctly by pressing Enter twice.

IR Payload

The IR payload is equipped for a forward look and side look cameras.  The side look IR camera includes a laser IR illuminator. The illuminator is visible with night vision goggles but not visible through the thermal imager in the IR payload. The IR Payload allows selection of either white-hot or black-hot polarity.