9-209	LOST COMMUNICATIONS

a.	Lost Communication in the CDSA.

(1)	In the event the Ground Control Station (GCS) loses communications with tower in the CDSA, the UAS operator will squawk 7600 and program the UAS to proceed to the recovery orbit for the active UAS landing area and hold until receiving the appropriate ALDIS lamp signals from the control tower.

(2)	Once established in the recovery orbit, the UAS operator will begin the auto-recovery sequence when given the appropriate signal from the tower.  Standard light signals will be used thereafter until the UAV is safely recovered.

b.	Lost Communications Enroute.

(1)	In the event the GCS loses communications while enroute to the restricted area, the UAS operator will squawk 7600, climb to 5,000’ MSL, and proceed to the restricted area exit/entry point.  At that time the UAV pilot will execute the procedures outlined in paragraph c. below.

(2)	UAV's losing communication enroute to the destination airport after exiting restricted airspace will continue to the landing runway and execute procedures as per paragraph a. above.

c.	Lost Communication in Restricted Airspace.

(1)	Remain in the restricted airspace while attempting to contact the controlling agency via secondary means to include land line.

(2)	In the event the Ground Control Station (GCS) is unable to re-establish communication with the airspace controlling agency while operating in the restricted area, the UAS operator will squawk 7600 and fly the UAS to the entry/exit point for the designated transit corridor to the destination airfield and begin flying the triangle pattern IAW section A, Paragraph 1(e) of the Flight Information Handbook (FIH).

(3)	IAW the FIH, ATC will acknowledge the lost communication by using standard ALDIS lamp signals from the control tower.  Once the GCS receives permission via the ALDIS signals to return to the airport, then, and only then will the UAS descend to 4,000’ MSL and proceed directly to the UAS recovery orbit for the active UAS landing area.