e.   LOST COMMUNICATIONS

1.	Lost Communications Enroute.  In the event the GCS loses communications while enroute to the restricted area, the UAS operator will squawk 7600, climb to and/or maintain 2,300’ MSL, and proceed to the restricted area exit/entry point.  At that time the UAV pilot will execute the procedures outlined in paragraph e.2 below.  	UAV's losing communication enroute to the Cannon Air Defense Complex landing strip after exiting restricted airspace will continue to the landing strip. The UAS operator will squawk 7600 and program the UAS to proceed to the recovery orbit an carry out their auto-recovery sequence in order to make an active UAS landing.  The VMU-4 Area of Operations within Class G/E airspace (the transition corridor) is automatically sterilized once the aircraft is given clearance to exit R2301W restricted airspace and transition back to Cannon Air Defense Complex. No aircraft will exit R2301W without the appropriate clearance.

2. 	Lost Communication in Restricted Airspace. In the event the GCS loses communications while in the  restricted area, the UAS shall remain in the restricted airspace while attempting to contact the controlling agency via secondary means to include land line.  In the event the Ground Control Station (GCS) is unable to re-establish communication with the airspace controlling agency, the UAS operator will squawk 7600 and make a pre-programmed landing to Aux II landing field which is located within R2301W.