- Lost link/mission abort procedures.  Emergency lost link procedures are loaded into the UAS prior to the mission and will remain the same whether the link is lost within Class G airspace or within R2301W. These procedures require the UAS to climb or descend to “Loiter Point 1” or “Loiter Point 2” within R2301W restricted airspace and maintain an altitude of 2300’ MSL. See Attachment 1 below for Loiter Point locations. These procedures can be updated in flight as the situation or phase of flight dictates.  The PIC shall notify Yuma ATC Range immediately when the data link is lost as well as the preprogrammed routing of the aircraft. In the event that the link cannot be restored with the UAS during the predetermined holding period, the Flight Termination System (FTS) will be activated to allow for a controlled recovery within R2301W.  The FTS is an emergency system comprised of a recovery parachute that can be deployed from either the control station or by the aircraft once a pre-determined set of conditions have been met.  Its function is to safely recover the air vehicle with minimum damage to the air vehicle, persons or property during an emergency flight termination.