Launch Description/Procedure:

The Raven RQ-11B is hand launched.  Upon completion of Preflight Check, the AV is ready for launch. To launch:

1. (VO) Determine wind direction. Always launch AV into wind.

2. (MO) Point AV nose in desired takeoff direction.

3. (VO) Select MAN mode (using Hot Key Button). Horizontal stabilizer will move from Auto land to flight position. VO states “Clear prop.”

4. (MO) Grasp AV from bottom aft of skid pad with Uplink Antenna positioned between palm and skid pad to ensure antenna stays clear of prop during launch. Ensure uplink antenna is hanging free and not tangled. Assume correct stance for launching AV. Acknowledge with “Prop clear.”

5. (VO) Throttle up to full power (press Toggle Switch forward and hold until 100% throttle achieved (three seconds)).

6. (MO) Wait for motor full power to stabilize (approx. 3 seconds), then throw AV into wind.

7. (VO)  Should be prepared to provide joystick inputs to adjust for desired flight path, however, if launch appears to be unrecoverable, VO must be prepared to auto land immediately.

8. (VO) Maintain climb direction into wind until approximately 200-300 feet AGL then select appropriate flight mode.