Landing Description/Procedure:

The RQ-11B is designed with an Auto land feature that causes the AV to descend in a deep stall.  The AV should always be landed into the wind. This helps to slow forward motion and stabilize the AV horizontally during descent. The landing site should be surveyed before initiating a landing or from the air just prior to activating the landing.  Upon command of Auto land, power is cut from the engine and the stabilator deflects fully upward. The AV pitches nose up and begins a steep descent to the ground with wings level. Impact with the ground usually causes the AV components to separate. This is normal and helps dissipate the impact of landing and protect the AV. A landing pad on the bottom of the fuselage also provides impact protection.

The following procedure outlines a VO-initiated landing.

1. Ensure laser illuminator is off if IR payload is attached.

2. Direct AV down to altitude of approximately 75 to 100 ft. AGL and orient AV into wind.

3. When AV is approximately 45 degrees from operator’s LOS and still at approximately 75 to 100 ft. AGL, AV is properly oriented into wind for Auto land.

4. Press Hot key and Enter Buttons simultaneously to manually enter Auto land. Auto land on VO Controller will flash.

5. VO should continue to fly AV to ground, ensuring nose of AV remains directed into wind.