Raven-B Loss of Link/Emergency Description

Loss of Link Procedures For Operations at MCBH:  The Go To Rally (Land WP) and Autoland Loss of Link mode will be utilized during operations at MCTAB and at MCAF.  The Go To Rally location will be the same as the land waypoint.  At MCTAB this will be located the Helicopter Landing Zone (LZ) located in the center of the training area.  At MCAF this will be the LZ at West field, located a safe distance away from the runway.

The Raven-B air vehicle has three automatic Loss of Link = LOL modes.  The aircraft detects loss of link if data is not received for longer than 3 seconds and then automatically responds and initiates its Loss of Link action.  The air vehicle is pre-programmed for one of three LOL modes:

1. Finish Flight

2. Go To Rally (Location of Recovery Site/Land)

3. Land Immediately

Finish Flight.  If this option is enabled, 3 seconds after the air vehicle detects a loss-of-comms, it will maintain altitude while continuing the programmed mission. You must ensure a LOL time (loop) is specified for the total flight time to finish its flight before going to land in a no comms environment. If comms are not re-established the air vehicle will fly the programmed mission and land at the programmed landing point after the flight loop out time expires. If comms are re-established, the air vehicle will return to its programmed altitude and the operator can resume normal operations. This feature is especially useful for conducting hand-offs during a mission.

Go To Rally.  This option is the default mode.  Three seconds after the air vehicle detects a loss-of-comms, it will ascend to the pre-programmed altitude above its current altitude.  It will return to rally (Land WP) and orbit. If comms are not re-established and the air vehicle is not re-tasked within this LOL flight time period, the air vehicle will execute the currently programmed E to L landing pattern and land.

An indication of “en-route rally” or a clock symbol on top right corner (of hand controller/laptop) is shown when LOL occurs and when uplink and downlink is re-established.

Land Immediately (Loss of Link Mode 3): This option should not be used unless the mission dictates to execute this LOL mode in a loss of communication.  In this situation, upon 3 seconds of no communication the air vehicle will automatically auto-land. Be advised that if this mode is selected there is a reasonable chance that you will not recover the downed aircraft.

Loss of GPS: The aircraft will maintain last bearing (blinking), and still be able to navigate using heading.  The operator is warned when GPS loss occurs and has the option to change to manual or ALT mode and return to base or command autoland.  When autoland is commanded, the motors will shut down and the air vehicle will begin a 1:1 glide-in landing at its present heading. If time permits, it is advisable to point the air vehicle into the wind or away from harsh terrain prior to using this command

Loss of GPS Procedures For Operations at MCBH:  During operations at MCTAB or at MCAF, if a Loss of GPS signal is indicated, control will be switched to manual mode, the UAS will be directed towards a safe heading for recovery, and then Autoland command will be given.  This will ensure that the UAS is recovered within the specified operating area boundaries.

Raven B Lost Communication Procedures.

In the event of lost communications with ATC, both radio and cell phone, the return to base and Autoland mode will be utilized during operations at MCTAB and at MCAF.

Unplanned Loss of Motor Power: Upon loss of motor power, the aircraft will glide and can be controlled for direction to a safe landing position.

Battery or Power Failure / Loss: If there is loss battery failure or power loss the aircraft generally will hold control surfaces in last commanded position.  The aircraft will either glide, spiral or spin to the ground in a stable manner. If a fuse is blown while in-flight the aircraft will lock its rudder hard left and spiral down in a hard left turn.