ATC & UAS PROCEDURES/PHRASEOLOGY

The UAS Mission Commander will contact the FWO at ATC facility 30 minutes prior to launch time to ensure that the mission is still on.  The UAS controller will contact the tower and inform them when ready for takeoff.

Nightowl 01:  “Cherry Point tower Nightowl 01 is ready for takeoff.”

- The tower controller will visually scan the area and the BRANDS to determine if the UAS flight path is clear.

Nightowl 01:  “Nightowl 01 (wind) cleared for takeoff to the north or the south.” Or “Nightowl 01, unable standby.”

- The UAS will proceed directly to the North or South Hold Point at 1000’.  Once established at one of the hold points. The UAS controller will request to climb to 2500’.

Nightowl 01: “Tower Nightowl 01 request 2500’.”

- The tower controller will visually scan the area and the brands to determine if the UAS flight path is clear.

Tower: “Nightowl 01, 2500’ approved. Or “Nightowl 01 continue holding.”

- The tower controller will switch UAS to Approach Control.

- The UAS will transition to the holding point that is closest to the restricted are they wish to work.  This will be done within the CDSA before the UAS is switched to approach.

Nightowl 01: “Cherry Point Approach, Nightowl 01 at N/S Holding Point at 2500’.”

- Approach will start to scan A-530 to determine if there is any conflicting traffic.

- It is the controllers’ discretions of when to send the UAS to the restricted area.

Approach: “Nightowl 01 no traffic observed proceed direct to (restricted area)” or “Nightowl 01 continue holding.”

- When leaving the restricted area the UAS will contact Cherry Point Approach in a holding pattern in the restricted area. At this time the Approach Controller will again scan A-530 to determine when to send the UAS toward Cherry Point CDSA.

Nightowl 01: “Cherry Point Approach, Nightowl 01 is in holding within the restricted area ready to depart toward North/South Hold Point.”

Approach:  “Nightowl 01, Cherry Point Approach, no traffic observed proceed direct to North/South Hold Point.” Or “Nightowl 01, continue holding.”

- The UAS will continue its holding pattern within the restricted area.

Nightowl 01: “Roger Approach, Nightowl 01 proceeding to North/South Hold Point.” Or “Roger approach, Nightowl 01 holding.”

- Once UAS is established at 2500’ and entering the CDSA the Approach Controller will switch the UAS to Tower.

Nightowl 01:  “Cherry Point Tower, Nightowl 01 is entering the CDSA from the North/South for the North/South Holding Point.”

Tower:   “Nightowl 01, Cherry Point Tower, roger report established at North/South Hold Point.”

Nightowl 01:  “Tower, Nightowl 01 is established at south/north holding point, request descent.”

- The tower controller will visually scan the area and the BRANDS to ensure the area and altitude are clear.

Tower:  “Nightowl 01 decent approved.” Or “Nightowl 01 continue holding”

Nightowl 01:  “Roger decent approved for Nightowl 01.” Or “Roger holding for Nightowl 01.”

- The UAS will descend to 1000’ still in the Holding point pattern.

Nightowl 01:  “Tower, Nightowl 01 requests transition to Loiter Point.”

- The Tower Controller will ensure that the area of the Loiter point is clear.  If the UAS is arriving via the Southern Hold point, the controllers will “Delta” aircraft in the tower pattern for runway 32L. This is due to the fact that the Southern Loiter Point is over runway 32L.

Tower:  “Nightowl 01 transition approved.” Or “Night Owl 01 continue holding.”

Nightowl 01:  “Transition approved for Night Owl 01.” Or “Roger holding for Nightowl 01.”

- Tower Controller will start spacing accordingly to not delay UAS landing.

Nightowl 01:  “Tower Nighowl 01 is at the 180/straight-in, request landing clearance.”

Tower:  “Nightowl 01 (wind) cleared to land” or “Nightowl 01 unable landing clearance standby.”

Nightowl 01:  “Cleared to land for Nightowl 01” or “Roger standing by for Nightowl 01.”

- In the event of a UAS wave off, the UAS will climb straight ahead at 1000’ and proceed to the North/South Hold point, depending upon which direction the UAS is landing.  The UAS will establish at the Holding Point request to climb to 2500’ then transition to the hold point they started their approach from.