System Description

The RQ-11B UAS is a hand-launched reconnaissance and surveillance tool.  The Raven RQ-11B has a wingspan of 55 inches and is 36 inches in length.  The air vehicle weighs approximately 4.2 pounds including payload.  The RQ-11B AV flies for 60-90 minutes on rechargeable Lithium Ion (Li-ion) battery packs.  TheRQ-11B system is typically operated by two Operators consisting of a Vehicle Operator (VO) and a Mission Operator (MO). The four major subsystems that comprise the air vehicle include:

i. AV Structure:  The air vehicle structure consists of modular Kevlar composite, foam, and fiberglass.

ii. AV payload:  The air vehicle incorporates three modular payloads:  forward-look IR camera, side-look IR camera (with laser illuminator), and a dual forward/side-look EO camera.  The system transmits live airborne video images, compass headings and location information to a ground control station (GCS) to a optional laptop if connected, and to a remote video transceiver (RVT).

iii. AV Avionics:  The avionics and control system is made up of navigation and system health sensors, RF data link components and electromechanical control devices. A central processing unit runs software that performs autonomous functions, responds to command and control received from the GCS and transmits data back to the GCS. A high level automatic flight control system is executed in the avionics software.

iv. AV Propulsion:  The primary propulsion components consist of an electric motor, speed control and propeller. These elements generate propulsive force for the AV in response to commands received from the avionics.