to:	"COMNAVAIRFOR SAN DIEGO CA"

cc:	"CG I MEF\(uc\)", "CG II MEF\(uc\)", "CG III MEF\(uc\)", "CG 1ST MAW\(uc\)", "CG 2ND MAW\(uc\)", "CG 3RD MAW\(uc\)", "CG 4TH MAW\(uc\)", "CG MCCDC QUANTICO VA CCID\(uc\)", "CG MCCDC QUANTICO VA\(uc\)", "CG TECOM QUANTICO VA\(uc\)", "CG TECOM GTB\(uc\)", "CMC WASHINGTON DC AVN APP\(uc\)", "CMC WASHINGTON DC AVN APC\(uc\)", "CMC WASHINGTON DC AVN APW\(uc\)", "CMC WASHINGTON DC AVN ASL\(uc\)", "CMC WASHINGTON DC AVN ASM\(uc\)", "CMC WASHINGTON DC AVN\(uc\)", "COMMARCORSYSCOM QUANTICO VA\(uc\)", "COMMARFORCOM G3-5-7\(uc\)", "COMMARFORCOM G-1\(uc\)", "COMMARFORCOM G-2\(uc\)", "COMMARFORCOM\(uc\)", "COMMARFORPAC G1\(uc\)", "COMMARFORPAC G2\(uc\)", "COMMARFORPAC G3\(uc\)", "COMMARFORPAC\(uc\)", "COMMARFORRES G1\(uc\)", "COMMARFORRES G2\(uc\)", "COMMARFORRES G3 G5\(uc\)", "COMMARFORRES\(uc\)", "COMMARFORSOC G1\(uc\)", "COMMARFORSOC G2\(uc\)", "COMMARFORSOC G3\(uc\)", "COMMARFORSOC\(uc\)", "COMNAVAIRFORES SAN DIEGO CA\(uc\)", "COMNAVAIRSYSCOM PATUXENT RIVER MD\(uc\)", "MACG 18\(uc\)", "MACG 28\(uc\)", "MACG 38\(uc\)", "MACG 48\(uc\)", "VMU1\(uc\)", "VMU 2\(uc\)", "VMU3\(uc\)"

Subject:	P262013Z APR 11 UAS CAT 3 IFC REVISION FOR RQ-7B VMU-2 CHERRY POINT AND BOGUE

PASS TO OFFICE CODES:
COMNAVAIRSYSCOM PATUXENT RIVER MD//5.0D/4.0P//
COMNAVAIRFOR SAN DIEGO CA//N421D/N3//
COMMARFORCOM//ALD/G3//
CG 2ND MAW//ALD/G3//
MACG 28//G3//
VMU2//S3/MO//
CMC WASHINGTON DC AVN//APX//
MSGID/GENADMIN/MIL-STD-6040(SERIES)/B.0.01.00/NAVAIRSYSCOM AIR-4.0P/-
/-/-/-/-/UNCLASSIFIED//
SUBJ/UAS CAT 3 IFC REVISION FOR RQ-7B VMU-2 CHERRY POINT AND BOGUE
/TRANSITION TRAINING//
REF/A/DESC:DOC/NAVAIR/200800ZAPR2011//
REF/B/MSGID:GENADMIN/NAVAIR/262011ZFEB2010//
REF/C/DESC:DOC/USA/24MAR2011//
REF/D/DESC:DOC/USA/01OCT2009//
REF/E/DESC:DOC/USA/01OCT2009//
REF/F/DESC:DOC/USA/01OCT2009//
REF/G/DESC:DOC/USMC/01JAN2009//
REF/H/DESC:DOC/USAF, USN, USA/02FEB2007//
REF/I/DESC:EML/COMNAVAIRLANT/26FEB2010//
REF/J/DESC:DOC/LSRB SER1010/JFM/22APR2010//
REF/K/DESC:DOC/NAVAIR/28NOV2005//
REF/L/DESC:DOC/NAVAIR/27AUG2010//
REF/M/DESC:DOC/NAVAIR/15JUL2010//
REF/N/DESC:DOC/AAI/03AUG2010//
REF/O/DESC:NAVAIRINST 13034.1D/COMNAVAIRSYSCOM/15MAR2010//
NARR/REF A IS THE SUBJECT FLIGHT CLEARANCE REQUEST.
REF B IS THE PREVIOUS UAS SOF IFC FOR RQ-7B VMU-2 TRANSITION
TRAINING AT CHERRY POINT.
REF C IS THE AIRWORTHINESS RELEASE (AWR), FOR OPERATION OF
RQ-7B SHADOW UNMANNED AIRCRAFT SYSTEM (UAS) (AWR RQ-7B20070620R2)
(TN 70590A).
REF D IS THE US ARMY SHADOW 200 OPERATORS MANUAL, TECH MANUAL
TM 1-1550-689-10 (VOLUMES 1&2), NAVY A1-RQ7BA-NFM-000 AND 010 WITH
UPDATES.
REF E IS THE US ARMY SHADOW 200 MAINTENANCE MANUAL, ORGANIZATIONAL
AND DIRECT SUPPORT, TECH MANUAL TM 1 1550-689-23&P (IETM), 
NAVY: A1-RQ7B-IETM WITH UPDATES.
REF F IS THE US ARMY SHADOW 200 OPERATORS AND CREW MEMBERS
CHECKLIST, TECH MANUAL TM 1-1550-689-CL, NAVY A-1-RQ7BA-NFM-500
WITH UPDATES.
REF G IS THE ATCFACO P3722.1 SERIES, MCAS CHERRY POINT AIR TRAFFIC
CONTROL FACILITY MANUAL.
REF H IS THE TRI SERVICE MEMORANDUM OF AGREEMENT (MOA), MUTUAL
ACCEPTANCE OF AIRWORTHINESS CERTIFICATION DATA.
REF I IS THE ACKNOWELDGEMENT OF HIGHER PROBABILITY OF LOSS OF UA.
REF J IS THE LASER SAFETY REVIEW BOARD (LSRB) APPROVAL TO FIELD POP
300D PAYLOAD.
REF K IS THE NAVAIR INSTRUCTION 5100.11A, RESEARCH AND ENGINEERING
TECHNICAL REVIEW OF RISK PROCESS AND PROCEDURES FOR PROCESSING
GROUNDING BULLETINS.
REF L IS THE USMC RQ-7B SHADOW UAS MEDIUM RISK HAZARDS SYSTEM
SAFETY ASSESSMENT RESIDUAL RISK ACCEPTANCE BY PMA-263.
REF M IS THE USMC RQ-7B SHADOW UAS LOW RISK HAZARDS SYSTEM SAFETY
ASSESSMENT RESIDUAL RISK ACCEPTANCE BY PMA-263.
REF N IS THE FIELD SERVICE BULLETIN (FSB) TUAV FSB-1165 REVISED
POST FLIGHT ENGINE CHECK.
REF O IS THE FLIGHT CLEARANCE POLICY FOR AIR VEHICLES AND AIRCRAFT
SYSTEMS.//
POC/RON LYLISTON/UAS FACILITATOR/UNIT:NAVAIRSYSCOM  
/NAME:PATUXENT RIVER, MD/TEL:301-757-0503  
/EMAIL:RONNIE.LYLISTON.CTR@NAVY.MIL//
GENTEXT/REMARKS/1. IRT REF A, NAVAIR CATEGORY 3 (SEE PARA 7.H) 
INTERIM FLIGHT CLEARANCE (IFC) REVISION IS GRANTED FOR RQ-7B
SHADOW OPERATIONS IN SUPPORT OF VMU-2 TRANSITION TRAINING AT MARINE
CORPS AIR STATION (MCAS) CHERRY POINT, NC, MARINE CORPS AUXILIARY
LANDING FIELD (MCALF) BOGUE, NC, AND SURROUNDING RESTRICTED
AREAS (R-5306). THE PURPOSE OF THIS FLIGHT CLEARANCE IS TO ALLOW
VMU-2 TO CONDUCT TRAINING FLIGHTS ON THE SHADOW RQ-7B SYSTEM WITHIN
THE COA AIRSPACE AT MCAS CHERRY POINT, NC AND THE R-5306 RESTRICTED
AREA. THIS FLIGHT CLEARANCE ALSO SUPPORTS FAA COA APPLICATION
FOR AIRSPACE AROUND MCAS CHERRY POINT, NC. REF B IS HEREBY
CANCELED AND SUPERSEDED. REVISIONS INCLUDE UPDATED REFS C THROUGH
F. THIS FLIGHT CLEARANCE IS SUBJECT TO THE FOLLOWING CONFIGURATION,
LIMITS, WARNING, CAUTION AND NOTES.
2. TAKEOFF CONFIGURATION: BASELINE CONFIGURATION IAW REFS C THRU F.
3. LIMITATIONS: IAW REFS C THROUGH G AND THE FOLLOWING (MOST
   RESTRICTIVE APPLIES):
   A. FLIGHT OPERATIONS SHALL BE CONDUCTED TO AVOID POPULATED
      AREAS, EXPOSURE TO MANNED AIRCRAFT, AND TO MINIMIZE THE RISK
      OF INADVERTENTLY LASING PERSONNEL ON THE GROUND.
   B. FLIGHTS ARE RESTRICTED TO VISUAL METEOROLOGICAL CONDITIONS
      (VMC).
   C. THE AIRCRAFT SHALL NOT BE FLOWN AT ALTITUDES BELOW 2000 FEET
      AGL EXCEPT FOR LAUNCH AND RECOVERY ACTIVITIES.
   D. ALL FLIGHT OPERATIONS SHALL BE CONDUCTED WITH A MINIMUM OF
      ONE CONTROLLING GCS AND ONE BACK-UP GCS OR PGCS.
   E. LASER ENGAGEMENTS SHOULD NOT BE CONDUCTED UNLESS BOTH COMMAND
      AND CONTROL AND VIDEO DATALINKS HAVE ADEQUATE SIGNALS.
   F. ONLY ONE AIR VEHICLE SHALL BE ASSIGNED TO A FREQUENCY AT A
      TIME MULTIPLE AIR VEHICLES CONTROLLED FROM THE SAME SITE MUST
      BE OPERATED ON DIFFERENT FREQUENCIES. THIS APPLIES BOTH TO
      AIR VEHICLES OPERATIONS ORIGINATING FROM A GIVEN SITE AS WELL
      AS THOSE AIR VEHICLES WHOSE CONTROL IS TRANSFERRED TO THAT
      SITE. DATALINK FREQUENCIES SHALL BE APPROVED AND DECONFLICTED
      THROUGH THE LOCAL FREQUENCY MANAGER/COORDINATOR PRIOR TO 
      CONDUCTING OPERATIONS.
   G. AIRCRAFT EQUIPPED WITH A LASER DESIGNATOR SHALL ONLY POWER
      THE LASER DESIGNATOR WHILE THE AIRCRAFT IS IN AN APPROVED
      LASER OPERATING AREA.
   H. A MINIMUM 10 KNOT FORWARD GROUND SPEED SHALL BE MAINTAINED
      WHILE FLYING INTO PREVAILING WINDS.
4. WARNINGS, CAUTIONS AND NOTES: IAW REFS C THRU F AND THE 
   FOLLOWING:
------------------------------WARNING------------------------------
   A. THE RQ-7B SHADOW UAS HAS NOT COMPLETED FULL AIRWORTHINESS 
      QUALIFICATION. ALL FLIGHT OPERATIONS SHALL BE CONDUCTED IN A 
      MANNER TO MINIMIZE EXPOSURE TO MANNED AIRCRAFT AND POPULATED
      GROUND AREAS.
      (1) IN THE EVENT OF AN ENGINE FAILURE OR OTHER CATASTROPHIC
          FAILURE, LOCAL ATC/RANGE CONTROL AUTHORITY WILL BE
          NOTIFIED IMMEDIATELY AND THE AIRCRAFT SHALL BE DITCHED
          OVER THE RETURN HOME WAYPOINT. IF NOT POSSIBLE TO MAKE
          THE RETURN HOME WAYPOINT, EVERY EFFORT SHALL BE MADE TO
          VISUALLY INSPECT THE PROBABLE IMPACT AREA WITH THE
          PAYLOAD PRIOR TO PARACHUTE DEPLOYMENT.
      (2) MISSION PROFILES SHOULD BE PLANNED TO MINIMIZE FLIGHT
          OPERATIONS THAT ARE OUTSIDE OF THE 4:1 GLIDE RATIO TO
          RETURN HOME WAYPOINT. IF MISSION PLANNING IDENTIFIES
          OPERATIONS OUTSIDE THE 4:1 GLIDE SLOPE, ADDITIONAL DITCH
          POINTS SHALL BE IDENTIFIED, PLANNED AND BRIEFED DURING
          THE MISSION BRIEFING.
      (3) VERIFIED LOSS OF ANY AIRCRAFT FLIGHT CRITICAL SUBSYSTEM
          OR GROUND CONTROL STATION (GCS) FLIGHT CRITICAL
          SUB-SYSTEM SHALL REQUIRE RTB.
      (4) THE RETURN HOME POINT SHALL BE OVER A SUITABLE DITCH
          POINT AND WITHIN RANGE OF THE OMNI-ANTENNA.
------------------------------WARNING------------------------------
   C. THE RQ-7B SHADOW UAS HAS NOT UNDERGONE TESTING TO DETERMINE
      SUSCEPTIBILITY TO INTERNAL OR EXTERNAL ELECTROMAGNETIC
      FIELDS. IMPINGING RADIO-FREQUENCY (RF) ENERGY MAY DISRUPT
      RQ-7B SHADOW DATALINKS, CAUSE LOSS OF AIR VEHICLE CONTROL
      OR NAVIGATION, OR CAUSE LOSS OF CONTROL OF THE LASER PAYLOAD.
      (1) SPECIFIC STAND-OFF DISTANCES FROM ACTIVE RADARS OR RADIO
          TRANSMITTERS CANNOT BE PROVIDED: RELEVANT TEST DATA ARE
          ABSENT.
      (2) BASED UPON OTHER UAS, AND IN THE ABSENCE OF TEST DATA, IT
          IS RECOMMENDED THAT THE AIR VEHICLE BE FLOWN NO CLOSER
          THAN 2 NM TO VERY-HIGH-POWERED AIR-SEARCH AND
          FIRE-CONTROL RADARS. AN ADEQUATE SEPARATION DISTANCE FROM
          LOW-POWERED (100 WATT OR LESS) TRANSMITTERS WITH LOW-GAIN
          ANTENNAS, WHICH OPERATE AWAY FROM RQ-7B SHADOW DATALINK
          FREQUENCIES, WILL LIKELY BE A FEW TENS OF METERS.
------------------------------WARNING------------------------------
   D. THE RQ-7B SHADOW DOES NOT HAVE A SENSE AND AVOID SYSTEM. ALL
      FLIGHT OPERATIONS SHALL BE CONDUCTED TO ENSURE THAT MINIMUM
      SEPARATION STANDARDS ARE MAINTAINED.
      (1) OPERATORS SHALL COORDINATE AIRSPACE USE WITH THE AIRSPACE
          CONTROLLING AUTHORITY AND SHALL BE IN COMMUNICATION WITH
          THIS AUTHORITY TO ACTIVELY DECONFLICT THE AIRSPACE WHEN
          OPERATING THE AIRCRAFT.
      (2) IF THE LOCATION OF THE AIRCRAFT IS IN QUESTION THE
          OPERATORS SHALL INITIATE A RETURN TO BASE (RTB) OR FLIGHT
          TERMINATION PROCEDURES.
      (3) IF LOSS OF COMMUNICATIONS BETWEEN THE AIRCRAFT OPERATOR
          AND LOCAL ATC AUTHORITY/RANGE CONTROL OCCURS, THE
          AIRCRAFT SHALL BE DIRECTED TO THE RETURN HOME WAYPOINT.
------------------------------WARNING------------------------------
   E. THE RQ-7B SHADOW UAS HAS NOT UNDERGONE EXPLOSIVE
      ATMOSPHERE TESTING. A SERIOUS FIRE OR EXPLOSION MAY RESULT
      IF THE AIRCRAFT IS POWERED WHILE FLAMMABLE VAPORS ARE
      PRESENT DURING GROUND OR FLIGHT OPERATIONS. 
      (1) THE AIRCRAFT SHALL BE UN-POWERED AND GROUNDED IAW THE
          OPERATORS MANUALS (REF D), CHECKLIST (REF F), AND
          INTERACTIVE ELECTRONIC TECHNICAL MANUAL (REF E) DURING
          REFUELING OPERATIONS.
      (2) GROUND OPERATIONS OF THE AIRCRAFT SHALL BE CONDUCTED AT
          THE GREATEST DISTANCE PRACTICAL (NO LESS THAN 50 FEET)
          FROM ALL OTHER AIRCRAFT AND FUEL DEPOTS.
------------------------------WARNING------------------------------
   F. THE RQ-7B SHADOW UAS AIRBORNE COMPUTING EQUIPMENT (ACE) II
      MAY EXPERIENCE ERRONEOUS REBOOTS (WARM BOOTS) IN FLIGHT.
      MULTIPLE WARM BOOTS IN FLIGHT HAVE RESULTED IN LOSS OF
      AIRCRAFT CONTROL. IN THE EVENT OF MULTIPLE WARM BOOTS THE
      OPERATOR SHALL INITIATE RETURN TO BASE (RTB), EXCEPT AS
      NEEDED TO MEET MISSION REQUIREMENTS AND REPORT THE ANOMALY TO
      MAINTENANCE PERSONNEL.
------------------------------WARNING------------------------------
   G. THE POP 300D LRF/D PAYLOAD EMPLOYS A HIGH POWER INFRARED
      LASER WHICH IS CAPABLE OF PERMANENTLY BLINDING PERSONNEL.
      ALL PERSONNEL USING THIS SYSTEM SHALL BE TRAINED IN HAZARDS
      AND CONTROL MEASURES OF THE LASER SYSTEM TO PREVENT INJURY
      DURING LASER OPERATIONS. LASING SHALL BE LIMITED ONLY TO
      BORE-SIGHTING AND TRAINING OPERATIONS ON APPROVED RANGES. THE
      RECOMMENDATIONS, SAFE DISTANCES, AND EYE PROTECTION
      REQUIREMENTS OF REF J SHALL BE FOLLOWED.
------------------------------WARNING------------------------------
   H. DUE TO POTENTIAL BLINDING HAZARDS FROM THE LASER, ALL GROUND
      PERSONNEL ARE REQUIRED TO WEAR APPROPRIATE PROTECTIVE GOGGLES
      AT ALL TIMES EXCEPT WHEN THE AIRCRAFT IS ON THE GROUND AND
      THE PAYLOAD IS IN STOW MODE, WHEN THE POWER BUS TO THE LASER
      IS PHYSICALLY DISCONNECTED, AND WHEN THE BORESIGHTING TOOL IS
      ATTACHED.
------------------------------WARNING------------------------------
   I. FAILURE TO COMPLY WITH TUAV FSB-1165 REVISED POSTFLIGHT
      ENGINE CHECK (REF N) MAY RESULT IN CATASTROPHIC ENGINE
      FAILURE AND POSSIBLE LOSS OF AIR VEHICLE.
------------------------------CAUTION------------------------------
   J. THE RQ-7B SHADOW UAS HAS NOT BEEN TESTED FOR THE EFFECTS OF
      LIGHTNING. FLIGHT OPERATIONS SHALL BE RESTRICTED TO NO LESS
      THAN 25 NAUTICAL MILES FROM LIGHTNING ACTIVITY.
------------------------------CAUTION------------------------------
   K. IF A GCS IS RESPONSIBLE FOR RECOVERING MORE THAN ONE AIR
      VEHICLE, ENSURE TALS IS RESET TO STANDBY BETWEEN RECOVERIES.
      FAILURE TO DO SO MAY RESULT IN INADVERTENT ENGINE CUT TO A
      FLYING AIR VEHICLE.
------------------------------CAUTION------------------------------
   L. THE RQ-7B AIRCRAFT MAY REPORT A FLIGHT ALTITUDE PLUS-MINUS
      200 FEET OF ACTUAL ALTITUDE. FLIGHT ALTITUDE SHOULD BE
      PLANNED ACCORDINGLY.
------------------------------NOTE---------------------------------
   M. ACCIDENTAL OPERATION OF THE RQ-7B SHADOW UAS OUTSIDE OF
      RESTRICTED AIRSPACE, OR APPROVED COA AREA, SHALL BE
      IMMEDIATELY REPORTED TO AIR TRAFFIC CONTROL (ATC)/RANGE
      CONTROL. THE OPERATOR SHALL TAKE IMMEDIATE ACTIONS TO
      CORRECT THE FLIGHT PATH AND/OR FOLLOW ATC DIRECTION.
------------------------------NOTE---------------------------------
   N. DURING PREFLIGHT SOMEONE OTHER THAN THE OPERATOR SHALL
      VERIFY THAT THE RETURN HOME WAYPOINT IS ENTERED CORRECTLY.
------------------------------NOTE---------------------------------
   O. LOCAL ATC AUTHORITY SHALL BE BRIEFED BEFORE EACH FLIGHT WITH
      A FLIGHT PLAN/STRIP AND EXPECTED MISSION DURATION TO AID IN
      AIRSPACE DECONFLICTION. REFER TO REF G FOR CHERRY POINT ATC
      POLICIES.  
5. TIME PERIOD: THIS INTERIM FLIGHT CLEARANCE EXPIRES 24 MAR 2013.
6. POINTS OF CONTACT:
   A. MARCUS RICHMAN, AIR 4.1.1.5, UAV CLASS DESK,
      TEL: 301-342-3728, EMAIL: MARCUS.RICHMAN@NAVY.MIL
   B. PETER DOBBS, US ARMY, COMM (256) 846-2864,
      EMAIL: PETER.DOBBS@US.ARMY.MIL
   C. DAVID HUNNICUTT, US ARMY, COMM 256-313-5364,
      EMAIL: DAVID.HUNNICUTT@US.ARMY.MIL
   D. RON LYLISTON, 4.0P UAS FLIGHT CLEARANCE FACILITATOR,
      TEL: 301-757-0503, EMAIL: RONNIE.LYLISTON.CTR@NAVY.MIL.
   E. AIRWORTHINESS GLOBAL CUSTOMER SUPPORT TEAM, 301-757-0187,
      E-MAIL: AIRWORTHINESS@NAVY.MIL.
7. OTHER REMARKS:
   A. TWENTY-NINE (29) MEDIUM AND THREE (3) LOW RISK HAZARDS
      (CATEGORIZED IAW REF K) HAVE BEEN IDENTIFIED WITH THE SHADOW
      UAS THAT ARE RELEVANT TO THIS IFC. REF L AND REF M
      RESPECTIVELY DOCUMENT THESE HAZARDS AS WELL AS THEIR RISK
      ACCEPTANCE BY PMA-263.
   B. IAW WITH REF H, TRI-SERVICE MOA, THE ARMY AWR (REF C) WAS
      USED FOR DATA PROVIDED AS THE BASIS FOR AUTHORIZING
      THIS NAVAIR IFC. CONSULT REF O FOR NAVAIR FLIGHT CLEARANCE
      POLICY.
   C. APPROVAL TO FLY UAS WITHIN THE US NATIONAL AIRSPACE SYSTEM
      (NAS) OUTSIDE OF RESTRICTED OR WARNING AREAS, IS SUBJECT TO
      FAA APPROVAL.
   D. COORDINATION OF CHANNEL SELECTION WITH OTHER SYSTEMS
      OPERATING IN THE AREA IS CRITICAL TO REDUCING ACCIDENT RATES.
   E. INITIATING REQUEST FOR AND ADHERING TO RANGE AND AIRSPACE
      CLEARANCE IS THE RESPONSIBILITY OF THE OPERATORS.
   F. THIS FLIGHT CLEARANCE DOES NOT ADDRESS INTEROPERABILITY WITH
      MANNED AIRCRAFT.
   G. UAV OPERATOR QUALIFICATIONS, MAINTAINER QUALIFICATIONS,
      CONFIGURATION MANAGEMENT, ESTABLISHED FLIGHT TEST PRACTICES, 
      SCHEDULED/UNSCHEDULED MAINTENANCE PRACTICES, AND OTHER
      ASSOCIATED PROCEDURES ARE BEYOND THE SCOPE OF THIS FLIGHT
      CLEARANCE BUT SHOULD BE ADEQUATELY ADDRESSED BY AN
      APPROPRIATE AUTHORITY TO ENSURE AN ACCEPTABLE LEVEL OF
      SAFETY FOR THE UAS AND TO MINIMIZE RISK TO GROUND PERSONNEL
      AND PROPERTY.
   H. PER REF O, THIS FLIGHT CLEARANCE PROVIDES A NAVAIR CATEGORY 3
      AIRWORTHINESS CERTIFICATION SUBSEQUENT TO AN ENGINEERING 
      REVIEW AND RISK ASSESSMENT TO ENSURE SAFETY OF FLIGHT AND 
      REDUCE RISK TO PEOPLE, PROPERTY, AND/OR ENVIRONMENT.
   I. PER REF I, CNAF ACKNOWLEDGES HIGHER PROBABILITY OF LOSS OF
      THE UA, AS DEFINED IN PARA 6.E.3.C OF REF O.
   J. THIS IFC DOES NOT AUTHORIZE AIRCRAFT/SYSTEM MODIFICATION, NOR
      DOES IT SATISFY NAVAIR REQUIREMENTS FOR CONFIGURATION
      MANAGEMENT. AS DIRECTED BY OPNAVINST 4790.2J, REFER TO
      CNAFINST 4790.2A FOR POLICY GUIDANCE ON CONFIGURATION
      MANAGEMENT AND MOD AUTHORITY.
   K. INFORMATION REGARDING THE AIRWORTHINESS OFFICE AND PROCESS,
      INCLUDING A LISTING OF ALL CURRENT INTERIM FLIGHT CLEARANCES
      ISSUED BY NAVAIR 4.0P AND 24/7 CONTACT NUMBERS, CAN BE FOUND
      ON OUR WEBSITE: AIRWORTHINESS.NAVAIR.NAVY.MIL.
   L. EPOWER FOLDER 942301, TRACKING NUMBER 41698.//