UNCLASSIFIED//

PASS TO OFFICE CODES:

COMNAVAIRSYSCOM PATUXENT RIVER MD//5.0D/4.0P// MSGID/GENADMIN/NAVAIRSYSCOM AIR-4.0P//

SUBJ/UAS SOF INTERIM FLIGHT CLEARANCE FOR RQ-7B SHADOW.// REF/A/DESC:DOC/NAVAIR/211010ZJUN2007//

REF/B/DESC:DOC/USA/20JUN2007//

REF/C/DESC:DOC/USA/20OCT2004//

REF/D/DESC:DOC/USA/24AUG2004//

REF/E/DESC:DOC/USA/22OCT2004//

REF/F/DESC:DOC/USA/22OCT2004//

REF/G/DESC:DOC/USA/01MAY2003//

REF/H/DESC:DOC/USAF, USN, USA/02FEB2007// REF/I/DESC:DOC/NAVAIR/28SEP2004// NARR/REF A IS SUBJECT FLIGHT CLEARANCE REQUEST.

REF B IS US ARMY AIRWORTHINESS RELEASE (AWR), FOR OPERATION OF RQ-7B SHADOW UNMANNED AIRCRAFT SYSTEM (UAS) (QUALIFICATION LEVEL 3, AWR RQ-7B20070620 TN 45919).

REF C IS US ARMY SHADOW 200 OPERATORS MANUAL, TECH MANUAL TM 9-5895 YYY-10.

REF D IS US ARMY SHADOW 200 OPERATORS AND CREW MEMBERS CHECKLIST, TECH MANUAL TM 9-5895-681-CL.

REF E IS US ARMY SHADOW 200 MAINTENANCE MANUAL, ORGANIZATIONAL AND DIRECT SUPPORT, TECH MANUAL TM 9-5895-YYY-20.

REF F IS US ARMY SHADOW 200 UNIT, DIRECT SUPPORT AND GENERAL SUPPORT MAINTENANCE REPAIR PARTS AND SPECIAL TOOLS LIST, TECH MANUAL TM 9-5895-YYY-20P.

REF G IS MCAS CHERRY POINT AIR TRAFFIC CONTROL FACILITY MANUAL, CHAPTER 9, SECTION 2, UNMANNED AERIAL VEHICLE PROCEDURES (ATCFACO P3722.1K).

REF H IS TRI SERVICE MEMORANDUM OF AGREEMENT (MOA), MUTUAL ACCEPTANCE OF AIRWORTHINESS CERTIFICATION.

REF I IS NAVAIRINST 13034.1C, FLT CLNC POLICY FOR AIR VEHICLES AND ACFT SYSTEMS.// POC/KRISTEN REEVES/UAV FACILITATOR/NAVAIRSYSCOM /LOC:PATUXENT RIVER, MD/TEL:301-757-9454/TEL:FAX 301-342-3776 /EMAIL:KRISTEN.REEVES@NAVY.MIL//

GENTEXT/REMARKS/1.  UAS SOF INTERIM FLIGHT CLEARANCE FOR RQ-7B SHADOW.  IRT REFS A AND B, SAFETY OF FLIGHT (SOF) INTERIM FLIGHT CLEARANCE IS GRANTED FOR RQ-7B SHADOW OPERATIONS IN SUPPORT OF VMU-2 TRANSITION TRAINING AT MARINE CORP AIR STATION (MCAS) CHERRY POINT, NC AND SURROUNDING RETSTRICTED AIRSPACES (R-5306). THE PURPOSE OF THIS FLIGHT CLEARANCE IS TO ALLOW VMU-2 TO CONDUCT TRAINING FLIGHTS ON THE SHADOW RQ-7B SYSTEM WITHIN THE COA AIRSPACE AT MCAS CHERRY POINT, NC AND THE R-5306 RESTRICTED AIRSPACES.  THIS FLIGHT CLEARANCE IS SUBJECT TO THE FOLLOWING CONFIGURATION AND LIMITS.

2.  TAKEOFF CONFIGURATION: BASELINE CONFIGURATION IAW REFS B THRU E.

3.  LIMITATIONS: IAW REFS B THROUGH E.

4.  SPECIAL WARNINGS, CAUTIONS AND NOTES: IAW REFS B THRU E AND THE FOLLOWING WHICH ARE REITERATED FOR EMPHASIS:

A. --------------------------WARNING---------------------------

THE RQ-7B SHADOW UAS HAS NOT COMPLETED FULL AIRWORTHINESS

QUALIFICATION.  ALL FLIGHT OPERATIONS SHALL BE CONDUCTED IN

A MANNER TO MINIMIZE EXPOSURE TO MANNED AIRCRAFT AND

POPULATED GROUND AREAS.

B. ---------------------------WARNING--------------------------

ACCIDENTAL OPERATION OF THE RQ-7B SHADOW UAS OUTSIDE OF

RESTRICTED AIRSPACE, OR APPROVED COA AREA, SHALL BE

IMMEDIATELY REPORTED TO AIR TRAFFIC CONTROL (ATC)/RANGE

CONTROL.  THE OPERATOR SHALL MAKE IMMEDIATE ACTIONS TO

CORRECT THE FLIGHT PATH AND/OR FOLLOW ATC DIRECTION.

C. ---------------------------WARNING--------------------------

THE RQ-7B SHADOW UAS HAS NOT UNDERGONE COMPLETE

ELECTROMAGNETIC INTERFERENCE/ELCETROMAGNETIC COMPATIBLITY

(EMI/EMC) TESTING.  FLIGHT INTO HIGH INTENSITY EMI AREAS MAY

RESULT IN ERRONEOUS DATA REPORTS AND/OR LOSS OF CONTROL OF

AIRCRAFT. OPERATORS SHALL AVOID KNOWN HIGH INTENSITY EMI

AREAS.

D. ---------------------------WARNING--------------------------

THE RQ-7B SHADOW DOES NOT HAVE A SENSE AND AVOID SYSTEM.

MID-AIR COLLISON IS A RISK. ALL FLIGHT OPERATIONS SHALL BE

CONDUCTED TO ENSURE THAT MANNED AND UNMANNED AIRCRAFT SHALL

NOT OCCUPY THE SAME AIRSPACE.

E. ---------------------------WARNING--------------------------

THE RQ-7B SHADOW UAS HAS NOT UNDERGONE SOF EXPLOSIVE

ATMOSPHERE TESTING.  A SERIOUS FIRE OR EXPLOSION MAY RESULT

IF THE AIRCRAFT IS POWERED WHILE FLAMMABLE VAPORS ARE PRESENT

DURING GROUND OR FLIGHT OPERATIONS.  ADHERANCE TO PRECAUTIONS

OF REF B, PARA 3 OF APPENDIX A, SHALL BE OBSERVED IN ORDER

TO ENSURE SAFE OPERATIONS.

F. ---------------------------WARNING--------------------------

IN THE EVENT OF AN ENGINE FAILURE OF OTHER CATASTROPHIC

FAILURE, LOCAL ATC/RANGE CONTROL AUTHORITY WILL BE NOTIFIED

IMMEDIATELY AND THE AIRCRAFT SHALL BE DITCHED OVER THE

RETURN HOME WAYPOINT IN.  IF NOT POSSIBLE TO MAKE THE RETURN

HOME WAYPOINT, EVERY EFFORT SHALL BE MADE TO VISUALLY

INSPECT THE PROBABLE IMPACT AREA WITH THE PAYLOAD PRIOR TO

PARACHUTE DEPLOYMENT.

G. ---------------------------WARNING--------------------------

IF LOSS OF COMMUNICATIONS BETWEEN THE AIRCRAFT OPERATOR AND

LOCAL ATC AUTHORITY/RANGE CONTROL OCCURS, THE AIRCRAFT SHALL

BE DIRECTED TO THE RETURN HOME WAYPOINT.

H. --------------------------CAUTION---------------------------

MISSION PROFILES SHOULD BE PLANNED TO MINIMIZE FLIGHT

OPERATIONS THAT ARE OUTSIDE OF THE 4:1 GLIDE RATIO TO RETURN

HOME WAYPOINT.  IF MISSION PLANNING IDENTIFIES OPERATIONS

OUTSIDE THE 4:1 GLIDE SLOPE, ADDITIONAL DITCH POINTS SHALL

BE IDENTIFIED, PLANNED AND BRIEFED DURING THE MISSION

BRIEFING.

I. ---------------------------CAUTION--------------------------

THE RQ-7B SHADOW UAS HAS NOT BEEN TESTED FOR THE EFFECTS OF

LIGHTNING. FLIGHT OPERATIONS SHALL BE RESTRICTED TO NO LESS THAN

25 NAUTICAL MILES FROM LIGHTNING ACTIVITY.

J. --------------------------NOTE------------------------------

THE RQ-7B SHADOW UNMANNED AIRCRAFT SYSTEM (UAS) HAS NOT

COMPLETED FULL AIRWORTHINESS QUALIFICATION TESTING.  THIS

FLIGHT CLEARANCE IS IN SUPPORT OF A COA APPLICATION FOR

AIRSPACE AROUND MCAS CHERRY POINT, NC.  STRICT

ADHERENCE TO OPERATORS MANUAL (REF C), OPERATORS CHECKLIST

(REF D), TECH MAINTENANCE MANUALS (REFS E AND F), THE AIR

TRAFFIC CONTROL FACILITY MANUAL (REF G) AND THE AWR (REF B)

WILL MINIMIZE THE HAZARDS OF OPERATION.

K. --------------------------NOTE------------------------------

DURING PREFLIGHT SOMEONE OTHER THAN THE OPERATOR SHALL

VERIFY THAT THE RETURN HOME WAYPOINT IS ENTERED CORRECTLY.

L. ---------------------------NOTE-----------------------------

LOCAL ATC AUTHORITY SHALL BE BRIEFED BEFORE EACH FLIGHT WITH

EXPECTED MISSION DURATION TO AID IN AIRSPACE DECONFLICTION.

M. ---------------------------NOTE-----------------------------

VERIFIED LOSS OF ANY AIRCRAFT FLIGHT CRITICAL SUBSYSTEM OR

GROUND CONTROL STATION (GCS) FLIGHT CRITICAL SUB-SYSTEM SHALL

REQUIRE RTB.

N. ---------------------------NOTE-----------------------------

ANY UNEXPLAINED ANOMALY SHALL INTITIATE RTB EXCEPT AS NEEDED

TO MEET MISSION REQUIREMENTS.

O. ---------------------------NOTE-----------------------------

ANY PROCEDURAL DEFICIENCIES OR FLIGHT ANOMALIES DETECTED

DURING OPERATIONS SHALL BE CORRECTED, ANNOTATED AND REPORTED

TO THE POCS IN PARAGRAPH 6.A THROUGH 6.C.

P. ---------------------------NOTE-----------------------------

THE RECOMMENDED FUEL FOR SHADOW RQ-7B AIR VEHICLES IS AVGAS

100 LL. IN THE EVENT THAT AVGAS IS NOT AVAILABLE, A MINIMUM

87 OCTANE UNLEADED MOGAS WITH NO ETHANOL ADDITIVES (PER ASTM

D4814) MAY BE USED.

5.  TIME PERIOD: THIS INTERIM FLIGHT CLEARANCE EXPIRES 20 JUN 2009 OR UPON ISSUANCE OF LATER AWR WHICH CANCELS AND SUPERCEDES REF B.

6.  POINTS OF CONTACT:

A. KEN JOHNSON, AIR 4.1.1.5 (UAV CLASS DESK), (301) 342-3728,

EMAIL: KENNETH.JOHNSON(AT)NAVY.MIL

B. TOMAS BRANSON, US ARMY, COMM (256) 313-5420,

EMAIL: TOMAS.BRANSON(AT)US.ARMY.MIL

C. JOSHUA PREUSSER, US ARMY, COMM (256) 313-5375,

EMAIL: JOSHUA.PREUSSER(AT)US.ARMY.MIL

D. KRISTI REEVES, 4.0P FLIGHT CLEARANCE SUPPORT, DSN 757-9454,

COMM 301-757-9454, EMAIL: KRISTEN.REEVES(AT)NAVY.MIL.

E. INFORMATION REGARDING THE AIRWORTHINESS OFFICE AND PROCESS,

INCLUDING A LISTING OF ALL CURRENT INTERIM FLIGHT CLEARANCES

ISSUED BY NAVAIR 4.0P AND 24/7 CONTACT NUMBERS CAN BE FOUND

AT OUR WEBSITE: AIRWORTHINESS.NAVAIR.NAVY.MIL.  OUR GLOBAL

SUPPORT TEAM CAN BE CONTACTED AT 301-757-0187 OR BY E-MAIL

AT AIRWORTHINESS(AT)NAVY.MIL.

7.  OTHER REMARKS:

A. PER REF I, THIS FLIGHT CLEARANCE PROVIDES NAVAIR

SAFETY OF FLIGHT CERTIFICATION SUBSEQUENT TO A DESIGN

ENGINEERING REVIEW.  IT DOES NOT AUTHORIZE AIRCRAFT/SYSTEM

MODIFICATION, NOR DOES IT SATISFY NAVAIR REQUIREMENTS FOR

CONFIGURATION MANAGEMENT.  REFER TO OPNAVINST 4790.2J FOR

POLICY GUIDANCE ON CONFIGURATION MANAGEMENT AND MOD.

B. THE RQ-7B SHADOW UAS IS CONSIDERED TO BE EXPENDABLE BUT

REUSABLE.  PROBABILITY OF LOSS OF AIR VEHICLES IS ACCEPTED

BY CNAF.

C. RANGE AND AIRSPACE CLEARANCE IS THE RESPONSIBILITY OF THE

OPERATORS.

D. IAW WITH REF H, TRI-SERVICE MOA, THE ARMY AWR (REF B) WAS

USED FOR DATA PROVIDED AS THE BASIS FOR AUTHORIZING

THIS NAVAIR IFC. CONSULT REF I FOR NAVAIR FLIGHT CLEARANCE

POLICY.

E. THE DOD DOES NOT HAVE THE AUTHORITY TO AUTHORIZE UAS FLIGHTS

IN THE NATIONAL AIRSPACE. APPROVAL TO FLY WITHIN NATIONAL

AIRSPACE MUST BE GRANTED BY THE FAA.

F. UAV OPERATOR QUALIFICATIONS, MAINTAINER QUALIFICATIONS,

CONFIGURATION MANANGEMENT, ESTABLISHED FLIGHT TEST PRACTICES,

SCHEDULED/UNSCHEDULED MAINTENANCE PRACTICES, AND OTHER

ASSOCIATED PROCEDURES ARE BEYOND THE SCOPE OF THIS FLIGHT

CLEARANCE BUT SHOULD BE ADEQUATELY ADDRESSED BY AN

APPROPRIATE AUTHORITY TO ENSURE AN ACCEPTABLE LEVEL OF

SAFETY FOR THE UAS AND TO MINIMIZE RISK TO GROUND PERSONNEL

AND PROPERTY.  THIS IFC DOES NOT AUTHORIZE AIRCRAFT/SYSTEM

MODIFICATION, NOR DOES IT SATISFY NAVAIR REQUIREMENTS FOR

CONFIGURATION MANAGEMENT (REFER TO OPNAVINST 4790.2J FOR

POLICY GUIDANCE ON CONFIGURATION MANAGEMENT AND MOD).

G. FOLDER NUMBER IS 426984, AND TRACKING NUMBER IS 27554.//