Control Station

The GCS provides command and control of the AV, and displays video from the AV’s payload cameras. A GCS with the transmitter turned off can be used as a Remote Video Transceiver (RVT).  RVT functionality allows remote viewing of video sensor data and metadata coming from the AV, screen capture, replay and range and bearing function.  The GCS will operate approximately 1.5 to 2.0 hours on a single lithium battery.  The GCS provides the Vehicle Operator with level 5 control of the RQ-11B system which includes full control of the AV during launch, flight and landing along with full control of payload.  The GCS is composed of four major functional units.  The major functional units are the Hub Unit, Hand Controller, RF Unit, and Antenna Mast.  The four functional units are described below:

i. Hub Unit — The Hub Unit is the main functional element of the GCS that ties all the other elements together. It includes interfaces for the Hand Controller, RF Unit, Batteries, video and data out to external interfaces.  Main battery power for the GCS is connected at the Hub Unit. Power is conditioned and directed to other elements of the GCS.  The Hub Unit has a central processing unit which runs software that controls other devices within the GCS.  Communications, video and data signals are passed between other GCS components and external interfaces through the Hub Unit.

ii. Hand Controller—The Hand Controller is the primary human interface to the system.  It provides the user a means of input and output to the system.  Buttons and switches are provided for the user to enter commands and manipulate data.  An LCD display screen is provided for the user to see video imagery and data displayed as text.  The Hand Controller is powered by the Hub Unit.  A central processing unit in the Hand Controller runs software that controls the display and communication of video and data on the screen and to the Hub Unit.

iii. RF Unit—The RF Unit contains the uplink transmitter and downlink receiver. The RF Unit receives data from the Hub Unit and transmits it to the AV. The RF Unit receives video and data signals from the AV and sends them to the Hub Unit.

iv. Antenna Mast— The antenna mast is a structural element used to position the RF Unit above the ground for better RF line-of-sight to the AV.