Raven-B Emergency Procedures

Emergencies at MCTAB and MCAF:  If any emergencies are encountered during operations at MCTAB or MCAF, the following steps will be completed:

- Discontinue tactical or operator training.

- Use the safest mode possible to return the UAS to the planned home site and auto land mode, or steer in a safe direction away from any populated area.

- Transmit via radio or cell phone to MCAF airfield operations and ATC the nature of the emergency and the status.

- Report the emergency to MCBH G-3 either directly or through MCTAB manager with a detailed description of the incident to include the operator name, unit, contact, and any damage that occurred.

Failsafe Modes:  The Raven-B System has several failsafe features that prevent accidental damage to the air vehicle by operating outside of normal parameters, including terrain and loss of link avoidance as well as an automatic Go to Rally function when the loop timeout expires. Each of these failsafe options can be enabled or disabled.

WARNING

Extreme Low AV battery (<19V). Switch to MAN mode and maintain altitude with throttle (55 to 60% power required to maintain altitude). Select a suitable landing area and autoland (ensure that you are recording information with AV Screener.  Retrieve down aircraft.

OverRide the Go To Rally Function.  This option is disabled by the vehicle operator or mission operator. When the air vehicle battery reaches 21.9 volts remaining battery power, a notification window will appear on screen and no action will be taken. When the air vehicle battery reaches loop time out , a notification window will appear on screen, and then the air vehicle will automatically initiate the Go To Rally Function function. If the vehicle operator or mission operator overrides the Go To Rally function, the air vehicle will continue its mission.

NOTE

The downlink video will not be available when the air vehicle is operating in a loss of link condition.

WARNING

If Digital Terrain Elevation Data (DTED) is not properly loaded, the air vehicle may come in contact with terrain causing damage to equipment and / or injury to personnel.