COMMUNICATIONS SYSTEMS

FOR

DRAGANFLYER X6

HELICOPTER

RF Control & Data Link:

- 2.4GHz Data Link

  - Link Type: Helicopter to Ground & Ground to Helicopter (Two-Way)

  - Controller Antenna: Omni-Directional

  - Helicopter Antenna: Wired Whip Antenna

  - Transmission Power (North America): 100mW (+20dBm) EIRP

  - RF Data Rate: 250kbps

  - Receiver sensitivity: -100dBm

  - Transmission Technique: DSSS (Direct Sequence Spread Spectrum)

  - Frequency band: 2.4000 - 2.4835 GHz

  - Certifications: CE, FCC, IC, ETSI

  - Data Link Channel Selection: Automatic (12 Channels)

- Channels Supported: 16 (9 used)

  - Throttle

  - Roll

  - Pitch

  - Yaw

  - Camera Shutter

  - Ascent

  - Camera Zoom

  - Camera Tilt

  - GPS Position Hold

Embedded Dual Diversity Wireless Video Receiver:

- Model: DraganEye™ Pro

- Diversity Type: Dual

- Diversity Video Unit Switches Between Received Signals Based On:

  - Video Quality

  - RSSI Signal Strength

- Video Link

  - Frequency: 5.8GHz

  - Link Type: Helicopter to Ground (One-Way)

  - Receiver Antennas: Omni-Directional & Flat Patch

  - Transmitter Antenna: Omni-Directional

  - Transmission Power: 12dBm

  - Transmitter Power Consumption: 500mW

  - NTSC and PAL Compatible

  - 7 Selectable Channels: 5740MHz, 5760MHz, 5780MHz, 5800MHz, 5820MHz, 5840MHz, 5860MHz