Center for Robot-Assisted Search and Rescue (CRASAR)

LOST COMMUNICATIONS

PROCEDURE

FOR

DRAGANFLYER X6

AIR OPERATIONS

The proposed flight operations area is in Class D and E airspace.  The requested flight operations ceiling is 200ft AGL.  Prior to flight operations Easterwood Field tower will be contacted at 979-846-3998 and briefed on the intended flight operation/s with location, operational parameters/objectives and flight duration/s.

While conducting flight operations a VHF radio will be maintained and programmed to operational frequencies utilized for air traffic by Easterwood Field tower.  Should radio communications become disrupted, an emergency landing would be conducted (per FAR 91.129 (c) and (d)) and then Easterwood Field tower would be contacted via cell phone or satellite phone at the above number.