TECHNICAL DATA

FOR

DRAGANFLYER X8

HELICOPTER

The Draganflyer X8 is a remotely operated, unmanned, miniature, vertical take off and land, aircraft designed to carry wireless video cameras and still cameras.

Helicopter Size

- Dimensions

  - Width: 87cm (34.25in)

  - Height: 32cm (12.6in)

  - Length: 87cm (34.25in)

  - Top Diameter: 70cm (27.5in)

Weight & Payload:

- Helicopter Weight: 1,700g (60oz)

- Payload Capability: 1,000g (35oz)

- Maximum Gross Take-Off Weight: 2,700g (95oz)

Materials:

- Carbon Fiber

- Glass Filled Injected Nylon

- Aluminum & Stainless Steel Fasteners

- RoHS Compliant

Landing Gear:

- Installed Height: 32cm (12.6in)

- Stance Width: 35.5cm (13.9in)

- Skid Length: 34cm (13.4in)

- Landing Gear Material: Molded Carbon Fiber

Rotor Blades:

- Four Counter-Rotating Pairs (Eight Rotors Total)

- Rotor Blade Material: Molded Carbon Fiber

- Upper Rotor Diameter: 40cm (16in)

- Lower Rotor Diameter: 38cm (15in)

- Upper Rotor Weight: 12g (0.42oz)

- Lower Rotor Weight: 11g (0.38oz)

Position Navigation Lights:

- Type: 1 Watt LED Variable Brightness Emitters

- Luminous Flux at Full Brightness: 40lm

- Purpose: Helicopter Orientation Confirmation

- Visible Condition Range: Full Darkness to Direct Sunlight

- Standard Aircraft Colors

  - Red: Left

  - Green: Right

  - White: Tail/Rear

Electric Motors:

- Brushless Motors: 8

- Configuration: Direct Drive (One Motor per Rotor)

- Safety Features: Stall Protection

- Ball Bearing: 2 per Motor

- Rotor Mounting Points: Integrated

- Nominal Current Draw Per Motor: 1.04 Amps

- Nominal Power / Motor: 15.4 Watts

- Nominal Total Helicopter Motor Power: 92.4 Watts

- Peak Total Helicopter Motor Power: 450 Watts

- Motor Speed at Hover: 2000 RPM

- Voltage: 14.8V nominal

- Weight: 1.34oz (38g)

Operating Requirements:

- Operating Temperature: 14° to 104°F (-10° to 40°C)

- Relative Humidity: 0% to 90% Noncondensing

- Maximum Windspeed: 18mph (30km/h)

Flight Characteristics:

- Climb Rate: 6.5ft/s (2m/s)

- Descent Rate: 6.5ft/s (2m/s)

- Turn Rate: 90°/second

- Cruise Speed: 6mph (10km/h)

- Maximum Speed: 30mph (50km/h)

- Minimum Speed: 0mph (0km/h)

- Launch Type: VTOL (Vertical Take Off and Landing)

- Maximum Altitude ASL: 8,000ft (2438m)

- Maximum Flight Time: 25 minutes

11 Onboard Sensors:

- 3 Solid State MEMS (Micro-Electro-Mechanical Systems) Gyros

- 3 Solid State MEMS (Micro-Electro-Mechanical Systems) Accelerometers

- 3 Magnetometers (Magnetoresistive Sensors)

- 1 Barometric Pressure Sensor

- 1 GPS Receiver

  - GPS Battery Backup: 75mAh Lithium Polymer

GPS:

- GPS Used For: Position Hold, Location & Velocity Data

- Maximum Satellites Tracked Simultaneously: 16

- Position Update Rate: 4 Hz

- GPS Antenna: Ceramic Patch

- Battery Backup: Lithium Polymer

Black Box Data Recorder:

- Flight Data Recording: On-Board

- Stored To: Removable 1Gb MicroSD Memory Card

- Data Recorded: Onboard Sensor Flight Data (Link quality, Orientation, Altitude, Speed, Direction)

RF Communications:

- 2.4GHz Data Link

  - Link Type: Helicopter to Ground & Ground to Helicopter (Two-Way)

  - Helicopter Antenna: Wired Whip Antenna

  - Controller Antenna: Omni-Directional

  - Transmission Power (North America): 100mW (+20dBm)

  - Transmission Power (International): 10mW (+10dBm)

  - RF Data Rate: 250kbps

  - Receiver sensitivity: -100dBm

  - Transmission Technique: DSSS (Direct Sequence Spread Spectrum)

  - Frequency band: 2.4000 - 2.4835 GHz

  - Certifications: CE, FCC, IC, ETSI

  - Data Link Channel Selection: Automatic (12 Channels)

- 5.8GHz Video Link

  - Link Type: Helicopter to Ground (One-Way)

  - Transmitter Antenna: Omni-Directional

  - Receiver Antennas: Omni-Directional & Flat Patch

  - Transmission Power: 12dBm

  - Transmitter Power Consumption: 500mW

  - NTSC and PAL Compatible

  - 7 Selectable Channels: 5740MHz, 5760MHz, 5780MHz, 5800MHz, 5820MHz, 5840MHz, 5860MHz

Rechargeable Helicopter Battery:

- Cell Chemistry: Lithium Polymer

- Voltage: 14.8V nominal

- Capacity: 2600mAh

- Cell Configuration 4s2p (4-series 2-parallel)

- Connectors: Integrated Balance and Power

- Recharge Time: 30 minutes (after typical flight)

- Length: 7.5cm (2.9in)

- Width: 6.7cm (2.6in)

- Height: 2.7cm (1.0in)

- Weight: 228g (8.0oz)

Camera Attachments:

- 10MP (Mega-Pixel) Digital Still Camera with Remote Controlled Tilt, Zoom & Shutter

- 12MP (Mega-Pixel) Digital Still Camera with Remote Controlled Tilt & Shutter

- 18MP (Mega-Pixel) Digital Still Camera with Remote Controlled Tilt & Shutter

- 1080p HD (High Definition) Video Camera with Remote Controlled Tilt

- Thermal FLIR (Forward Looking Infra-Red) Camera with Remote Controlled Tilt

- Low Light (0.001lux) Dusk/Dawn Black & White Video Camera with Remote Controlled Tilt

- 2MP (Mega Pixel) Digital 802.11n IP Day/Night Video Camera with Remote Controlled Tilt